| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0169-v1 |
| --- | --- |

| Title: | AHG9/AHG12: Bitstream conformance requirements on subpicture ID |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Olena Chubach, Lulin Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chingyeh.chen, olena.chubach, lulin.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, bitstream conformance requirements related to the subpicture ID and the length of subpicture ID are proposed to guarantee that each subpicture in one coded picture shall have one unique subpicture ID. First, the maximum value of subpicture ID derived by the signalled subpicture ID length shall be greater than or equal to the number of subpicture IDs. Second, for each signalled subpicture ID, it shall be different from all other subpicture IDs in the same picture.

# Introduction

It is known [1] that subpicture IDs of the coded picture can be signalled in SPS, PPS, or PH as shown in the following specification. sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS. sps/pps/ph_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the SPS/PPS/PH. If sps/pps/ph_subpic_id_signalling_present_flag is equal to 1, sps/pps/ph_subpic_id_len_minus1, and a set of subpicture IDs are signalled in the SPS/PPS/PH. sps/pps/ph_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the sps/pps/ph_subpic_id signalled in the SPS/PPS/PH. sps/pps_num_subpics_minus1 is signalled in SPS/PPS, respectively, to indicate the total number of subpictures in the coded pictures.

Table 1. Sequence parameter set syntax RBSP syntax

| if( subpics_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 |  |
| … |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpic_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpic_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

sps_num_subpics_minus1 plus 1 specifies the number of subpictures. sps_num_subpics_minus1 shall be in the range of 0 to 254. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS.

sps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the SPS. sps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the SPS. When not present, the value of sps_subpic_id_signalling_present_flag is inferred to be equal to 0.

sps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sps_subpic_id[ i ]. The value of sps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

sps_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits. When not present, and when sps_subpic_id_present_flag equal to 0, the value of sps_subpic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to sps_num_subpics_minus1, inclusive

Table 2. Picture parameter set syntax RBSP syntax

| pps_subpic_id_signalling_present_flag | u(1) |
| --- | --- |
| if( pps_subpic_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |

pps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PPS. pps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PPS. When sps_subpic_id_present_flag is 0 or sps_subpic_id_signalling_present_flag is equal to 1, pps_subpic_id_signalling_present_flag shall be equal to 0.

pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS.

It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to sps_num_subpics_minus1.

pps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element pps_subpic_id[ i ]. The value of pps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of pps_subpic_id_len_minus1 shall be the same for all PPSs that are referred to by coded pictures in a CLVS.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

Table 3. Picture header RBSP syntax

| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| --- | --- |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

ph_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PH. ph_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PH.

ph_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element ph_subpic_id[ i ]. The value of pic_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of ph_subpic_id_len_minus1 shall be the same for all PHs that are referred to by coded pictures in a CLVS.

ph_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the ph_subpic_id[ i ] syntax element is ph_subpic_id_len_minus1 + 1 bits.

The list SubpicIdList[ i ] is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	SubpicIdList[ i ] = sps_subpic_id_present_flag ?	(76)
		( sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] :
		( ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ] ) ) : i

# Proposed method

It is proposed to add bitstream conformance requirements as follows.

## Length of subpicture ID and the number of subpictures

sps/pps/ph_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the subpicture IDs. For bitstream conformance requirement, it shall be further constrained depending on the value of sps/pps/ph_num_subpic_minus1.

The related syntax modifications are highlighted in blue.

sps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sps_subpic_id[ i ]. The value of sps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of (1 << (sps_subpic_id_len_minus1 plus 1)) shall be greater than or equal to (sps_num_subpics_minus1 plus 1).

pps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element pps_subpic_id[ i ]. The value of pps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of pps_subpic_id_len_minus1 shall be the same for all PPSs that are referred to by coded pictures in a CLVS. It is a requirement of bitstream conformance that the value of (1 << (pps_subpic_id_len_minus1 plus 1)) shall be greater than or equal to (pps_num_subpics_minus1 plus 1).

ph_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element ph_subpic_id[ i ]. The value of pic_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of ph_subpic_id_len_minus1 shall be the same for all PHs that are referred to by coded pictures in a CLVS. It is a requirement of bitstream conformance that the value of (1 << (ph_subpic_id_len_minus1 plus 1)) shall be greater than or equal to (sps_num_subpics_minus1 plus 1).

## Unique subpicture IDs in a coded picture

The subpicture ID mapping for the coded picture is signalled in SPS, PPS, or PH. In each coded picture, the list SubpicIdList[i] is derived as follows:

The list SubpicIdList[ i ] is derived as follows:

for( i = 0; i <= sps_num_subpics_minus1; i++ )
	SubpicIdList[ i ] = sps_subpic_id_present_flag ?	(76)
		( sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] :
		( ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ] ) ) : i

After that, SubpicIdList[i] can be used on each slice as the subpicture identifier of the subpicture that contains the slice. To well distigush each subpicture, each element in SubpicIdList[] shall be different from others. That is, each subpicture in a coded picture shall be presented by a unique ID. To achieve that, a bitstream conformance requirement of SubpicIdList[] is added as follows:

It is a requirement of bitstream conformance that SubpicIdList[i] and SubpicIdList[j] shall be different, if i is not equal to j.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).