| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0172-v1 |
| --- | --- |

| Title: | AHG9: Chroma format and bitdepth constraints for multi-layer structures |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Lulin Chen, Olena Chubach, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, lulin.chen, olena.chubach, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 7, the chroma format and bitdepth of each video layer are independently signalled in SPS without any constraint. In this contribution, for chroma format, it is proposed to add a bitstream conformance requirement that the values of chroma_format_idc in all video layers shall be the same, or the value of chroma_format_idc in a video layer shall be greater than or equal to the value in any lower reference video layer. For bitdepth, a bitstream conformance requirement is proposed: the value of bit_depth_minus8 in a video layer shall be greater than or equal to the value in any lower reference video layer. The corresponding decoding process modifications for supporting different chroma formats or different bitdepths in reference picture are also provided in this contribution.

# Introduction

In VVC Draft 7, multi-layer structures in one coded video sequence can be supported. In VPS, vps_max_layers_minus1 is signalled to specify the maximum allowed number of layers in each coded video sequence referring to the VPS. In each coded layer video sequence, the chroma_format_idc and bit_depth_minus8 are signalled in SPS to specify the chroma sampling format and the bitdepth of the samples of the luma and chroma arrays. The chroma_format_idc and bit_depth_minus8 are signalled independently regardless of the layer referencing structure, and there is no constraint on the values of chroma_format_idc and bit_depth_minus8 among different video layers. Different video layers may have different chroma formats and bitdepths, but the current VVC draft cannot support different chroma formats and bitdepths in inter-layer prediction process.

# Proposed constraint and method for chroma format

## Single chroma format in multiple layers

In case of allowing only one chroma format in multple layers, it is proposed to have a bitstream conformance requirement that the chroma formats of two or more layers that have dependency relation (e.g., one is referenced by others) shall be the same. The chroma format of a video layer shall be the same as that of any lower reference layer. The proposed text is provided in the following and highlighted in yellow.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements.

SPS NAL units, regardless of the nuh_layer_id values, share the same value space of sps_seq_parameter_set_id.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2.

It is a requirement of bitstream conformance that the value of chroma_format_idc of the current layer shall be the same as the value of chroma_format_idc of each reference layer of the current layer.

## Multiple chroma formats in multiple layers

In case of allowing multiple chroma formats in multiple layers, it is proposed to have a bitstream conformance requirement that the value of chroma format index (e.g. chroma_format_idc) in a video layer shall be greater than or equal to that in any lower reference video layer. The higher/current video layer can have more chroma information than the lower reference video layer. The proposed text is provided in the following and highlighted in yellow.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements.

SPS NAL units, regardless of the nuh_layer_id values, share the same value space of sps_seq_parameter_set_id.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2.

It is a requirement of bitstream conformance that the value of chroma_format_idc of the current layer shall be greater than or equal to the value of chroma_format_idc of each reference layer of the current layer.

Since the reference picture may have a different chroma format compared to the current picture, the chroma interpolation process needs to be modified as well. In the case of the chroma format of the reference picture is monochrome and the current picture is not monochrome, there is no available chroma reference sample. A default value should be assigned for the chroma predictors. The proposed text is provided in the following and highlighted in yellow.

#### 8.5.6.3 Fractional sample interpolation process

##### 8.5.6.3.1 General

Inputs to this process are:

- …

- a variable RefChromaHorCollocatedFlag specifying the sps_chroma_horizontal_collocated_flag value of the reference picture

- a variable RefChromaVerCollocatedFlag specifying the sps_chroma_vertical_collocated_flag value of the reference picture

- a variable RefSubWidthC specifying the SubWidthC value of the reference picture

- a variable RefSubHeightC specifying the SubHeightC value of the reference picture

- a variable RefChromaFormatIdc specifying the chroma_format_idc value of the reference picture

…

  - Otherwise if cIdx is not equal to 0 and RefChromaFormatIdc is equal to 0, the following applies:

    - All the values in array predSamplesLX are assigned as (1 << (BitDepth – 1)).

  - Otherwise (cIdx is not equal to 0 and RefChromaFormatIdc is not equal to 0), the following applies:

    - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

    - The top-left coordinate of the bounding block for reference sample padding ( xSbIntC, ySbIntC ) is set equal to ( (xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5), ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) ).

    - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- Let (refxSbC, refySbC) and ( refxC, refyC ) be chroma locations pointed to by a motion vector (refMvLX[ 0 ], refMvLX[ 1 ]) given in 1/32-sample units. The variables refxSbC, refySbC, refxC and refyC are derived as follows:

addXcur = (sps_chroma_horizontal_collocated_flag ? 0 : 16) * scalingRatio[ 0 ] / SubWidthC

addXref = (RefChromaHorCollocatedFlag ? 0 : 16) * ( 1  <<  14 ) / RefSubWidthC

addX = addXcur – addXref

addYcur = (sps_chroma_vertical_collocated_flag ? 0 : 16) * scalingRatio[ 1 ] / SubHeightC

addYref = (RefChromaVerCollocatedFlag ? 0 : 16) * ( 1  <<  14 ) / RefSubHeightC

addY = addYcur – addYref

refxSbC = ( ( ( xSb − scaling_win_left_offset ) / SubWidthC  <<  5 ) + refMvLX[ 0 ] ) *
	scalingRatio[ 0 ] + addX	(947)

refxC = ( ( Sign( refxSbC ) * ( ( Abs( refxSbC ) + 256 )  >>  9 )
		+ xC * ( ( scalingRatio[ 0 ] + 8 )  >>  4 ) ) * (SubWidthC/RefSubWidthC) + fRefLeftOffset / RefSubWidthC + 16 )  >>  5	(948)

refySbC =  ( ( ( ySb − scaling_win_top_offset ) / SubHeightC  <<  5 ) + refMvLX[ 1 ] ) *
	scalingRatio[ 1 ] + addY	(949)

refyC = ( ( Sign( refySbC ) * ( ( Abs( refySbC ) + 256 )  >>  9 ) + yC* ( ( scalingRatio[ 1 ] + 8 )  >>  4 ) ) *  (SubHeightC/RefSubHeightC)   + fRefTopOffset / RefSubHeightC + 16 )  >>  5	(950)

# Proposed constraint and method for bitdepth

## Supporting of different bitdepths in layers

In Scalable Extensions of HEVC (SHVC), bitdepth scalability is supported. The bitdepth in a video layer shall be greater than or equal to the bitdepth in any lower reference video layer. It is proposed to add the same bitstream conformance requirement in VVC. The proposed text is as follows and highlighted in yellow.

bit_depth_minus8 specifies the bit depth of the samples of the luma and chroma arrays, BitDepth, and the value of the luma and chroma quantization parameter range offset, QpBdOffset, as follows:

BitDepth   = 8 + bit_depth_minus8		(45)

QpBdOffset = 6 * bit_depth_minus8		(46)

bit_depth_minus8 shall be in the range of 0 to 8, inclusive.

It is a requirement of bitstream conformance that the value of bit_depth_minus8 of the current layer shall be greater than or equal to the value of bit_depth_minus8 of each reference layer of the current layer.

Since the reference picture may have a smaller bitdepth than that in the current picture, the decoding process of the inter prediction needs to be modified. The proposed text is as follows and highlighted in yellow.

8.5.6.3.2	Luma sample interpolation filtering process

…

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

- Let the RefBitDepth be the BitDepth of the reference picture. The variable shift1 is set equal to Max(0, Min(RefBitDepth + 4 – BitDepth , RefBitDepth – 8)), the variable shift2 is set equal to 6 , the variable shift3 is set equal to Max( 2 + BitDepth – RefBitDepth, 14 − RefBitDepth) and the veriable shift4 is set equal to Max(0, Max(BitDepth – RefBitDepth – 4, 8 – RefBitDepth)).

- The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

…

The predicted luma sample value predSampleLXL is derived as follows:

- If both xFracLand yFracL are equal to 0, and both scalingRatio[ 0 ] and scalingRatio[ 1 ] are less than 20481, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(961)

- Otherwise, if yFracL is equal to 0 and scalingRatio[ 1 ] is less than 20481, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fLH[ xFracL ][ i ] * (refPicLXL[ xInti ][ yInt3 ] ≪ shift4)   >>  shift1	(962)

- Otherwise, if xFracL is equal to 0 and scalingRatio[ 0 ] is less than 20481, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fLV[ yFracL ][ i ] * (refPicLXL[ xInt3 ][ yInti ]≪ shift4)   >>  shift1	(963)

- Otherwise, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fLH[ xFracL ][ i ] * (refPicLXL[ xInti ][ yIntn ]≪ shift4)   >>  shift1	(964)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fLV[ yFracL ][ i ] * temp[ i ]   >>  shift2	(965)

8.5.6.3.3	Luma sample interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

Let the RefBitDepth be the BitDepth of the reference picture. The variable shift is set equal to Max( 2 + BitDepth – RefBitDepth, 14 − RefBitDepth).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

…

8.5.6.3.4	Chroma sample interpolation process

…

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

- Let the RefBitDepth be the BitDepth of the reference picture. The variable shift1 is set equal to Max(0, Min(RefBitDepth + 4 – BitDepth , RefBitDepth – 8)), the variable shift2 is set equal to 6 , the variable shift3 is set equal to Max( 2 + BitDepth – RefBitDepth, 14 − RefBitDepth) and the veriable shift4 is set equal to Max(0, Max(BitDepth – RefBitDepth – 4, 8 – RefBitDepth)).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

…

The predicted chroma sample value predSampleLXC is derived as follows:

–	If both xFracC and yFracC are equal to 0, and both scalingRatio[ 0 ] and scalingRatio[ 1 ] are less than 20481, the value of predSampleLXC is derived as follows:

predSampleLXC = refPicLXC[ xInt1 ][ yInt1 ] << shift3	(977)

–	Otherwise, if yFracC is equal to 0 and scalingRatio[ 1 ] is less than 20481, the value of predSampleLXC is derived as follows:

predSampleLXC =  i=03fCH[ xFracC ][ i ] * (refPicLXC[ xInti ][ yInt1 ] ≪ shift4)  >>  shift1	(978)

–	Otherwise, if xFracC is equal to 0 and scalingRatio[ 0 ] is less than 20481, the value of predSampleLXC is derived as follows:

predSampleLXC =  i=03fCV[ yFracC ][ i ] * (refPicLXC[ xInt1 ][ yInti ]≪ shift4)   >>  shift1 	(979)

–	Otherwise, the value of predSampleLXC is derived as follows:

- The sample array temp[ n ] with n = 0..3, is derived as follows:

temp[ n ] =  i=03fCH[ xFracC ][ i ] * (refPicLXC[ xInti ][ yIntn ] ≪ shift4)  >>  shift1	(980)

- The predicted chroma sample value predSampleLXC is derived as follows:

predSampleLXC =( fCV[ yFracC ][ 0 ] * temp[ 0 ] +
			         fCV[ yFracC ][ 1 ] * temp[ 1 ] +
				      fCV[ yFracC ][ 2 ] * temp[ 2 ] +	(981)
				     fCV[ yFracC ][ 3 ] * temp[ 3 ] ) >> shift2

Note that, if the bitdepth of current picture (BitDepth) is smaller than or equal to 12 bits, the shift4 is always equal to 0. For the profile supporting bitdepth smaller than or equal to 12 (e.g., Main 10, Main 12, Monochrome 12, Main 4:4:4/4:2:2 10/12 profiles in HEVC), the modification related to shift4 can be removed.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).