| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0173-v1 |
| --- | --- |

| Title: | AHG9: On signalling the DPB parameters with delta values |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Chia-Ming Tsai, Tzu-Der Chuang, Olena Chubach, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chia-ming.tsai, peter.chuang, olena.chubach, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to signal the delta values of decode picture buffer (DPB) parameters between two successive temporal sublayers instead of directly signalling the absolute values of DPB parameters in each temporal sublayer. The relevant syntax elements are max_dec_pic_buffering_minus1[] and max_num_reorder_pics[]. In VVC Draft 7, these two syntax elements are signalled from the lowest temporal sublayer to the highest one, and one bitstream conformance requirement is applied to guarantee that the values of these two syntax elements in the higher temporal sublayer shall be greater than or equal to those in the lower temporal sublayer. This bitstream conformance requirement can be removed by signalling the delta values between two successive temporal layers. Moreover, the number of total coded bits for these two syntax elements can be reduced by the proposed syntax.

# Introduction

It is known [1] that the DPB parameter structure can be signalled in the VPS and SPS. The DPB parameter structure includes three DPB parameters, max_dec_pic_buffering_minus1[i], max_num_reorder_pics[i], and max_latency_increase_plus1[i], for i in the range of 0 to maxSubLayersMinus1. Moreover, if subLayerInfoFlag is equal to 1, the DPB parameters are signalled for each temporal sublayer. Otherwise, only the DPB parameters in the highest temporal sublayer (i==maxSubLayersMinus1) are siganlled, and the DPB parameters in the other sublayers are inferred to be the DPB parameters in the highest sublayer. Since the number of pictures is increased or kept the same when decoding more and more temporal sublayers, max_dec_pic_buffering_minus1[] and max_num_reorder_pics[] in the higher temporal sublayer shall be greater than or equal to the values in the lower temporal sublayer. In VVC Draft 7, a bitstream conformance requirement is used to guarantee this, which is highlighted in blue in the following syntax.

Table 1. DPB parameters syntax

| dpb_parameters( dpbSizeOnlyFlag, maxSubLayersMinus1, subLayerInfoFlag ) { | Descriptor |
| --- | --- |
| for( i = ( subLayerInfoFlag ? 0 : maxSubLayersMinus1 ); i <= maxSubLayersMinus1; i++ ) { |  |
| max_dec_pic_buffering_minus1[ i ] | ue(v) |
| if( !dpbSizeOnlyFlag ) { |  |
| max_num_reorder_pics[ i ] | ue(v) |
| max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

max_dec_pic_buffering_minus1[ i ] plus 1 specifies, for each for each CLVS of the CVS, the maximum required size of the DPB in units of picture storage buffers when Htid is equal to i. The value of max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause A.4.2. When i is greater than 0, max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to max_dec_pic_buffering_minus1[ i − 1 ]. When max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to  maxSubLayersMinus1 − 1, inclusive, due to subLayerInfoFlag being equal to 0, it is inferred to be equal to max_dec_pic_buffering_minus1[ maxSubLayersMinus1 ].

max_num_reorder_pics[ i ] specifies, for each CLVS of the CVS, the maximum allowed number of pictures of the CLVS that can precede any picture in the CLVS in decoding order and follow that picture in output order when Htid is equal to i. The value of max_num_reorder_pics[ i ] shall be in the range of 0 to max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, max_num_reorder_pics[ i ] shall be greater than or equal to max_num_reorder_pics[ i − 1 ]. When max_num_reorder_pics[ i ] is not present for i in the range of 0 to maxSubLayersMinus1 − 1, inclusive, due to subLayerInfoFlag being equal to 0, it is inferred to be equal to max_num_reorder_pics[ maxSubLayersMinus1 ].

max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ], which specifies, for each CLVS of the CVS, the maximum number of pictures in the CLVS that can precede any picture in the CLVS in output order and follow that picture in decoding order when Htid is equal to i.

When max_latency_increase_plus1[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] is specified as follows:

MaxLatencyPictures[ i ] = max_num_reorder_pics[ i ] + max_latency_increase_plus1[ i ] − 1	(7-73)

When max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When max_latency_increase_plus1[ i ] is not present for i in the range of 0 to maxSubLayersMinus1 − 1, inclusive, due to subLayerInfoFlag being equal to 0, it is inferred to be equal to max_latency_increase_plus1[ maxSubLayersMinus1 ].

# Proposed method

In this contribution, it is proposed to modify the syntax design of DPB parameters in order to replace the bitstream confermance requirement. The proposed syntax design is to signal the delta values of DPB parameters between two successive temporal sublayers instead of signalling the absolute values of DPB parameters directly. The relevant DPB parameters are max_dec_pic_buffering_minus1[] and max_num_reorder_pics[]. In this way, the number of coded bits for DPB parameters may be further reduced, and the bitstream conformance requirement can be removed to reduce the conformance test effort.

The related syntax modifications are highlighted in blue.

Table 2. DPB parameters syntax

| dpb_parameters( dpbSizeOnlyFlag, maxSubLayersMinus1, subLayerInfoFlag ) { | Descriptor |
| --- | --- |
| for( i = ( subLayerInfoFlag ? 0 : maxSubLayersMinus1 ); i <= maxSubLayersMinus1; i++ ) { |  |
| max_dec_pic_buffering_minus1delta [ i ] | ue(v) |
| if( !dpbSizeOnlyFlag ) { |  |
| max_delta_num_reorder_pics[ i ] | ue(v) |
| max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

max_dec_pic_buffering_minus1delta[ i ] plus 1 specifies, for each for each CLVS of the CVS, the delta value between two successive sublayers to derive the maximum required size of the DPB in units of picture storage buffers, maxReqDpbSize [i], when Htid is equal to i, as follows:

if (i ==0),

maxReqDpbSize [0] =  max_dec_pic_buffering_delta[ 0 ]

otherwise,

maxReqDpbSize [i] =  ((subLayerInfoFlag) ? maxReqDpbSize [i-1] : 0) + max_dec_pic_buffering_delta[ i ]

The value of max_dec_pic_buffering_minus1delta[ i ] shall be in the range of 0 to MaxDpbSize, inclusive, where MaxDpbSize is as specified in clause A.4.2. When i is greater than 0, max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to max_dec_pic_buffering_minus1[ i − 1 ]. When max_dec_pic_buffering_minus1delta[ i ] is not present for i in the range of 0 to maxSubLayersMinus1 − 1, inclusive, due to subLayerInfoFlag being equal to 0, it is inferred to be equal to 0 max_dec_pic_buffering_minus1[ maxSubLayersMinus1 ]. And maxReqDpbSize [i] for i in the range of 0 to  maxSubLayersMinus1 − 1, inclusive,  is inferred to be equal to maxReqDpbSize [maxSubLayersMinus1]

max_delta_num_reorder_pics[ i ] specifies, for each CLVS of the CVS, the delta value between two successive sublayers used to derive the maximum allowed number of pictures of the CLVS that can precede any picture in the CLVS in decoding order and follow that picture in output order when Htid is equal to i as follows:

if (i ==0),

maxReorderPics [0] =  max_delta_num_reorder_pics[ 0 ]

otherwise,

maxReorderPics [i] =  ((subLayerInfoFlag) ? maxReorderPics [i-1]:0) + max_delta_num_reorder_pics [ i ]

The value of max_delta_num_reorder_pics[ i ] shall be in the range of 0 to maxReqDpbSize[i] -1max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, max_num_reorder_pics[ i ] shall be greater than or equal to max_num_reorder_pics[ i − 1 ]. When max_num_reorder_pics[ i ] is not present for i in the range of 0 to maxSubLayersMinus1 − 1, inclusive, due to subLayerInfoFlag being equal to 0, it is inferred to be equal to 0.max_num_reorder_pics[ maxSubLayersMinus1 ]. And maxReorderPics [i] for i in the range of 0 to  maxSubLayersMinus1 − 1, inclusive, is inferred to be equal to maxReorderPics [maxSubLayersMinus1]

max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ], which specifies, for each CLVS of the CVS, the maximum number of pictures in the CLVS that can precede any picture in the CLVS in output order and follow that picture in decoding order when Htid is equal to i.

When max_latency_increase_plus1[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] is specified as follows:

MaxLatencyPictures[ i ] = maxReorderPicsmax_num_reorder_pics[ i ] + max_latency_increase_plus1[ i ] − 1	(7-73)

When max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When max_latency_increase_plus1[ i ] is not present for i in the range of 0 to maxSubLayersMinus1 − 1, inclusive, due to subLayerInfoFlag being equal to 0, it is inferred to be equal to max_latency_increase_plus1[ maxSubLayersMinus1 ].

All related syntax modifications are shown in the attached specification Word file.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team, JVET-P2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).