| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0177-v1 |
| --- | --- |

| Title: | AHG9: On picture identification and PH repetition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Ching-Yeh Chen, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chingyeh.chen, jenny.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to specify the picture order count in the picture header (PH) as the picture identifier for the picture associated to the PH. There are two aspects of the picture identifier.

- The first aspect is the identifier for marking the picture in temporal domain.

- The second aspect is the identifier for differentiating the PH from the PH repetition.

# Introduction

In the VVC Draft 7 [1], the PH is added. The picture header collectively stores those parameters that may not change per slice for the slices associated to it and the PH is required as start of a picture. The PH structure is specified as follows.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| if( !sps_loop_filter_across_virtual_boundaries_disabled_present_flag ) { |  |
| ph_loop_filter_across_virtual_boundaries_disabled_present_flag | u(1) |
| if( ph_loop_filter_across_virtual_boundaries_disabled_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| pic_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| if ( !pps_dep_quant_enabled_flag ) |  |
| pic_dep_quant_enabled_flag | u(1) |
| if( !pic_dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

In the last meeting, there were some discussions on the PH repetition mechanism and further study is encouraged. In this contribution, it is proposed to specify the picture order count in the PH for the picture associated to the PH. It may be used as the identifier in temporal domain and the identifier when the PH repetition occurs which is used as a kind of loss detection mechanism. There are two aspects of the picture identifier. The first aspect is the identifier for marking the picture in temporal domain. The second aspect is the identifier for differentiating the PH from the PH repetition.

The proposed picture identifier or picture header identifier may benefit more using the dependent PH for the PH size reduction as proposed in JVET-Q0198.

The proposed syntax and semantics are as follows and highlighed in green.

Aspect 1: the identifier for marking the picture in temporal domain

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| ph_pic_order_cnt_lsb | u(v) |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| … |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

ph_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the ph_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the ph_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

NOTE – ph_pic_order_cnt_lsb may be used as the identifier when the PH repetition occurs. The value of ph_pic_order_cnt_lsb shall be the same in all repetitive picture headers. The first slice with slice_pic_order_cnt_lsb equal to ph_pic_order_cnt_lsb of the first PH is the first slice of the picture associated with the PH.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| … |  |
| byte_alignment( ) |  |
| } |  |

      - 7.4.8.1 General slice header semantics

When present, the value of the slice header syntax element slice_pic_order_cnt_lsb shall be the same in all slice headers of a coded picture.

The variable CuQpDeltaVal, specifying the difference between a luma quantization parameter for the coding unit containing cu_qp_delta_abs and its prediction, is set equal to 0. The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr, specifying values to be used when determining the respective values of the Qp′Cb, Qp′Cr, and Qp′CbCr quantization parameters for the coding unit containing cu_chroma_qp_offset_flag, are all set equal to 0.

slice_pic_order_cnt_lsb shall be equal to ph_pic_order_cnt_lsb for the slices associted with the PH. specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(114)

Aspect 2: the identifier for differentiating the PH from the PH repetition

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| sps_ph_repetition_enabled_flag | u(1) |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | u(4) |
| chroma_format_idc | u(2) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| … |  |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_ph_repetition_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| num_reserved_constraint_bytes | u(8) |
| for( i = 0; i < num_reserved_constraint_bytes; i++ ) |  |
| gci_reserved_constraint_byte[ i ] | u(8) |
| } |  |

no_ph_repetition_constraint_flag equal to 1 specifies that there shall be no picture header repetitions present in OlsInScope. sps_ph_repetition_enabled_flag shall be equal to 0. no_ph_repetition_constraint_flag equal to 0 does not impose such a constraint.

sps_ph_repetition_enabled_flag equal to 1 specifies that picture header repetitions may be present in CLVSs for pictures associated to the PHs and referring to the SPS. sps_ph_repetition_enabled_flag equal to 0 specifies that picture header repetitions are not present in CLVSs for pictures referring to the SPS.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| if( sps_ph_repetition_enabled_flag ) |  |
| ph_repetition_present_flag | u(1) |
| If ( ph_repetition_present_flag ) |  |
| ph_pic_order_cnt_lsb | u(v) |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| … |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

ph_repetition_present_flag equal to 1 specifies that ph_pic_order_cnt_lsb is present for the coded picture associated with the PH. The picture header repetition may or may not occur for the coded picture associated with the PH. ph_repetition_flag equal to 0 specifies that ph_pic_order_cnt_lsb is not present.

ph_pic_order_cnt_lsb, when present, specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the ph_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the ph_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

NOTE – ph_pic_order_cnt_lsb may be used as the identifier when the PH repetition occurs. The value of ph_pic_order_cnt_lsb shall be the same in all repetitive picture headers. The first slice with slice_pic_order_cnt_lsb equal to ph_pic_order_cnt_lsb of the first PH is the first slice of the picture associated with the PH.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| … |  |
| byte_alignment( ) |  |
| } |  |

      - 7.4.8.1 General slice header semantics

When present, the value of the slice header syntax element slice_pic_order_cnt_lsb shall be the same in all slice headers of a coded picture.

The variable CuQpDeltaVal, specifying the difference between a luma quantization parameter for the coding unit containing cu_qp_delta_abs and its prediction, is set equal to 0. The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr, specifying values to be used when determining the respective values of the Qp′Cb, Qp′Cr, and Qp′CbCr quantization parameters for the coding unit containing cu_chroma_qp_offset_flag, are all set equal to 0.

slice_pic_order_cnt_lsb, when ph_pic_order_cnt_lsb is not present, specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When ph_pic_order_cnt_lsb is present, slice_pic_order_cnt_lsb shall be equal to ph_pic_order_cnt_lsb for the slices associted with the PH.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(114)

# Conclusion

This contribution proposes to specify the picture order count in the PH as the picture identifier for the picture associated to the PH. The identifier may be used to indicate the PH repetition when the PH repetition is allowed in VVC. The proponents suggest the group adopt the proposed picture identifier.

# References

- “Versatile Video Coding (Draft 7),” JVET-P2001-vE, October 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).