| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0191-v1 |
| --- | --- |

| Title: | Deblocking filter process with considering dependent quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Chen-Yen Lai, Zhi-Yi Lin, Olena Chubach, Tzu-Der Chuang, Chun-Chia Chen, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, jenny.lai, zhi-yi lin, olena.chubach, peter.chuang, chunchia.chen, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a bugfix is proposed to consider the effective quantization parameter (QP) in deblocking filter (DF) process when the dependent quantization (DQ) process is applied. In VVC Draft 7, when DQ is applied, the QP for dequantization is equal to QP minus 5. However, the QP minus 5 effect is not considered in deriving DF parameters (β and tc). As a simple fix, if one side of the current deblocking edge is quantized by using DQ (i.e., not coded with transform skip), the QP at the side is subtracted by 5 for deriving DF parameters. It is asserted that the proposed bugfix can reduce over-smoothing.

# Introduction

In VVC Draft 7 [1], QPs at both sides of the current deblocking edge are used for deriving β and tc. However, when DQ is applied, DF does not consider the effect of (QP – 5) used in DQ. Therefore, if DQ is applied to any side of the current deblocking edge (i.e., any side not coded with transform skip), the QP used in DF is higher than the effective QP used in dequantization. That means, the reconstructed pictures can be over-smoothed by DF.

# Proposed method

In the proposed method, when DQ is enabled in the current slice, if one side of the current deblocking edge is not coded with transform skip mode, the QP at the side is subtracted by 5 for deriving DF parameters (β and tc).

# Experimental results

The proposed method has been implemented on VTM7.0 [2]. Table 1 shows the objective results of the proposed method under the common test condition (CTC) [3]. The proponents would like to point out that BD-rate numbers in Table 1 are not relevant when deciding whether to adopt this proposal. It is suggested to further study this issue in a Core Experiment (CE) if the QP mismatch between the current DF and dequantization when DQ is applied is confirmed a possible issue of over-smoothing by experts in the 17th JVET meeting.

Table 1. Results of the proposed method under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.74% | -0.23% | 100% | 102% |
| Class A2 | -0.64% | -0.70% | -0.77% | 101% | 99% |
| Class B | -0.72% | -0.76% | -0.77% | 100% | 100% |
| Class C | -0.47% | -0.87% | -0.89% | 100% | 100% |
| Class E | -0.50% | -0.42% | -0.09% | 100% | 100% |
| Overall | -0.60% | -0.72% | -0.60% | 100% | 100% |
| Class D | -0.45% | -0.73% | -0.68% | 100% | 99% |
| Class F | -0.33% | -0.88% | -1.10% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.59% | -0.21% | 100% | 100% |
| Class A2 | -0.56% | -0.56% | -0.65% | 100% | 100% |
| Class B | -0.43% | -0.75% | -0.83% | 99% | 98% |
| Class C | -0.22% | -1.09% | -0.99% | 100% | 101% |
| Overall | -0.38% | -0.77% | -0.71% | 100% | 99% |
| Class D | -0.22% | -1.01% | -0.88% | 100% | 99% |
| Class F | -0.36% | -1.14% | -1.31% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.59% | -0.51% | -0.30% | 101% | 100% |
| Class C | -0.43% | -0.52% | -0.52% | 100% | 100% |
| Class E | -1.06% | -0.10% | -0.09% | 100% | 100% |
| Overall | -0.65% | -0.41% | -0.32% | 100% | 100% |
| Class D | -0.31% | 0.43% | -0.09% | 99% | 99% |
| Class F | -0.66% | -0.90% | -0.51% | 100% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.45% | -0.12% | -0.59% | 100% | 99% |
| Class C | -0.32% | -0.26% | -0.56% | 100% | 100% |
| Class E | -0.64% | -0.39% | -0.11% | 100% | 99% |
| Overall | -0.45% | -0.23% | -0.46% | 100% | 100% |
| Class D | -0.17% | -0.27% | 0.42% | 99% | 101% |
| Class F | -0.57% | -0.91% | -0.71% | 100% | 99% |

# Conclusion

A bugfix is proposed to consider the effective QP in DF when DQ is applied. It is asserted that the proposed bugfix can reduce over-smoothing.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001.

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).