| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0200-v2 |
| --- | --- |

| Title: | [AHG9]: On picture level and slice level tool parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim SangHeon Lee 10225 Willow Creek Rd, San Diego, CA 92131, USA 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Email: | dr.hendry@lge.com seunghwan3.kim@lge.com sangheon1.lee@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

At the 16th JVET meeting, it was agreed to move signalling of syntax elements that are applied at picture level to picture header. It was also agreed to have overriding mechanism using a flag that is signalled in picture header to specify whether signalling for a particular tool is present in PH or in slice header.

It is asserted that allowing picture level and slice level signalling to change at every picture header is not necessary. In many use-cases, for a coding tool that may be applicable at both picture level and slice level, encoder is implemented to operate either at picture level or slice level and not changing between these two frequently.

This contribution proposes the following:

- For signalling override mechanism between picture header and slice header, instead of signalling a flag for a particular tool to specify whether the signalling is present in picture header or in slice header, the flag is signalled in PPS instead.

- The above override mechanism may be applied to signalling of syntax element related to the following tools: reference picture lists, SAO, ALF, and deblocking.

In the first revision of this contribution, the following was added:

- The control flag(s) for deblocking in PPS was updated from two flags to become one flag to specify whether deblocking override is may be present in PH or in SH.

- The control flag for signalling of the presence of weighted prediction table whether it may be present in PH or in SH. The syntax table for weighted prediction is also updated to remove dependency to the signalling of number of active reference pictures when it is present in PH.

# Introduction

At the 16th JVET meeting, it was agreed to move signalling of syntax elements that is applied at picture level to picture header (PH). It was also agreed to have overriding mechanism using a flag is signalled in PH to specify whether signalling for a particular tool is present in PH or in slice header (SH). PH should contain syntax elements that apply at picture level, i.e., applied to all slices of the picture associated with the picture header. When a particular syntax element applies to certain slice only, it should not be signalled in picture header but in slice header.

In the current design, there are several tools / features of which syntax elements related to these tools may be present in picture header and also can be overriden by signalling them in slice header. Such design offers flexibility that tool control can be both picture level and slice level. However, it causes a burden to decoder since it needs to check both picture header and then still have to check slice header. It may be argued whether such flexibility is really needed or some simplification is desireable.

In our assertion, allowing picture level and slice level signalling to change at every picture header is not necessary. In many use-cases, for a coding tool that may be applicable at both picture level and slice level, encoder is implemented to operate either at picture level or slice level and not changing between these two frequently.

# Proposals

We propose the following:

- For signalling override mechanism between picture header and slice header, instead of signalling a flag for a particular tool to specify whether the signalling is present in picture header or in slice header, the flag is signalled in PPS instead.

- The above override mechanism may be applied to signalling of syntax element related to the following tools: reference picture lists, SAO, ALF, and deblocking.

The necessary text for the above proposed items are given below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rpl_present_in_ph_flag | u(1) |
| sao_present_in_ph_flag | u(1) |
| alf_present_in_ph_flag | u(1) |
| … |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| if( pps_weighted_pred_flag ¦¦ pps_weighted_bipred_flag ¦¦ rpl_present_in_ph_flag ) |  |
| weighted_pred_table_present_in_ph_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_present_in_ph_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| constant_slice_header_params_enabled_flag | u(1) |
| … |  |
| } |  |

…

rpl_present_in_ph_flag equal to 1 specifies the reference picture list signalling is not present in the slice headers referring to the PPS but may be present in the PHs referring to the PPS. rpl_present_in_ph_flag equal to 0 specifies the reference picture list signalling is not present in the PHs referring to the PPS but may be present in the slice headers referring to the PPS.

sao_present_in_ph_flag equal to 1 specifies the syntax elements for enabling SAO use is not present in the slice headers referring to the PPS but may be present in the PHs referring to the PPS. sao_present_in_ph_flag equal to 0 specifies the syntax elements for enabling SAO use is not present in the PHs referring to the PPS but may be present in the slice headers referring to the PPS.

alf_present_in_ph_flag equal to 1 specifies the syntax elements for enabling ALF use is not present in the slice headers referring to the PPS but may be present in the PHs referring to the PPS. alf_present_in_ph_flag equal to 0 specifies the syntax elements for enabling ALF use is not present in the PHs referring to the PPS but may be present in the slice headers referring to the PPS.

…

weighted_pred_table_present_in_ph_flag equal to 1 specifies that weighted prediction table is not present in the slice headers referring to the PPS but may be present in the PHs referring to the PPS. weighted_pred_table_present_in_ph_flag equal to 0 specifies that weighted prediction table is not present in the PHs referring to the PPS but may be present in the slice headers referring to the PPS. When not present, the value of weighted_pred_table_present_in_ph_flag is inferred to be equal to 0.

…

deblocking_filter_override_enabled_flag equal to 1 specifies that deblocking filter override may be present in PHs or in slice headers referring to the PPS. deblocking_filter_override_enabled_flag equal to 0 specifies that that deblocking filter override is not present in PHs nor in slice headers referring to the PPS. When not present, the value of deblocking_filter_override_enabled_flag is inferred to be equal to 0.

deblocking_filter_override_present_in_ph_flag equal to 1 specifies that deblocking filter override is not present in the slice headers referring to the PPS but may be present in the PHs referring to the PPS. deblocking_filter_override_present_in_ph_flag equal to 0 specifies that deblocking filter override is not present in the PHs referring to the PPS but may be present in slice headers referring to the PPS.

…

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| if( rpl_present_in_ph_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && weighted_pred_table_present_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| … |  |
| if( sps_sao_enabled_flag && sao_present_in_ph_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| if( sps_alf_enabled_flag && alf_present_in_ph_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag && deblocking_filter_override_present_in_ph_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pic_rpl_present_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| if( !rpl_present_in_ph_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| if( slice_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| slice_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( slice_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| slice_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( rpl_present_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| if( slice_type != I ) { |  |
| … |  |
| if( ( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) && !weighted_pred_table_present_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( sps_sao_enabled_flag && !sao_present_in_ph_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag && !alf_present_in_ph_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if( deblocking_filter_override_enabled_flag && !deblocking_filter_override_present_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( weighted_pred_table_present_in_ph_flag ) |  |
| num_l0_weighted_ref_pics | ue(v) |
| for( i = 0; i < num_l0_weighted_ref_pics; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < num_l0_weighted_ref_pics; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < num_l0_weighted_ref_pics; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| if( weighted_pred_table_present_in_ph_flag ) |  |
| num_l1_weighted_ref_pics | ue(v) |
| for( i = 0; i < num_l1_weighted_ref_pics; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < num_l1_weighted_ref_pics; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < num_l1_weighted_ref_pics; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

num_l0_weighted_ref_pics specifies the number of reference pictures in reference picture list 0 that are weighted. The value of num_l0_weighted_ref_pics shall ranges from 0 to num_ref_entries[ 0 ][ PicRplsIdx[ 0 ] ], inclusive. When not present, the value of num_l0_weighted_ref_pics is inferred to be equal to NumRefIdxActive[ 0 ].

num_l1_weighted_ref_pics specifies the number of reference pictures in reference picture list 1 that are weighted. The value of num_l1_weighted_ref_pics shall ranges from 0 to num_ref_entries[ 0 ][ PicRplsIdx[ 0 ] ], inclusive. When not present, the value of num_l0_weighted_ref_pics is inferred to be equal to NumRefIdxActive[ 1 ].

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).