| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0209 |
| --- | --- |

| Title: | [AHG9][AHG15]: On chroma Qp offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email: | dr.hendry@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution asserted that the current signalling mechanism for chroma Qp offsets and chroma Qp offset list may be improved in the following aspects:

- Consistency in the signalling related to chroma Qp offsets in transform unit and palette coding syntax table. Currently in transform unit syntax table, syntax element cu_chroma_qp_offset_idx is present only if there is more than one chroma Qp offset is present in the chroma Qp offset list signalled in PPS. This condition should be applicable as well for chroma Qp offset index signalling in palette coding syntax table.

- In the PPS, the syntax element pps_cb_qp_offset, pps_cr_qp_offset, and pps_joint_cbcr_qp_offset are signalled separately from the list of cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ]. However, when used for deriving the values of Qp′Cb and Qp′Cr, and Qp′CbCr, the values pps_cb_qp_offset is always together with CuQpOffsetCb as shown in equation (8-929). Likewise for pps_cr_qp_offset and pps_joint_cbcr_qp_offset. Such design is not optimal and they should be considered signalled together.

This contribution proposes the following:

- Synchronize the signalling related to chroma Qp offset in palette coding syntax table with the same signalling in transform unit syntax table.

- Simplify the chroma Qp offset signalling in PPS by combining the signalling of pps_cb_qp_offset, pps_cr_qp_offset, and pps_joint_cbcr_qp_offset with the list of cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ].

# Problem statements

## Problem #1

When the value of cu_chroma_qp_offset_flag is equal to 1, the syntax element cu_chroma_qp_offset_idx is always signalled regardless the value of chroma_qp_offset_list_len_minus1. This may cause redundancy when the value of chroma_qp_offset_list_len_minus1 is equal to 0 (i.e., there is only one set of Qp chroma offset in cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ]). When the value of chroma_qp_offset_list_len_minus1 is equal to 0, the only possible value of cu_chroma_qp_offset_idx is 0 so there is no need for signalling it. This has been done for the signalling in transform unit syntax table so that it is naturally if we also do it similarly for palette coding syntax table. Snippets below show the signalling related to the chroma Qp offsets in palettle coding and transform unit syntax tables. The condition difference for signalling the syntax element cu_chroma_qp_offset_idx between the two syntax tables is shown in the highlighted text

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| … |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ) && treeType != DUAL_TREE_LUMA ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

## Problem #2

The signalling of syntax elements pps_cb_qp_offset, pps_cr_qp_offset, and pps_joint_cbcr_qp_offset are signalled separately from the list of cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ] in PPS. However, when used for deriving the values of Qp′Cb and Qp′Cr, and Qp′CbCr, the values pps_cb_qp_offset is always together with CuQpOffsetCb as shown in equation (8-929). Likewise for pps_cr_qp_offset and pps_joint_cbcr_qp_offset. Such design is not optimal as the summation (i.e., pps_cb_qp_offset + CuQpOffsetCb, pps_cr_qp_offset + CuQpOffsetCr, and pps_joint_cbcr_qp_offset + CuQpOffsetCbCr) will have to be done possibly at every coding unit (CU). This can be prevented if the syntax elements pps_cb_qp_offset, pps_cr_qp_offset, pps_joint_cbcr_qp_offset, cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ] can be merged.

Snippet from PPS below show the signaling of chroma Qp offsets and chroma Qp offset list. The signalled cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ] are used to derive variable CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr as defined in the semantics of cu_chroma_qp_offset_idx.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

cu_chroma_qp_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ] that is used to determine the value of CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr. When present, the value of cu_chroma_qp_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1, inclusive. When not present, the value of cu_chroma_qp_offset_idx is inferred to be equal to 0.

When cu_chroma_qp_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

- The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are derived as follows:

- If cu_chroma_qp_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ cu_chroma_qp_offset_idx ]	(186)

CuQpOffsetCr = cr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(187)

CuQpOffsetCbCr = joint_cbcr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(188)

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are all set equal to 0.

While chroma Qp offsets and chroma Qp offset list are signalled separately in the PPS, at the block level when they are used, they are always used together as shown in the equations below. Thus, it makes more sense if these signalling be simplified by merging them.

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	….

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset + CuQpOffsetCb )
	 + QpBdOffset		(1122)

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset + CuQpOffsetCr )
	 + QpBdOffset		(1123)

Qp′CbCr = Clip3( −QpBdOffset, 63, qPCbCr + pps_joint_cbcr_qp_offset + 
	slice_joint_cbcr_qp_offset +CuQpOffsetCbCr ) + QpBdOffset	(1124)

# Proposals

We propose the followings:

- Synchronize the signalling related to chroma Qp offset in palette coding syntax table with the same signalling in transfrom unit syntax table. The highlighted text is addition.

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| … |  |
| } |  |

- Simplify the chroma Qp offset signalling in PPS by combining the signalling of pps_cb_qp_offset, pps_cr_qp_offset, and pps_joint_cbcr_qp_offset with the list of cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ]. The unified list may be called pps_cb_qp_offset_list[ ], pps_cr_qp_offset_list[ ], and pps_joint_cbcr_qp_offset_list[ ]. The values of the syntax elements in the unified list are set as described below. We believe that this simplification would not not affect the signalling at transform unit syntax table nor at palette coding syntax table.

- pps_cb_qp_offset_list[ 0 ], pps_cr_qp_offset_list[ 0 ], and pps_joint_cbcr_qp_offset_list[ 0 ] have the same values as current pps_cb_qp_offset, pps_cr_qp_offset, and pps_joint_cbcr_qp_offset.

- pps_cb_qp_offset_list[ 1 ], pps_cr_qp_offset_list[ 1 ], and pps_joint_cbcr_qp_offset_list[ 1 ] have the same values as pps_cb_qp_offset + cb_qp_offset_list[ 0 ], pps_cr_qp_offset + cr_qp_offset_list[ 0 ], and pps_joint_cbcr_qp_offset + joint_cbcr_qp_offset_list[ 0 ].

- pps_cb_qp_offset_list[ n ], pps_cr_qp_offset_list[ n ], and pps_joint_cbcr_qp_offset_list[ n ] have the same values as current pps_cb_qp_offset + cb_qp_offset_list[ n − 1 ], pps_cr_qp_offset + cr_qp_offset_list[ n − 1 ], and pps_joint_cbcr_qp_offset + joint_cbcr_qp_offset_list[ n − 1 ].

Updates to the VVC specs for supporting the above proposed items are provided in the attached spec text. The first attachment provides text for supporting the first two proposed items. The second attachment provides text for supporting all the proposed items.

The snippets from PPS, palette coding, and transform unit syntax tables below show the signalling changes incorporating the first two proposal above.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| pps_chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= pps_chroma_qp_offset_list_len_minus1; i++ ) { |  |
| pps_cb_qp_offset_list[ i ] | se(v) |
| pps_cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && pps_chroma_qp_offset_list_len_minus1 > 1 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| … |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ) && treeType != DUAL_TREE_LUMA ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && pps_chroma_qp_offset_list_len_minus1 > 1 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).