| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0215-v3 |
| --- | --- |

| Title: | AHG9/AHG12: Comments on signalling subpicture layout |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes some subpicture position and size value inference as an improvement on signalling the subpicture layout in the SPS.

- infer the size information when only one subpicture in the picture. When sps_num_subpics_minus1 is equal to 0, the subpicture size is the picture size.

- infer the top-left positon for the first subpicture and infer the last subpicture position when more than one subpicture in the picture. The top-left positon for the first subpicture is [0, 0] and the last subpicture is the right and bottom area of the picture.

In JVET-Q0215-v2.docx, the derivation for the last subpicture is added. In JVET-Q0215-v3.docx, some cleanups are made.

Introduction

In the VVC Draft [1], the subpicture layout is specified in the SPS as highlighted in grey.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | u(4) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| ref_pic_resampling_enabled_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Proposal

This contribution proposes some improvements on the signalling syntax for the subpicture layout in the SPS.

- infer the size information when only one subpicture in the picture. When sps_num_subpics_minus1 is equal to 0, the subpicture size is the picture size.

- infer the top-left positon for the first subpicture and infer the last subpicture position when more than one subpicture in the picture. The top-left positon for the first subpicture is [0, 0] and the last subpicture is the right and bottom area of the picture.

The proposed changes are as follows and highlighed in green.

| Subpics_present_flag | u(1) |
| --- | --- |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| if( sps_num_subpics_minus1 ) { |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| if (i < sps_num_subpics_minus1) { |  |
| if( i > 0) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| } |  |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| } |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When sps_num_subpics_minus1 is greater than 0, for i equal to sps_num_subpics_minus1, the value of subpic_width_minus1[ i ] is derived as follows.

subpic_ctu_top_left_x[ i ] =  Ceil( pic_width_max_in_luma_samples / CtbSizeY ) - subpic_width_minus1[ i ] - 1

Otherwise when not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When sps_num_subpics_minus1 is greater than 0, for i equal to sps_num_subpics_minus1, the value of subpic_height_minus1[ i ] is derived as follows.

subpic_ctu_top_left_y[ i ] = Ceil( pic_height_max_in_luma_samples / CtbSizeY ) - subpic_height_minus1[ i ] - 1

Otherwise when not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When sps_num_subpics_minus1 is greater than 0, for i equal to sps_num_subpics_minus1, the value of subpic_width_minus1[ i ] is derived as follows. Otherwise when not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

remainingWidthInCtbsY = Ceil( pic_width_max_in_luma_samples / CtbSizeY )

for( j = 0; j  <   sps_num_subpics_minus1; j++ )

if( ( subpic_ctu_top_left_y[ j ] + subpic_height_minus1[ j ] + 1 ) == PicHeightInCtbsY )

remainingWidthInCtbsY -= subpic_width_minus1[ j ] + 1

subpic_width_minus1[ i ] = remainingWidthInCtbsY - 1

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When sps_num_subpics_minus1 is greater than 0, for i equal to sps_num_subpics_minus1, the value of subpic_height_minus1[ i ] is derived as follows. Otherwise when not present, the value of subpic_height_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

remainingHeightInCtbsY = Ceil( pic_height_max_in_luma_samples / CtbSizeY )

for( j = 0; j  <   sps_num_subpics_minus1; j++ )

if( ( subpic_ctu_top_left_x[ j ] + subpic_width_minus1[ j ] + 1  ) == PicWidthInCtbsY )

remainingHeightInCtbsY -= subpic_height_minus1[ j ] + 1

subpic_height_minus1[ i ] = remainingHeightInCtbsY - 1

Conclusion

This contribution proposes some subpicture position and size value inference as an improvement on signalling the subpicture layout in the SPS. The Proponents suggest the group adopt the proposed syntax improvement.

References

- “Versatile Video Coding (Draft 7),” JVET-P2001-vE, October 2019, Geneva, Switzerland.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).