| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0216-v1 |
| --- | --- |

| Title: | On Picture Timing Information Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

A syntax modification is proposed to picture timing SEI message. It is proposed that when cpb_removal_delay_delta_enabled_flag[i] is equal to 1 for i in the range of TemporalId to bp_max_sub_layers_minus1-1, inclusive the syntax element cpb_removal_delay_delta_idx[i] be only signalled if num_cpb_removal_delay_deltas_minus1 is greater than 0. Otherwise the value of cpb_removal_delay_delta_idx[i] is inferred.

# Introduction

VVC WD [1] includes HRD which supports decoding unit based HRD signalling and operation. This is achieved by signalling in picture timing SEI message, and decoding unit SEI message and SPS.

A syntax modification is proposed to picture timing SEI message. It is proposed that when cpb_removal_delay_delta_enabled_flag[i] is equal to 1 for i in the range of TemporalId to bp_max_sub_layers_minus1-1, inclusive the syntax element cpb_removal_delay_delta_idx[i] be only signalled if num_cpb_removal_delay_deltas_minus1 is greater than 0. Otherwise the value of cpb_removal_delay_delta_idx[i] is inferred.

# Proposal

A syntax modification is proposed to picture timing SEI message. It is proposed that when cpb_removal_delay_delta_enabled_flag[i] is equal to 1 for i in the range of TemporalId to bp_max_sub_layers_minus1-1, inclusive the syntax element cpb_removal_delay_delta_idx[i] be only signalled if num_cpb_removal_delay_deltas_minus1 is greater than 0. Otherwise the value of cpb_removal_delay_delta_idx[i] is inferred. This saves bits.

The proposed changes are shown highlighted compared with VVC WD.

D.2.3 Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ] | u(v) |
| if( alt_cpb_params_present_flag ) { |  |
| cpb_alt_timing_info_present_flag | u(1) |
| if( cpb_alt_timing_info_present_flag ) { |  |
| for( i = 0; i < bp_cpb_cnt_minus1 + 1; i++ ) { |  |
| cpb_alt_initial_cpb_removal_delay_delta[ i ] | u(v) |
| cpb_alt_initial_cpb_removal_offset_delta[ i ] | u(v) |
| } |  |
| cpb_delay_offset | u(v) |
| dpb_delay_offset | u(v) |
| } |  |
| } |  |
| for( i = TemporalId; i < bp_max_sublayers_minus1; i++ ) { |  |
| pt_sublayer_delays_present_flag[ i ] | u(1) |
| if( pt_sublayer_delays_present_flag[ i ] ) { |  |
| if( cpb_removal_delay_deltas_present_flag ) |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| if( num_cpb_removal_delay_deltas_minus1 > 0 ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag && decoding_unit_cpb_params_in_pic_timing_sei_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| for( i = TemporalId; i <= bp_max_sublayers_minus1; i++ ) |  |
| if( pt_sublayer_delays_present_flag[ i ] ) |  |
| du_common_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| for( j = TemporalId; j <= bp_max_sublayers_minus1; j++ ) |  |
| if( pt_sublayer_delays_present_flag[ j ] ) |  |
| du_cpb_removal_delay_increment_minus1[ i ][ j ] | u(v) |
| } |  |
| } |  |
| if( additional_concatenation_info_present_flag ) |  |
| delay_for_concatenation_ensured_flag | u(1) |
| } |  |

cpb_removal_delay_delta_enabled_flag[ i ] equal to 1 specifies that cpb_removal_delay_delta_idx[ i ] is present in the PT SEI message. cpb_removal_delay_delta_enabled_flag[ i ] equal to 0 specifies that cpb_removal_delay_delta_idx[ i ] is not present in the PT SEI message. When not present, the value of cpb_removal_delay_delta_enabled_flag[ i ] is infered to be equal to 0.

cpb_removal_delay_delta_idx[ i ] specifies the index of the CPB removal delta that applies to Htid equal to i in the list of cpb_removal_delay_delta[ j ] for j ranging from 0 to num_cpb_removal_delay_deltas_minus1, inclusive. The length of cpb_removal_delay_delta_idx[ i ] is Ceil( Log2( num_cpb_removal_delay_deltas_minus1 + 1 ) ) bits. When cpb_removal_delay_delta_idx[ i ] is not present and cpb_removal_delay_delta_enabled_flag[ i ] is equal to 1 and num_cpb_removal_delay_deltas_minus1 is equal to 0, the value of cpb_removal_delay_delta_idx[ i ] is inferred to be equal to 0.

# Conclusion

This document proposes modifications to picture timing information signalling in VVC. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-P2001-vE, Versatile Video Coding (Draft 7), Geneva, October 2019