| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0217-v1 |
| --- | --- |

| Title: | On Reference Picture List Information Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

We propose modifications to the reference picture list information signaling for cleanup and bug-fix purposes.

Following is proposed:

- Proposal 1:  Asserted bug-fixes are proposed to the semantics of slice_rpl_sps_flag[ i ].

- Proposal 2:  It is proposed to specify an inference rule for the syntax element pps_ref_pic_list_sps_idc[ i ]  when it is not present.

- Proposal 3: Asserted bug-fix is proposed for the semantics of rpl1_idx_present_flag to correctly account for the reference picture list related signalling in picture header and slice header.

- Proposal 4:  It is proposed to move the location of syntax element rpl1_idx_present_flag in PPS and it is proposed to conditionally signal rpl1_idx_present_flag based on the value of pps_ref_pic_list_sps_idc[ 1 ].

- Proposal 5: The condition for calculation of AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] is modified to signal the value as a “minus1” for the 0-th entry.

- Proposal 6: A bug-fix is proposed for the signalling of POC LSB information related syntax to correctly use the derived variable related to picture header.

- Proposal 7: Editorial improvement is proposed for ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] since the information related to this can be now present in picture header or slice header.

# Introduction

VVC WD [1] includes reference picture management based on reference picture list signaling.

We propose modifications to the reference picture list information signaling.

Following is proposed:

- Proposal 1:

It is asserted that currently there is following bug in the semantics for slice_rpl_sps_flag[ i ]:

When pic_rpl_present_flag is equal to 0, the inference of slice_rpl_sps_flag[ i ] when not present, does not correctly account for the presence and use of pps_ref_pic_list_sps_idc[ i ].

The bug-fix is proposed to the semantics of slice_rpl_sps_flag[ i ] to fix this.

- Proposal 2: It is asserted that the signalling of syntax elements: pic_rpl_sps_flag[ i ], slice_rpl_sps_flag[ i ] use an if condition which includes checking the value of pps_ref_pic_list_sps_idc[ i ]. However the syntax element pps_ref_pic_list_sps_idc[ i ] is not present under certain conditions.

It is proposed to specify an inference rule for the syntax element pps_ref_pic_list_sps_idc[ i ]   when not present.

- Proposal 3:

A bug-fix is proposed for the semantics of rpl1_idx_present_flag to correctly account for the reference picture list related signalling in picture header and slice header.

- Proposal 4:

It is observed that rpl1_idx_present_flag controls the presence of both the index (pic_rpl_idx[ 1 ], slice_rpl_idx[ 1 ]) and the flags (pic_rpl_sps_flag[ 1 ], slice_rpl_sps_flag[ 1 ]). As a result, it is asserted that the syntax elements in PPS pps_ref_pic_list_sps_idc[ 1 ] and rpl1_idx_present_flag both independently specify information regarding the presence of syntax element pic_rpl_sps_flag[ 1 ] in PHs referring to the PPS or slice_rpl_sps_flag[ i ] in slice header referring to the PPS.

Currently both these syntax elements (pps_ref_pic_list_sps_idc[ 1 ] and rpl1_idx_present_flag) are signalled independently and it is asserted that it is currently possible to signal contradictory values for these syntax elements. To avoid this following is proposed:

  - It is proposed to move the location of syntax element rpl1_idx_present_flag in PPS.

  - It is proposed to conditionally signal rpl1_idx_present_flag based on the value of pps_ref_pic_list_sps_idc[ 1 ].

- Proposal 5:

The condition for calculation of AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] is modified to signal the value as a “minus1” for the 0-th entry. It is asserted that since for the 0-th entry the abs_delta_poc_st[ listIdx ][ rplsIdx ][ 0 ]  specifies the delta between the picture order count values of the current picture and the picture referred to by the 0-th entry, this value can be coded with a minus1 coding. This provides bit-savings as the syntax element abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] is ue(v) coded.

- Proposal 6: A bug-fix is proposed for the signalling of POC LSB information related syntax to correctly use the derived variable related to picture header.

- Proposal 7: Editorial improvement is proposed for ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] since the information related to this can be now present in picture header or slice header.

# Proposal

## Proposal 1

It is asserted that currently there is following bug in the semantics for slice_rpl_sps_flag[ i ]:

When pic_rpl_present_flag is equal to 0, the inference of slice_rpl_sps_flag[ i ] when not present, does not correctly account for the presence and use of pps_ref_pic_list_sps_idc[ i ]. It is proposed to fix this.

The proposed bug-fix to the semantics of slice_rpl_sps_flag[ i ] is shown highlighted below compared VVC WD7.

slice_rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i of the current slice is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. slice_rpl_sps_flag[ i ] equal to 0 specifies that reference picture list i of the current slice is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in the slice headers of the current picture.

When slice_rpl_sps_flag[ i ] is not present, the following applies:

- If pic_rpl_present_flag is equal to 1, the value of slice_rpl_sps_flag[ i ] is inferred to be equal to pic_rpl_sps_flag[ i ].

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is equal to 0, the value of ref_pic_list_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is greater than 0 and if rpl1_idx_present_flag is equal to 0, the value of slice_rpl_sps_flag[ 1 ] is inferred to be equal to slice_rpl_sps_flag[ 0 ].

- Otherwise,  the value of slice_rpl_sps_flag[ i ] is inferred to be equal to pps_ref_pic_list_sps_idc[ i ] − 1.

## Proposal 2

It is asserted that the signalling of syntax elements: pic_rpl_sps_flag[ i ], slice_rpl_sps_flag[ i ] use an if condition which includes checking the value of pps_ref_pic_list_sps_idc[ i ] as follows:

| for( i = 0; i < 2; i++ ) { |  |
| --- | --- |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |

and

| for( i = 0; i < 2; i++ ) { |  |
| --- | --- |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |

However the syntax element pps_ref_pic_list_sps_idc[ i ]   is conditionally present only when constant_slice_header_params_enabled_flag is equal to 1. It is proposed to specify an inference rules for the syntax element pps_ref_pic_list_sps_idc[ i ]   when not present.

The proposed bug-fix for the semantics of slice_rpl_sps_flag[ i ] are shown highlighted below compared to VVC WD7.

pps_ref_pic_list_sps_idc[ i ] equal to 0 specifies that the syntax element pic_rpl_sps_flag[ i ] is present in PHs referring to the PPS or slice_rpl_sps_flag[ i ] is present in slice header referring to the PPS. pps_ref_pic_list_sps_idc[ i ] equal to 1 or 2 specifies that the syntax element pic_rpl_sps_flag[ i ] is not present in PHs referring to the PPS and slice_rpl_sps_flag[ i ] is not present in slice header referring to the PPS. pps_ref_pic_list_sps_idc[ i ] equal to 3 is reserved for future use by ITU-T | ISO/IEC.

When not present pps_ref_pic_list_sps_idc[ i ] is inferred to be equal to 0, for i in the range of 0 to 1, inclusive.

## Proposal 3

A bug-fix is proposed for the semantics of rpl1_idx_present_flag to correctly account for the reference picture list related signalling in picture header and slice header.

The proposed modifications are shown highlighted below compared to VVC WD7:

rpl1_idx_present_flag equal to 0 specifies that pic_rpl_sps_flag[ 1 ] and pic_rpl_idx[ 1 ] are not present in the picture header referring to the PPS and ref_pic_listslice_rpl_sps_flag[ 1 ] and ref_pic_listslice_rpl_idx[ 1 ] are not present in slice headers referring to the PPS. rpl1_idx_present_flag equal to 1 specifies that pic_rpl_sps_flag[ 1 ] and pic_rpl_idx[ 1 ] may be present in the picture header referring to the PPS and ref_pic_listslice_rpl_sps_flag[ 1 ] and ref_pic_listslice_rpl_idx[ 1 ] may be present in slice headers referring to the PPS.

## Proposal 4

It is observed that rpl1_idx_present_flag controls the presence of both the index (pic_rpl_idx[ 1 ], slice_rpl_idx[ 1 ]) and the flags (pic_rpl_sps_flag[ 1 ], slice_rpl_sps_flag[ 1 ]). As a result, it is asserted that the syntax elements in PPS pps_ref_pic_list_sps_idc[ 1 ] and rpl1_idx_present_flag both independently specify information regarding the presence of syntax element pic_rpl_sps_flag[ 1 ] in PHs referring to the PPS or slice_rpl_sps_flag[ i ] in slice header referring to the PPS.

Currently both these syntax elements (pps_ref_pic_list_sps_idc[ 1 ] and rpl1_idx_present_flag) are signalled independently and it is asserted that it is currently possible to signal contradictory values for these syntax elements. To avoid this following is proposed:

- It is proposed to move the location of syntax element rpl1_idx_present_flag in PPS.

- It is proposed to conditionally signal rpl1_idx_present_flag based on the value of pps_ref_pic_list_sps_idc[ 1 ].

The proposed modifications are shown highlighted compared to VVC WD7, with changes from proposal 3 above also shown highlighted.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| scaling_window_flag | u(1) |
| if( scaling_window_flag ) { |  |
| scaling_win_left_offset | ue(v) |
| scaling_win_right_offset | ue(v) |
| scaling_win_top_offset | ue(v) |
| scaling_win_bottom_offset | ue(v) |
| } |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| if( !no_pic_partition_flag ¦ ¦ entropy_coding_sync_enabled_flag ) |  |
| entry_point_offsets_present_flag | u(1) |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| if( sps_transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |
| if( !constant_slice_header_params_enabled_flag ¦ ¦ (constant_slice_header_params_enabled_flag && (pps_ref_pic_list_sps_idc[ 1 ] == 0 ¦ ¦ pps_ref_pic_list_sps_idc[ 1 ] == 3)) ) |  |
| rpl1_idx_present_flag | u(1) |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_ref_pic_list_sps_idc[ i ] equal to 0 specifies that the syntax element pic_rpl_sps_flag[ i ] is present in PHs referring to the PPS or slice_rpl_sps_flag[ i ] is present in slice header referring to the PPS. pps_ref_pic_list_sps_idc[ i ] equal to 1 or 2 specifies that the syntax element pic_rpl_sps_flag[ i ] is not present in PHs referring to the PPS and slice_rpl_sps_flag[ i ] is not present in slice header referring to the PPS. pps_ref_pic_list_sps_idc[ i ] equal to 3 is reserved for future use by ITU-T | ISO/IEC.

…

rpl1_idx_present_flag equal to 0 specifies that pic_rpl_sps_flag[ 1 ] and pic_rpl_idx[ 1 ] are not present in the picture header referring to the PPS and ref_pic_listslice_rpl_sps_flag[ 1 ] and ref_pic_listslice_rpl_idx[ 1 ] are not present in slice headers referring to the PPS. rpl1_idx_present_flag equal to 1 specifies that pic_rpl_sps_flag[ 1 ] and pic_rpl_idx[ 1 ] may be present in the picture header referring to the PPS and ref_pic_listslice_rpl_sps_flag[ 1 ] and ref_pic_listslice_rpl_idx[ 1 ] may be present in slice headers referring to the PPS.

When not present rpl1_idx_present_flag is inferred to be equal to 0.

Additionally, the inference rules are changed as follows:

pic_rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i for the picture associated with the PH is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. ref_pic_list_sps_flag[ i ] equal to 0 specifies that reference picture list i of the picture is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in the PH.

When pic_rpl_sps_flag[ i ] is not present, the following applies:

- If num_ref_pic_lists_in_sps[ i ] is equal to 0, the value of pic_rpl_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise if num_ref_pic_lists_in_sps[ i ] is greater than 0 and rpl1_idx_present_flag is equal to 0, the value of pic_rpl_sps_flag[ 1 ] is inferred to be equal to pic_rpl_sps_flag[ 0 ].

- Otherwise,  if rpl1_idx_present_flag is not present, the value of pic_rpl_sps_flag[ i ] is inferred to be equal to pps_ref_pic_list_sps_idc[ i ] − 1.

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is greater than 0 and rpl1_idx_present_flag is equal to 0, the value of pic_rpl_sps_flag[ 1 ] is inferred to be equal to pic_rpl_sps_flag[ 0 ].

pic_rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element pic_rpl_idx[ i ] is represented by Ceil( Log2( num_ref_pic_lists_in_sps[ i ] ) ) bits.

When not present, the value of pic_rpl_idx[ i ] is inferred to be equal to 0. The value of pic_rpl_idx[ i ] shall be in the range of 0 to num_ref_pic_lists_in_sps[ i ] − 1, inclusive.

When pic_rpl_sps_flag[ i ] is equal to 1 and num_ref_pic_lists_in_sps[ i ] is equal to 1, the value of pic_rpl_idx[ i ] is inferred to be equal to 0.

When pic_rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of pic_rpl_idx[ 1 ] is inferred to be equal to pic_rpl_idx[ 0 ].

The variable PicRplsIdx[ i ] is derived as follows:

PicRplsIdx[ i ] = pic_rpl_sps_flag[ i ] ? pic_rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]

slice_rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i of the current slice is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. slice_rpl_sps_flag[ i ] equal to 0 specifies that reference picture list i of the current slice is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in the slice headers of the current picture.

When slice_rpl_sps_flag[ i ] is not present, the following applies:

- If pic_rpl_present_flag is equal to 1, the value of slice_rpl_sps_flag[ i ] is inferred to be equal to pic_rpl_sps_flag[ i ].

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is equal to 0, the value of ref_pic_list_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise,  if rpl1_idx_present_flag is not present, the value of slice_rpl_sps_flag[ i ] is inferred to be equal to pps_ref_pic_list_sps_idc[ i ] − 1.

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is greater than 0 and if rpl1_idx_present_flag is equal to 0, the value of slice_rpl_sps_flag[ 1 ] is inferred to be equal to slice_rpl_sps_flag[ 0 ].

## Proposal 5

The condition for calculation of AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] is modified to signal the value as a “minus1” for the 0-th entry. It is asserted that since for the 0-th entry the abs_delta_poc_st[ listIdx ][ rplsIdx ][ 0 ]  specifies the delta between the picture order count values of the current picture and the picture referred to by the 0-th entry, this value can be coded with a minus1 coding. This provides bit-savings as the syntax element abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] is ue(v) coded.

The proposed bug-fix the semantics of slice_rpl_sps_flag[ i ] are shown highlighted below compared VVC WD7.

### Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idx[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] specifies the value of the variable AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] as follows:

if( (sps_weighted_pred_flag  | |  sps_weighted_bipred_flag) &&  (i != 0))
	AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] = 	abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] 	(7-136)
else
	AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] = 	 abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] + 1

The value of abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to 215 − 1, inclusive.

## Proposal 6

It is asserted that there are two different derived variable arrays, namely, PicRplsIdx[ i ] and  RplsIdx[ i ]:

The variable PicRplsIdx[ i ] is derived in the semantics of pic_rpl_idx[ i ]  during the parsing of picture header as follows:

PicRplsIdx[ i ] = pic_rpl_sps_flag[ i ] ? pic_rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]

The variable RplsIdx[ i ] is derived in the semantics of slice_rpl_idx[ i ]  during the parsing of slice header as follows:

if( pic_rpl_present_flag )
	RplsIdx[ i ] = PicRplsIdx[ i ]	
else
	RplsIdx[ i ] = slice_rpl_sps_flag[ i ] ? slice_rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]

A bug-fix is proposed for the signalling of picture POC LSB information related syntax to correctly use the derived variable PicRplsIdx[ i ] related to picture header.

The proposed modification is shown highlighted compared to VVC WD7.

#### Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ PicRplsIdxRplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ PicRplsIdxRplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Proposal 7

Editorial improvement is proposed for ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] since the information related to this can be now present in picture header or slice header.

The proposed modification is shown highlighted compared to VVC WD7.

#### Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

pic_poc_lsb_lt[ i ][ j ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th LTRP entry in the i-th reference picture list for the picture associated with the picture header. The length of the pic_poc_lsb_lt[ i ][ j ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

The variable PicPocLsbLt[ i ][ j ] is derived as follows:

PicPocLsbLt[ i ][ j ] = ltrp_in_slice_header_flag[ i ][ PicRplsIdx[ i ] ] ?	(7-46)
			pic_poc_lsb_lt[ i ][ j ] : rpls_poc_lsb_lt[ listIdx ][ PicRplsIdx[ i ] ][ j ]

### Slice header syntax

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pic_rpl_present_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| if( slice_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| slice_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( slice_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| slice_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

slice_poc_lsb_lt[ i ][ j ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th LTRP entry in the i-th reference picture list. The length of the slice_poc_lsb_lt[ i ][ j ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

The variable PocLsbLt[ i ][ j ] is derived as follows:

if( pic_rpl_present_flag )
	PocLsbLt[ i ][ j ] = PocLsbLt[ i ][ j ]		(7-126)
else
	PocLsbLt[ i ][ j ] = ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ?	(7-127)
			slice_poc_lsb_lt[ i ][ j ] : rpls_poc_lsb_lt[ listIdx ][ RplsIdx[ i ] ][ j ]

### Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idx[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] equal to 0 specifies that the POC LSBs of the LTRP entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are present in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] equal to 1 specifies that the POC LSBs of the LTRP entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are not present in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure.

# Conclusion

This document proposes modifications to reference picture list information signalling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-P2001-vE, Versatile Video Coding (Draft 7), Geneva, October 2019