| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0223-v1 |
| --- | --- |

| Title: | AHG9/AHG12: On dependent slice and slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable the dependency between slices in decoding order. There are two aspects of the dependency.

- One is the slice dependency, similar to that in HEVC. Some coding tools may be used across the dependent slice boundaries for the coding efficiency, while a design with a smaller slice size of independent or dependent slice may help in applications with low latency and IP packet size matching.

- The other is the slice header only dependency. A shorter version of the slice header (SH) may be used as the result of the SH size reduction.

# Introduction

In the VVC Draft 7 [1], the picture partition and slice layout information are specified in the PPS.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| … |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

slice: An integer number of complete tiles or an integer number of consecutive complete CTU rows within a tile of a picture that are exclusively contained in a single NAL unit.

slice header: A part of a coded slice containing the data elements pertaining to all tiles or CTU rows within a tile represented in the slice.

# Proposal

In this contribution, we propose to enable the dependency between slices in decoding order, i.e., a slice may depend on a preceding slice with some values of the syntax elements of the slice header within a tile or a picture. There are two aspects of the dependency. One is the slice dependency similar to that in HEVC which affects the slice header (SH) syntax element inference and the decoding process. The other is the SH only dependency which affects only the SH syntax element inference.

- With the slice dependency, some coding tools may be used across the dependent slice boundaries for coding efficiency, while a design with a smaller slice size of independent or dependent slice may help in applications with low latency and IP packet size matching.

- With the SH only dependency, a shorter version of the SH may be used as the result of the SH size reduction.

The proposed syntax and semantics for pic_parameter_set_rbsp( ) and slice_header( ) are as follows and highlighted in green.

Aspect 1: the slice dependency

The proposed slice depency may benefit more the raster scan slice in units of CTU as proposed in JVET-Q0224 (AHG9/AHG12: On raster scan slice within a picture).

slice: An integer number of complete tiles or an integer number of consecutive complete CTU rows within a tile of a picture or an integer number of consecutive CTUs within a picture or a tile of a picture that are exclusively contained in a single NAL unit.

slice header: A part of a coded slice containing the data elements pertaining to all tiles or CTU rows within a tile represented in the slice or consecutive CTUs within a tile or a picture represented in the slice.

dependent slice: A slice for which the values of some syntax elements of the slice header are inferred from the values for the preceding independent slice in decoding order. The CTUs contained in one dependent slice are the continuance of the consecutive CTUs in the preceding slice, and/or the continuance of the consecutive CTUs in the most recent independent slice in decoding order.

NOTE – In the current version of VVC standard, the dependent slices are applied only for the case of slices within a tile or within a picture when no tile partitions.

independent slice: A slice for which the values of the syntax elements of the slice header are not inferred from the values for a preceding slice. An independent slice may or may not have subsequent dependent slices in decoding order. An independent slice with no dependent slices forms a slice in the decoding process. One independent slice and all subsequent dependent slices (if any) that precedes the next independent slice (if any) also form a slice in the decoding process.

It is a requirement of bitstream conformance that the first slice in a tile or a picture shall be an independent slice. It is a requirement of bitstream conformance that when a dependent slice belongs to a tile, the preceding slice in decoding order shall also belong to the tile, and the most recent independent slice shall also belong to the tile.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| … |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| pps_dependent_slices_enabled_flag | u(1) |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| … |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( pps_dependent_slices_enabled_flag ) { |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) |  |
| num_dependent_slices | ue(v) |
| } |  |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| … |  |
| } |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

pps_dependent_slices_enabled_flag equal to 1 specifies the presence of the syntax element dependent_slice_flag in the slice headers for coded pictures referring to the PPS. pps_dependent_slices_enabled_flag equal to 0 specifies the absence of the syntax element dependent_slice_flag in the slice headers for coded pictures referring to the PPS.

dependent_slice_flag equal to 1 specifies that the value of each slice header syntax element that is not present is inferred to be equal to the value of the corresponding slice header syntax element in the slice header. When not present, the value of dependent_slice_flag is inferred to be equal to 0.

num_dependent_slices specifies the number of the subsequent dependent slices present in the coded picture depending on the slice header. The current slice together with all subsequent dependent slices form a slice in the decoding process. When not present, num_dependent_slices is inferred to be equal to 0.

Aspect 2: the slice header only dependency

In the aspect 1, an additional flag may be added to signal only the slice header dependency of independent slices in decoding order for the slice header size reduction. The proposed additional text may be as follows (highlighed in yellow)

dependent slice header: A slice header for which the values of some syntax elements of the slice header are inferred from the values for the preceding independent slice in decoding order.

independent slice: A slice for which the values of the syntax elements of the slice header are not inferred from the values for a preceding slice. An independent slice may or may not have subsequent dependent slices in decoding order. An independent slice with no dependent slices forms a slice in the decoding process. One independent slice and all subsequent dependent slice (if any) that precedes the next independent slice (if any) also form a slice in the decoding process. An independent slice may or may not have a dependent slice header.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( pps_dependent_slices_enabled_flag ) { |  |
| dependent_slice_header_flag | u(1) |
| if( dependent_slice_header_flag ) { |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) |  |
| num_dependent_slices | ue(v) |
| } |  |
| } |  |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( !dependent_slice_flag ¦ ¦ !dependent_slice_header_flag) { |  |
| slice_type | ue(v) |
| … |  |
| } |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

dependent_slice_header_flag equal to 1 specifies that the value of each slice header syntax element that is not present is inferred to be equal to the value of the corresponding slice header syntax element in the slice header of the preceding independent slice in decoding order. When not present, the value of dependent_slice_header_flag is inferred to be equal to 0.

# Conclusion

This contribution proposes to enable the dependency between slices in decoding order. There are two aspects of the dependency and they may be combined and applied in all or part. The proponents suggest the group adopt the support of the proposed slice related dependency.

# References

- “Versatile Video Coding (Draft 7),” JVET-P2001-vE, October 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).