| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0227 |
| --- | --- |

| Title: | Dependent Quantization with Qp offset adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. de Lagrange, F. Hiron, F. Le Léannec, E. François | Tel: Email: | Philippe.delagrange@interdigital.com franck.hiron@interdigital.com Fabrice.leleannec@interdigital.com Edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution relates to Dependent Quantization (DepQ). When DepQ applies, an qP offset of +1 is added to the qP value when applying the scaling process for transform coefficients. No offset is used when DepQ is off. In this contribution, it is proposed to adapt the offset value depending on the qP value. The reported psnr-Y, U, V BD-rate variations compared to VTM7.0 are -0.07%, 0.29%, 0.28% for the AI configuration, -0.07%, -0.23%, -0.34%for the RA configuration, -0.18%, 0.08%, -0.17% for the LDB configuration.

# Introduction

In VVC, the scaling factors ls[ x ][ y ] used for the coefficients scaling is specified as follows (see JVET-P2001-vE.docs):

–	If pic_dep_quant_enabled_flag is equal to 1 and transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(1143)

–	Otherwise (pic_dep_quant_enabled_flag is equal to 0 or transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(1144)

As observed in eq.1143, in case of Dependent quantization, an offset of +1 is applied to the qP value.

Investigations have been made on using alternate offset values, resulting in the current proposal.

# Proposal

Based on the performed investigations, it is proposed to use a qP-adaptive offset, defined as follows:

- If qP > (30 + QpBdOffset), qPOffset = 2

- Otherwise if qP > (23 + QpBdOffset), qPOffset = 1

- Otherwise, qPOffset = 0

And the formula for deriving the scaling factors ls[ x ][ y ] is specified as follows:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + qPOffset) % 6 ] ) << ( (qP + qPOffset) / 6 )

An adaptation of the lambda is also applied to take into account the change of the qP. Instead of multiplying the lambda value by 2^(0.25/3)  1.05 as currently done in case of Dependent Quantization, the lambda value is multiplied by 1.1  2^(0.5/3).

# Spec changes

The spec changes are indicated below (in yellow).

–	If pic_dep_quant_enabled_flag is equal to 1 and transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

- The value qPOffset is derived as follows:

If qP > (30 + QpBdOffset), qPOffset = 2

Otherwise if qP > (23 + QpBdOffset), qPOffset = 1

Otherwise, qPOffset = 0

- The scaling factor ls[ x ][ y ] is derived as follows

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + qPOffset) % 6 ] ) << ( (qP + qPOffset) / 6 )				(1143)

–	Otherwise (pic_dep_quant_enabled_flag is equal to 0 or transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(1144)

# Tests results

BD-rate variations relative to the VTM7.0 are reported in the tables below.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.24% | 97% | 100% |
| Class A2 | -0.32% | 0.34% | 0.26% | 101% | 99% |
| Class B | -0.06% | 0.33% | 0.17% | 97% | 99% |
| Class C | 0.01% | 0.45% | 0.37% | 97% | 99% |
| Class E | -0.03% | 0.30% | 0.41% | 100% | 103% |
| Overall | -0.07% | 0.29% | 0.28% | 98% | 100% |
| Class D | 0.00% | 0.49% | 0.49% | 101% | 98% |
| Class F | -0.08% | 0.28% | 0.34% | 101% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.21% | -0.26% | 98% | 100% |
| Class A2 | -0.20% | -0.34% | -0.47% | 101% | 100% |
| Class B | -0.08% | -0.32% | -0.31% | 99% | 98% |
| Class C | -0.02% | -0.35% | -0.34% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.23% | -0.34% | 100% | 99% |
| Class D | -0.05% | -0.38% | -0.44% | 97% | 101% |
| Class F | -0.07% | -0.08% | -0.17% | 98% | 103% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.22% | -0.12% | 100% | 103% |
| Class C | -0.26% | 0.04% | 0.04% | 101% | 100% |
| Class E | -0.04% | 0.64% | -0.54% | 99% | 101% |
| Overall | -0.18% | 0.08% | -0.17% | 100% | 101% |
| Class D | -0.02% | 0.44% | 0.19% | 98% | 96% |
| Class F | -0.34% | -0.57% | 0.52% | 100% | 100% |

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).