| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0230 |
| --- | --- |

| Title: | AHG12: Singalling of rectangular slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee (XRis Corp) Dongsan Jun (Kyungnam University) | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr dsjun9643@kyungnam.ac.kr |
| Source: | Xris Corporation, Kyungnam University |  |  |

_____________________________

- Abstract

This contribution proposes add new syntax elements for rectangular slice signalling. This contribution is claimed to reduce bit amount for signalling of rectangular slices compared to VVC draft 7.

Introduction

It is proposed to add a new syntax element multiple_slice_in_tile_present_flag and multiple_slice_in_tile_flag[i] on top of JVET-Q228 proposal 1 and proposal 2.

In this proposal, multiple_slice_in_tile_present_flag indicates whether at least one tile is composed of more than slice or not and multiple_slice_in_tile_flag[i] indicates whether a tile is composed of more than one slice or not.

Proposal

If slice_width_in_tiles_minus1[i] and slice_width_in_tiles_minus1[i] are equal to 0, then tile is composed of one slice or multiple slices. That is, syntax elements slice_width_in_tiles_minus1[i] and slice_width_in_tiles_minus1[i] are used to indicate whether a tile is composed of multiple slices.

On top of JVET-Q0228 proposal 1 and 2, it is proposed to add new syntax elements multiple_slices_in_tile_present_flag and multiple_slices_in_tile_flag[i] to indicate whether a tile is composed of multiple slices as shown in right figure of Figure1 and syntax elements num_exp_slices_in_tile[i] proposed in JVET-Q0228 proposal 2 replace into num_exp_slices_in_tile_minus1[i].

It is proposed to directly singal syntax element “multiple_slices_in_tile_flag[i]” whether a tile which contains i-th slice is composed of more than one slice.

If multiple_slices_in_tile_flag[i] is equal to 1, then a tile which contains i-th rectangular slice is composed of more than one slice. Otherwise, if multiple_slices_in_tile_flag[i] is equal to 0, then a tile which contains i-th rectangular slice is composed of more than one slice or i-th slice is composed of more than one tile.

If multiple_slices_in_tile_flag[i] is equal to 1, there is no need to singal slice_width_in_tiles_minus1[ i ] and slice_height_in_tiles_minus1[ i ]. In this case, a tile has at least 2 slices. Therefore, it is also propsed replace num_slices_in_tile_minus1[ i ] into num_slices_in_tile_minus2[ i ].

There is no need to signal slice_width_in_tiles_minus1[ i ] and slice_height_in_tiles_minus1[ i ] when the value of multiple_slices_in_tile_flag[i] is equal to 1. In addition, it is proposed to replace num_slices_in_tile_minus1[ i ] into num_slices_in_tile_minus2[ i ], since a tile has at least 2 slices in this case.

If there is no tile which contains i-th rectangular slice is composed of more than one slice in the picture, it is not efficient to signal multiple_slices_in_tile_flag[i] for each slice. Therefore, it is proposed to signal syntax element “multiple_slices_in_tile_present_flag” which is present flag of multiple_slices_in_tile_flag[i] in the picture.

In addition, num_exp_slices_in_tile[i] proposed in JVET-Q0228 proposal 2 is replaced into num_exp_slices_in_tile_minus1[i], since the number of slices explicitly provided is greater than 0 when multiple_slices_in_tile_flag[i] is euqal to 1.

multiple_slices_in_tile_present_flag indicates whether there is a tile which is composed multiple slices.

If multiple_slices_in_tile_present_flag is equal to 0, then there is no tile which is composed of multiple slices.

| VVC draft 7 | Proposal 2 |
| --- | --- |

Figure 1

The propsed syntax changes are shown in the Table 1.

Table 1

| rect_slice_flag | u(1) |
| --- | --- |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| multiple_slices_in_tile_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if (multiple_slice_in_tile_present_flag) |  |
| multiple_slices_in_tile_flag[i] | u(1) |
| if (!multiple_slices_in_tile_flag[i] ¦¦ !multiple_slices_in_tile_present_flag ){ |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| }else{ |  |
| num_exp_slices_in_tile_minus1[ i ] | ue(v) |
| numExpSlicesInTile = num_exp_slices_in_tile_minus1[ i ]+1 |  |
| for( j = 0; j < numSlicesInTile; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| if(numExpSlicesInTile ) |  |
| i += RemNumSlices |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ){ |  |
| tile_idx_delta_minus1[i] | ue(v) |
| if (i>0) |  |
| tile_idx_delta_sign[i] | u(1) |
| } |  |
| } |  |
| } |  |

multiple_slices_in_tile_present_flag is euqal to 1 indicates there is at least one tile which is composed of more than one slice. multiple_slices_in_tile_present_flag is euqal to 0 indicates there is no tile which is composed of more than one slice.

multiple_slices_in_tile_flag[i] specify whether a tile which contains i-th slice is composed of more than one slice. multiple_slices_in_tile_flag[i] is euqal to 1 indicates a tile which contains i-th rectnagular siice is composed of more than one slice. multiple_slices_in_tile_flag[i] is euqal to 1 indicates a tile which contains i-th rectnagular siice is composed of more than one slice.

num_exp_slices_in_tile_minus1[i] plus 1 specifies the slices number of explicitly provided in a tile which contains i-th slice. The value of num_exp_slices_in_tile[i] shall be in the range of 0 to RowHeight[TileY]-2, inclusive.

- 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

tileIdx = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		if( i  = =  num_slices_in_pic_minus1 ) {
			slice_width_in_tiles_minus1[ i ] = NumTileColumns − 1 − tileX
			slice_height_in_tiles_minus1[ i ] = NumTileRows − 1 − tileY
			num_slices_in_tile_minus1[ i ] = 0
		}
		if( (multiple_slice_in_tile_flag[i] && multiple_slice_in_tile_present_flag) {	(29)
			ctbY = tileRowBd[ tileY ]
			numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ]

SumExpCtuHeight = 0 ; 
			numExpSlicesInTile = num_exp_slices_in_tile_minus1[ i ]+1
			for( j = 0; j < numExpSlicesInTile; j++ ) {

AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )
				ctbY  +=  slice_height_in_ctu_minus1[ i ] + 1

SumExpCtuHeight += slice_height_in_ctu_minus1[ i ] + 1
				i++
			}
			AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, tileRowBd[ tileY + 1 ] )

RemNumSlices = (RowHeight[TileY] – SumExpCtuHeight+ slice_height_in_ctu_minus1[i-1]) / (slice_height_in_ctu_minus1[i-1] + 1)

for( j = 0; j < RemNumSlices; j++ ) {

AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + slice_height_in_ctu_minus1[ i-1 ] + 1 )

ctbY += slice_height_in_ctu_minus1[ i-1 ] + 1
			}
		}

Bit saving analysis

The following OMAF and related use cases [2] [3] are used for bit saving analysis.

Figure 2-a and Figure 2-b present the tile and slice partitioning that achieve the 6K effective ERP and CMP schemes that are described in OMAF clauses D.6.3 and D.6.4 (respectively) [2].

The scheme presented in Figure 2-c is equivalent to that in in Figure 1b but uses a different picture aspect ration

| a) 6K effective ERP scheme of OMAF clause D.6.3. 6×2 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and tile row heights of 2048 and 256 luma samples | b) 6K effective CMP scheme of OMAF clause D.6.4. 3×6 tile grid with tile column widths of 768, 768, and 384 luma samples and all tile row heights equal to 768 luma samples. | c) 6K effective CMP scheme. 6×3 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and all tile row heights equal to 768 luma samples. |
| --- | --- | --- |

Figure 2

The following Table 2 shows bit savings comparison of rectangular slices signaling in VVC darft7 and proposal #3 of use cases shown in Figure 2. The details of analysis are attached to this contribution as an excel file.

Table 2

|  | Figure 1-a | Figure 1-b | Figure 1-c |
| --- | --- | --- | --- |
|  | Number of bits | Number of bits | Number of bits |
| VVC draft7 | 70 | 94 | 94 |
| Proposal #3 | 59 | 77 | 77 |
| Bit savings ratio | 16% | 18% | 18% |

References

[1] JVET-P2001-vE, Versatile Video Coding (Draft 7), Geneva, Oct 2019

[2] N18393, WD5 of ISO/IEC 23090-2 OMAF 2nd edition

[3] JVET-P0096, AHG12: On signalling of tile and brick partitioning, Geneva, Oct 2019

Patent rights declaration(s)

Xris Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kyungnam University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).