| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0232-v1 |
| --- | --- |

| Title: | AHG8/AHG12 Subpicture-based reference picture resampling signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirabayashi Mitsuhiro Mitsuru Katsumata Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuhiro.Hirabayashi@sony.com Mitsuru.Katsumata@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes subpicture mapping syntax for subpicture-based RPR. Those inherit the syntax of RPR for the picture.

The following changes to the VVC specification related to subpicture-based RPR:

- It is proposed to signal two flags that indicate to apply subpicture-based RPR in the SPS.

  - The flag indicates that subpicture-based RPR for some subpicture may be applied.

  - The flag indicates that subpicture-based RPR for i-th subpicture may be applied.

- It is proposed to change the resolution information of subpictures in the SPS with maximum resolution information in CVS.

- It is proposed to signal a resolution information of subpictures in the PPS.

- For the upper left position of subpicture in the CVS is fixed, subpicture id mapping information is signal in the SPS.  This is sps_subpic_id_present_flag=0 or sps_subpic_id_signalling_present_flag=1.

Introduction

In the current VVC specification draft in JVET-P2001-vE, reference picture resampling (RPR) is supported and can be used for network bandwidth adaption in case of real time encoding of the live streaming service.  And the subpicture-based dynamic resolution change is also required in the following use cases.

- use case 1: 360 cubemap live streaming:  Each subpicture represents a cubemap face for a 360 cubemap projection. When the bandwidth is limited, the subpictures in the viewport are encoded with high resolution and other subpictures are encoded with low resolution.

Fig. 1 Concept of encoding of 360 cubemap live streaming. 360 cube map, 6 face subpictures.

- use case 2: 8K 2D live streaming:  The picture is 8K resolution. The picture is divided into 4x4 subpictures. When the bandwidth is limited, the subpictures in the center are encoded with high resolution and other subpictures are encoded with low resolution.

Fig. 2 Concept of encoding of the 8K 2D live streamig.  8K picture, 4x4 subpictures

Therefore, in the 16th JVET meeting (JVET-P), subpicture-based RPR was proposed on JVET-P0403 and further study was encouraged in the minutes.  In this contribution, subpicture-based RPR was achieved by changing subpicture ID mapping in the CVS without changing subpicture mapping in the CVS.

Fig. 3 The proposed subpicture-based RPR in JVET-P0403

By our consideration, the following three problems should be solved to support subpicture-based RPR

- The subpics_present_flag is also used for subpicture-based RPR use condition. Only subpicture function is required in general, so subpicture-based RPR use condition should be signaled separately.

- Due to the subpicture resolution change and subpicture position change within the picture, the processing load of encode and decode becomes higher. For example, when encoding or decoding process for each subpicture is in parallel, the pictures must be stored in a common memory and accessible by multiple cores. The independence of parallel processing is most important feature for subpicture.

- Only the resolution described in subpicture mapping could be used for the subpicture resolution change. For subpicture-based RPR, the flexibility of subpicture resolution change is also required.

This contribution proposes subpicture mapping syntax for subpicture-based RPR. Those inherit the syntax of RPR for the picture.

Proposal

The syntax of RPR for the picture is as follows:

- The ref_pic_resampling_enabled_flag in the SPS specifies that reference picture resampling may be applied in each picture.

- The maxmum resolution of picture in the CVS for deriving the DPB size is in the SPS.

- The resolution of picture for RPR process is in the PPS.

- The upper left position of picture is fixed even if the picture is resampled.

By inheriting the syntax of RPR for the picture, it is proposed as follows.

- The flag in the SPS specifies that subpicture-based RPR may be applied in each picture.

- The maxmum resolution of subpicture in the CVS is in the SPS.

- The resolution of subpicture is in the PPS.

- The upper left position of subpicture is fixed if the subpicture is resampled.

Fig. 4 concept of syntax of subpicture based RPR

Fig. 4 shows the above proposal.

Note that the decoded picture in the DPB contains unnecessary output samples because RPR process is applied for each subpicture and picture size is constant.

Then this proposal solves the problems described in introduction.

- Determine whether subpicture-based RPR is applied.

- Independence of parallel processing of subpicture

- Flexibility of subpicture resolution change

Proposal Text

The following changes to the VVC specification related to subpicture-based RPR:

- It is proposed to signal two flags that indicates to apply subpicture-based RPR ins the SPS.

  - The flag indicates that subpicture-based RPR for some subpicture may be applied.

  - The flag indicates that subpicture-based RPR for i-th subpicture may be applied.

- It is proposed to change the resolution information of subpictures in the SPS with maximum resolution information in CVS.

- It is proposed to signal a resolution information of subpictures in the PPS.

- For the upper left position of subpicture in the CVS is fixed, subpicture id mapping information is signal in the SPS.  This is sps_subpic_id_present_flag=0 or sps_subpic_id_signalling_present_flag=1.

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-P2001-vE)

The proposed syntax and semantics changes in the SPS on top of JVET-P2001-vE are as follows

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| ref_subpic_resampling_in_pic_flag | u(1) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_width_max_minus1[ i ] | u(v) |
| subpic_height_max_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| if(ref_subpic_resampling_in_pic_flag) |  |
| ref_subpic_resampling flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_minus8 | ue(v) |

Semantics related to proposal 1:

ref_subpic_resampling_in_pic_flag equal to 1 specifies that reference picture resampling for some subpicture may be applied.  ref_subpic_resampling_enabled_flag equal to 0 specifies that reference picture resampling for any subpicture is not applied.

ref_subpic_resampling_flag[ i ] equal to 1 specifies that reference picture resampling for i-th subpicture may be applied. ref_subpic_resampling_flag[ i ] equal to 0 specifies that reference picture resampling for i-th subpicture is not applied.  If ref_subpic_resampling_in_pic_flag is equal to 0, ref_subpic_resampling_flag[ i ] shall be equal to 0;

Semantics related to proposal 2:

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

subpic_width_max_minus1[ i ] plus 1 specifies the maximum width of the i-th subpicture region in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_max_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

subpic_height_max_minus1[ i ] plus 1 specifies the max height of the i-th subpicture region in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_max_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

BThe proposed syntax and semantics changes in the PPS on top of JVET-P2001-vE are as follows

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if (ref_subpic_resampling_in_pic_flag) { |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| if(ref_subpic_resampling_flag[ i ]){ |  |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |

Semantics is related to proposal 3.

Furthermore, using the ref_subpic_resampling_in_pic_flag and ref_subpic_resampling_flag[ i ] in Proposal 1, syntax can be omitted when subpicture-based RPR is not applied in each subpicture.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to subpic_width_max_minus1[ i ].

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to subpic_height_max_minus1[ i ].

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).