| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0235-v1 |
| --- | --- |

| Title: | AHG12 Simplifying the nesting condition of subpicIdList[i] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirabayashi Mitsuhiro Mitsuru Katsumata Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuhiro.Hirabayashi@sony.com Mitsuru.Katsumata@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to replace the sps_subpic_id_present_flag with the new flag indicating whether to describe subpicture ID mapping in SPS.  This new flag could simplify the nesting condition of the list subpicIdList[i].

Introduction

In the 16th JVET meeting (JVET-P), subpicture ID mapping was agreed to be signaled in SPS, PPS or PH.  The list SubpicIdList[ i ] is derived as following flow chart

Fig. 1 flow chart of deriving the list subpicIdList[i]

This contribution proposes to replace the sps_subpic_id_present_flag with the new flag that indicate whether to describe subpicture ID mapping in SPS. This flag could simplify the nesting condition of the list subpicIdList[i].

Proposal

This contribution proposes to replace the sps_subpic_id_present_flag with subpic_id_mapping_in_sps_flag indicating whether to describe subpicture ID mapping in SPS.  As shown in Fig. 2, this flag can simplify the nesting condition of the list subpicIdList [i].

Fig. 2 proposed flow chart of deriving the list subpicIdList[i]

Proposal Text

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-P2001-vE)

Below are the proposed syntax and semantics changes in the SPS on top of JVET-P2001-vE:

| seq_parameter_set_rbsp( ){ | Descriptor |
| --- | --- |
| … |  |
| sps_subpic_id_present_flag | u(1) |
| subpic_id_mapping_in_sps_flag | u(1) |
| if(ssps_subpic_id_present_flag ) { |  |
| if(subpic_id_mapping_in_sps_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS.

subpic_id_mapping_in_sps_flag equal to 1 specifies that subpicture ID mapping is decided in the SPS. subpic_id_mapping_in_sps_flag equal to 0 specifies that subpicture ID mapping is not decided in the SPS.

sps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is explicitly signalled in the SPS. sps_subpic_id_signalling_present_flag subpic_id_mapping_in_sps_flag equal to 0 specifies that subpicture ID mapping is not implicitly signalled in the SPS using subpicture mapping order. When not present, the value of sps_subpic_id_signalling_present_flag is inferred to be equal to 0.

sps_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits. When not present, and when sps_subpic_id_present_flag equal to 0 subpic_id_mapping_in_sps_flag equal to 1, the value of sps_subpic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to sps_num_subpics_minus1, inclusive

Below are the proposed semantics changes in the PPS on top of JVET-P2001-vE:

pps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PPS. pps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PPS. When sps_subpic_id_present_flag is 0 or sps_subpic_id_signalling_present_flag is equal to 1 subpic_id_mapping_in_sps_flag is equal to 1, pps_subpic_id_signalling_present_flag shall be equal to 0.

Below are the proposed syntax and semantics changes in the PH on top of JVET-P2001-vE:

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| if(!subpic_id_mapping_in_sps_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |

ph_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the ph_subpic_id[ i ] syntax element is ph_subpic_id_len_minus1 + 1 bits.

The list SubpicIdList[ i ] is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	SubpicIdList[ i ] = sps_subpic_id_present_flag ?	(76)
		( sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] :
		( ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ] ) ) : i

if (subpic_id_mapping_in_sps_flag) 
		SubpicIdList[ i ] = sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] :i
	else
		SubpicIdList[ i ] = ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ]

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).