| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0245 |
| --- | --- |

| Title: | [AHG9]: On signalling slice type information in the picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Hendry Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com dr.hendry@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the existing design of the picture header (PH) does not take into account the type of the slices within the picture associated with the PH. Consequently, it always requires both syntax elements for intra coded slices and for inter coded slices have to be present in PH even when the picture has a uniform type of slices.

This contribution proposes the following:

- Signal two new flags namely intra_signalling_present_flag and inter_signalling_present_flag in PH.

- intra_signalling_present_flag is used to condition the presence of syntax elements that are needed by intra coded slices; likewise, inter_signalling_present_flag is used to condition the presence of syntax elements that are needed by inter coded slices.

- When the picture associated with a PH contains one or more intra coded slice, the value of intra_signalling_present_flag shall be equal to 1; likewise, when the picture contains one or more inter coded slice, the value of inter_signalling_present_flag shall be equal to 1.

- When the value of intra_signalling_present_flag is equal to 1, the picture associated with the PH may or may not have intra coded slice. Encoder may set this flag to be equal to 1 even when the picture does not have intra slice in anticipation of the picture has subpictures and the subpicture containing one or more slices of the picture is extracted and merged with subpicture(s) from other bitstream.

- When the value of inter_signalling_present_flag is equal to 1, the picture associated with the PH may or may not have inter coded slices. Encoder may set this flag to be equal to 1 even when the picture does not have inter coded slice in anticipation of the picture has subpictures and the subpicture containing one or more slices of the picture is extracted and merged with subpicture(s) from other bitstream.

# Introduction

In the existing design of picture header (PH) in the current VVC Specification, a picture header contains syntax elements needed for both intra and inter coded slices. Such signalling would have redundancy in the case when the picture has a uniform slice type, which is common in many use cases.

In one of discussion sessions regarding picture header at the 16th JVET meeting, the idea of signalling slice type or picture type in PH was discussed but was not agreed due to the concern about the need to update PH in the case of subpicture merging. In our assertion, we need to signal syntax element for both intra and intra coded slices in a PH only in these two cases:

- The picture associated with the PH has slices of mixed type.

- The picture associated with the PH has uniform slice type but it is partitioned into several subpictures and it is anticipated that one or more of those subpictures could be extracted and merged with other subpicture(s) which would result the new picture contains slices with mixed type.

The PH design should be such that syntax elements needed for both intra and inter coded slices are present in a PH for the above two cases. Otherwise, we should not waste bits in the PH by signalling syntax elements that are not needed.

# Proposal

We propose the following:

- Signal two new flags in PH namely intra_signalling_present_flag and inter_signalling_present_flag in the PH.

- intra_signalling_present_flag is used to condition the presence of syntax elements that are needed by intra coded slices; likewise, inter_signalling_present_flag is used to condition the presence of syntax elements that are needed by inter coded slices

- When a picture contains one or more intra coded slice, the value of intra_signalling_present_flag in the PH associated with the picture shall be equal to 1; likewise, when the picture contains one or more inter coded slice, the value of inter_signalling_present_flag shall be equal to 1.

- When the value of intra_signalling_present_flag is equal to 1 in a PH, the picture associated with the PH may or may not have intra coded slice. Encoder may set this flag to be equal to 1 even when the picture does not have intra slice in anticipation of the subpicture containing one or more slices of the picture is extracted and merged with subpicture(s) from other bitstream.

- When the value of inter_signalling_present_flag is equal to 1 in a PH, the picture associated with the PH may or may not have inter coded slice. Encoder may set this flag to be equal to 1 even when the picture does not have inter coded slice in anticipation of the subpicture containing one or more slices of the picture is extracted and merged with subpicture(s) from other bitstream.

Table 1 Shows the syntax for the picture header

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| intra_signalling_present_flag | u(1) |
| inter_signalling_present_flag | u(1) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| if( intra_signalling_present_flag ){ |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( inter_signalling_preset_flag ){ |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( intra_signalling_present_flag ){ |  |
| if( cu_qp_delta_enabled_flag ) |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( inter_signalling_present_flag ){ |  |
| if( cu_qp_delta_enabled_flag ) |  |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| pic_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| ... |  |
| } |  |

intra_signalling_present_flag equal to 1 specifies that syntax elements used by intra coded slices are present in the picture header. intra_signalling_present_flag equal to 0 specifies that syntax elements used by intra coded slices are not present in the picture header.

It is a requirement of bitstream conformance that, the value of intra_signalling_present_flag shall be equal to 1 when the picture associated with the picture header has one or more slice with slice_type equal to I.

inter_signalling_present_flag equal to 1 specifies that syntax elements used by inter coded slices are present in the picture header. inter_signalling_present_flag equal to 0 specifies that syntax elements used by inter coded slices are not present in the picture header.

It is a requirement of bitstream conformance that, the value of inter_signalling_present_flag shall be equal to 1 when the picture associated with the picture header has one or more slice with slice_type equal to P or B.

NOTE – : The values of both intra_signalling_present_flag and inter_signalling_present_flag are set equal to 1 in the picture header associated with picture containing one or more subpictures containing intra coded slice(s) which may be merged with one or more subpicure(s) containing inter coded slices(s).

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).