| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0250 |
| --- | --- |

| Title: | CE5-related: Removing number of filters for CC-ALF in slice and picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. When CC-ALF is applied to Cb (Cr resp.) component, an ALF APS ID and the number of Cb (Cr resp.) filters in this ALF APS are signalled in picture header and slice header. When CC-ALF is applied to a Cb (Cr resp.) coding tree block (CTB), the filter index of this CTB is signalled based on and the number of Cb (Cr resp.) filters signalled at picture and slice headers. In this contribution, it is proposed to remove the number of Cb (Cr resp.) filters for CC-ALF in picture and slice header.

Specifically, the following syntax elements are removed: pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_cr_filters_signalled_minus1, slice_cross_component_cb_filters_signalled_minus1 and slice_cross_component_cr_filters_signalled_minus1

Compared to CE5 anchor, similar coding efficiency and running time are observed.

# Experimental results

The proposed methods are implemented on top of CE5 anchor with common test condition defined in JVET-N1010.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class C | 0.00% | 0.00% | -0.02% | 101% | 100% |
| Class E | 0.00% | 0.00% | -0.05% | 100% | 103% |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Class D | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class B | 0.00% | -0.04% | -0.08% | 100% | 100% |
| Class C | 0.00% | -0.05% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.04% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.04% | 100% | 100% |
| Class F | 0.00% | -0.01% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.15% | -0.07% | 99% | 100% |
| Class C | 0.02% | 0.17% | -0.13% | 100% | 99% |
| Class E | -0.03% | -0.28% | -0.10% | 100% | 101% |
| Overall | 0.00% | -0.07% | -0.10% | 100% | 100% |
| Class D | -0.01% | 0.33% | 0.03% | 100% | 101% |
| Class F | -0.05% | -0.21% | 0.45% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.49% | 100% | 98% |
| Class C | -0.02% | -0.03% | 0.00% | 100% | 101% |
| Class E | 0.01% | -0.11% | -0.11% | 100% | 100% |
| Overall | 0.00% | -0.03% | -0.23% | 100% | 99% |
| Class D | 0.01% | -0.59% | -0.20% | 100% | 101% |
| Class F | -0.04% | -0.27% | -0.04% | 100% | 101% |

# Patent rights declaration(s)

Qualcomm Inc. does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).