| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0251 |
| --- | --- |

| Title: | CE5-related: Unified cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. To get an offset for a chroma sample, the filter is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample. In an APS, a Cb (Cr resp.) CC-ALF filter set with up to 4 filters could be signalled and a filter from this Cb (Cr resp.) filter set could be applied to a Cb (Cr resp.) coding tree block. On the other hand, in an APS, a chroma adaptive loop filter (ALF) filter set with up to 8 filters could be signalled and a filter from this chroma filter set could be applied to a chroma coding tree block. In this contribution, CC-ALF filters are unified for the two chroma components, where Cb and Cr components share the same filters in an APS, which is the same method used in a regular chroma ALF. Under common test conditions, compared with CE5 anchor, the average BD rate of the proposed method is

AI   : -0.01%(Y), 0.05%(U), -0.17%(V)

RA  : 0.02%(Y), -0.04%(U), -0.06%(V)

LDB: 0.xx%(Y), 0.xx%(U), 0.xx%(V)

LDP: 0.04%(Y), -0.12%(U), -0.40%(V)

# Introduction

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. Filters are signalled in adaptive loop filter (ALF) adaptation parameter sets (APS). In one APS, a Cb (Cr resp.) filter set with up to 4 filters could be signalled and a Cb (Cr resp.) filter from this Cb (Cr resp.) set could be applied to a Cb (Cr resp.) coding tree block (CTB). On the other hand, in an ALF APS, a chroma ALF filter set with up to 8 filters could be signalled. Both chroma components share the same filter set. A filter from a chroma filter set in an APS could be applied to a chroma (either Cb or Cb) coding tree block.

# Proposed method

To unify the filter storage between chroma ALF and CC-ALF. In this contribution, the CC-ALF filters for chroma components are unified, where Cb and Cr components share the same filters in an APS, which is the same method used in a regular chroma ALF. An ALF APS could signal CC-ALF filter set with up to 8 CC-ALF filters. In a slice header, one ALF APS index is signalled to indicate where the CC-ALF filter set is coming from. A chroma CTB can utilize any CC-ALF filter in this ALF APS.

The proposed method is implemented on top of CE5 anchor. The proposed method is tested under common test condition defined in JVET-N1010 against CE5 anchor.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.10% | 0.16% | 101% | 100% |
| Class A2 | 0.06% | -0.12% | -0.92% | 99% | 98% |
| Class B | 0.02% | -0.36% | 0.17% | 101% | 101% |
| Class C | -0.06% | 0.22% | 0.02% | 100% | 102% |
| Class E | -0.09% | 0.64% | -0.58% | 100% | 99% |
| Overall | -0.01% | 0.05% | -0.17% | 100% | 100% |
| Class D | -0.01% | -0.15% | -0.65% | 98% | 100% |
| Class F | -0.02% | 0.01% | 0.09% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.30% | 101% | 99% |
| Class A2 | 0.08% | -0.30% | -0.70% | 100% | 97% |
| Class B | 0.02% | -0.05% | 0.05% | 100% | 100% |
| Class C | -0.03% | 0.00% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.06% | 100% | 100% |
| Class D | -0.03% | -0.29% | -0.30% | 102% | 100% |
| Class F | -0.02% | 0.09% | -0.10% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | -0.02% | -0.38% | 100% | 101% |
| Class E | -0.08% | 0.23% | -0.92% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.49% | -1.07% | 100% | 100% |
| Class F | 0.07% | -0.09% | 0.79% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.03% | 0.07% | 100% | 99% |
| Class C | -0.01% | -0.19% | -0.26% | 100% | 100% |
| Class E | 0.14% | -0.25% | -1.38% | 100% | 101% |
| Overall | 0.04% | -0.12% | -0.40% | 100% | 100% |
| Class D | -0.02% | -1.43% | -1.85% | 99% | 100% |
| Class F | -0.07% | -0.27% | -0.08% | 101% | 101% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).