| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0256 |
| --- | --- |

| Title: | AHG9: Temporal sublayer level indication and conformance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains a proposal for clarification in VVC regarding support for decoding temporal subset in VVC decoders. The contribution also contains a proposal for a new NAL unit type used to indicate the highest temporal layer that is present in the CLVS. It is asserted that mandating presence of the proposed NAL unit in the first access unit of the CLVS makes it clear for a decoder to upfront determine if it is capable of decoding a bitstream or not.

# Introduction

In the profile definition of  Main 10 Profile in the current draft of VVC, the term “sublayer representation” is used in the definition of the capabilities of a VVC decoder:

Decoders conforming to the Main 10 profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 profile.

- The bitstream or sublayer representation is indicated to conform to a level that is not level 8.5 and is lower than or equal to the specified level.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

From the perspective of the decoder it is not clear what it is that differentiate a sublayer representation from a bitstream. A sublayer representation is defined as:

sublayer representation: A subset of the bitstream consisting of NAL units of a particular sublayer and the lower sublayers.

In particular, it would be highly desirable to provide a clear answer to the following questions:

- Shall a level 5.1 decoder be required to be able to extract and decode a level 5.1 sublayer representation from a level 5.2. bitstream?

- If a level 5.1 sublayer representation has been extracted from level 5.2 bitstream, shall a level 5.1 decoder be required to be able to decode it (even though general_level_idc indicates level 5.2)?

The authors of this proposals advocates that the answer to the first question should be “no” and that the answer to the second question should be “yes”. This contribution includes two proposals to clarify and improve the support for sublayer level indication and conformance.

## HEVC temporal subsets in DVB

In the DVB specification TS 101 154 [1] the following is expressed related to decoding of temporal sublayers:

When sps_max_sub_layers_minus1 is greater than "0", IRDs may ignore general_level_idc and shall make use of the sub_layer_level_idc[i] syntax element, where 'i' is equal to temporal_id_max carried within the HEVC Video Descriptor, to determine whether a bitstream or sub-bitstream can be decoded.

It can be noted TS 101 154 includes a decoder requirement that goes beyond the decoder requirements expressed in the HEVC specification since decoders are required to use sub_layer_level_idc[i] to determine if they can decode a bitstream. Furthermore, it can be noted that a system-specific variable; temporal_id_max from the HEVC Video Descriptor in the MPEG-2 transport stream, to determine decodability. Other systems might not be able to carry the same information. It is asserted that it would be desirable to include the corresponding indication within the compressed video and express the conformance definition so that decoders can make determination about decodability in alignment with the conformance definition.

# Proposal A

It is proposed to update the conformance definition as follows (with changes highlighted in yellow):

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Active SPSs for the base layer shall have chroma_format_idc equal to 0 or 1 only.

- Active SPSs for the base layer shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- Active SPSs for the base layer shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- The tier and level constraints specified for the Main 10 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 1.

Decoders conforming to the Main 10 profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 profile.

- The bitstream or sublayer representation is indicated through general_level_idc to conform to a level that is not level 8.5 and is lower than or equal to the specified level, or the bitstream is indicated through sublayer_level_idc[ SublayerRepTemporalId ] to conform to a level that is not level 8.5 and is lower than or equal to the specified level, where SublayerRepTemporalId is the highest TemporalId of all VCL Nal units in the bitstream.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

It is asserted that these changes clarify the decoder’s responsibility related to temporal subsets and that it makes it clear that for example a level 5.1 decoder shall decode a level 5.1 bitstream that is created by extracting a sublayer representation from a 5.2 bitstream even if general_level_idc indicates level 5.2.

# Proposal B

In order for a decoder to determine the actual highest temporal id present in CVS, it is proposed to introduce a new NAL unit type that is required to present in the first access unit of the CVS for all temporal layers that are present in the CVS.

The following semantics changes are proposed (highlighted in green):

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| … | … | … | … |
| 17 18 | PREFIX_APS_NUT SUFFIX_APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | PH_NUT | Picture header picture_header_rbsp( ) | non-VCL |
| 20 | AUD_NUT | AU delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 21 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 22 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 23 24 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 25 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 26 | TSP_NUT | Temporal sublayer presence temporal_sublayer_presence_rbsp( ) | non-VCL |
| 26 27 | RSV_NVCL_26 RSV_NVCL_27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC_28.. UNSPEC_31 | Unspecified non-VCL NAL unit types | non-VCL |

NAL units with nal_unit_type equal to TSP_NUT are required to be present before the first VCL NAL unit in the first Access Unit of a Coded Video Sequence for all temporal sub-layers that may contain one or more VCL NAL units in the CVS. When no TSP_NUT with TemporalId equal to tidA is present before the first VCL NAL unit in the first Access Unit of the CVS it is a requirement of bitstream conformance that no VCL NAL unit with TemporaldId equal to or greater than tidA is present in the CVS, for all tidA in the range of 0 to 6, inclusive.

The variable MaxTid is set equal to the highest value of TemporalId of all TSP_NUT NAL units present before the first VCL NAL unit in the first access unit of the CVS.

With this change it is possible to further modify the conformance definition as follows (changes relative to proposal A highlighted in green):

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Active SPSs for the base layer shall have chroma_format_idc equal to 0 or 1 only.

- Active SPSs for the base layer shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- Active SPSs for the base layer shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- The tier and level constraints specified for the Main 10 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 1.

Decoders conforming to the Main 10 profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 profile.

- The bitstream or sublayer representation is indicated through general_level_idc to conform to a level that is not level 8.5 and is lower than or equal to the specified level, or the bitstream is indicated through sublayer_level_idc[ MaxTid SublayerRepTemporalId ] to conform to a level that is not level 8.5 and is lower than or equal to the specified level, where SublayerRepTemporalId is the highest TemporalId all VCL Nal units in the bitstream.

# Example

An example of a problematic bitstream is shown in Figure 1. For the first 8 seconds of the coded video sequence (CVS) there are no NAL units with temporal ID 1 but the decoder cannot know if NAL units with temporal ID 1 will appear later in the CVS. It is not possible for a level 5.1 decoder to determine at the start of decoding the CVS if it will be able to decode the CVS or not.

Figure 1. Example of problematic bitstream

If TSP_NUT NAL units are added in each temporal layer that is present in the CVS, the decoder will be able to determine at the start of decoding the CVS if it will be able to decode the CVS or not.

Figure 2. Example of bitstream with TSP_NUT in each present temporal layer

# Summary

Clarifications to the conformance definition is proposed related to temporal sublayers. It is proposed that a decoder is not required to extract a temporal subbitstream but it is proposed that a decoder is required to decode a bitstream that has been extracted from a bitstream of higher level. A new NAL unit type is proposed to indicate which temporal layers are present in the bitstream.

# References

- ETSI TS 101 154 V2.6.1 (2019-09) Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications
https://www.etsi.org/deliver/etsi_ts/101100_101199/101154/02.06.01_60/ts_101154v020601p.pdf

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).