| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0260 |
| --- | --- |

| Title: | AHG9: Intended display resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains a proposal for syntax elements to be added to the Decoding Parameter Set (DPS). The syntax elements indicate the intended display resolution of the pictures encoded in the layer to which the DPS applies. It is asserted that it is of high importance for receivers and decoders to determine the resolution that the decoded pictures are intended to be displayed at. It is asserted that dynamic encoding techniques used for example in Adaptive Bit Rate (ABR) streaming applications and Reference Picture Resampling (RPR) makes it difficult for receivers and decoders to determine the intended display resolution.

# Introduction

In the current draft of VVC, the resolution of coded pictures is indicated in the PPS and it can change at any point in a bitstream, also within a CVS since the introduction of RPR. The resolution of the pictures can be reduced as a technique to improve the quality of the compressed video rather than increasing quantization which can lead to objectionable compression artifacts. This technique is used both in Adaptive Streaming Applications and video conferencing. Regardless of if the resolution changes occur at IDR pictures (e.g. at scene changes within a DASH segment) or within a CVS (e.g. in response to bandwidth fluctuations in a video conferencing call) it is assumed the reduced resolution will be upsampled by the receiver to the full resolution in order to provide a good user experience when the video is displayed. It is asserted that it is desirable for a receiver/decoder to know the intended display resolution (typically the original resolution of the video) even when the coded video starts with reduced resolution. It is asserted that knowing the intended display resolution is of high importance in order to properly set up memory allocations, rescaling and cropping configuration and display areas at the start of decoding a bitstream.

It can be argued that the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in the SPS may be used to determine intended display resolution. However, since no cropping information is present in the SPS, the actual information about output resolution cannot be determined from the SPS. Furthermore, since a new SPS can be signaled and activated at an IDR picture at any point in the bitstream, the output resolution cannot be determined even if a PPS with pic_width_in_luma_samples equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples equal to pic_height_max_in_luma_samples is activated in the first access unit of the bitstream.

# Example bitstreams

Figure 1 -3 shows examples of valid VVC bitstreams according to the latest VVC draft. Without any additional information about intended display resolution it is extremely difficult for a receiver/decoder to make correct assumptions of how the decoded pictures should be displayed.

Figure 1. Valid bitsteam A

Figure 2. Valid bitsteam B

Figure 3. Valid bitsteam C

# Proposal

It is proposed to add intended display width and intended display height as syntax elements in the DPS. Since the DPS contains information that applies to an entire bitstream, the receiver/decoder can use this information to make appropriate allocation and configuration for correct display of the decoded video.

Figure 4. Bitsteam A with addition of intended display resolution in DPS

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_max_sublayers_minus1 | u(3) |
| dps_reserved_zero_5bits | u(5) |
| dps_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dps_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, dps_max_sublayers_minus1 ) |  |
| intended_display_width_in_luma_samples | u(16) |
| intended_display_height_in_luma_samples | u(16) |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

intended_display_width_in_luma_samples specifies the intended display width, in units of luma samples, of each cropped decoded picture referring to the DPS.

intended_display_height_in_luma_samples specifies the intended display height, in units of luma samples, of each cropped decoded picture referring to the DPS.

NOTE – The intended display width and intended display height provide guidance for the width, height and aspect ratio to which the cropped decoded pictures are inteded to be mapped. When the intended display width and height is different from the coded display width and height, a resampling process may be applied to the cropped decoded picture but no specific resampling method is mandated or suggested. The intended display width and intended display height does not impose any constraints on the coded picture width and coded picture heights as expressed by pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

# Summary

This contribution contains a proposal for syntax elements to be added to the Decoding Parameter Set (DPS) indicating the intended display resolution. It is proposed to add intended_display_width_in_luma_samples and intended_display_height_in_luma_samples in the next draft of the VVC.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).