| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0261 |
| --- | --- |

| Title: | AHG9: On mixed NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution relates to support for mixed NAL unit types in VVC. It is proposed to modify the semantics of mixed_nalu_types_in_pic_flag so that CRA NAL units are handled differently than other NAL units and allow leading pictures.

# Proposal

In order to also provide full support for CRA NAL units in Access Units with mixed NAL unit types, the following is proposed:

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	If nalUnitTypeA is equal to CRA:

For all the following PUs in the CLVS following the current picture in decoding order and in output order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

–	Otherwise:

For all the following PUs in the CLVS following the current picture in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

# Summary

It is proposed to modify the semantics of the mixed_nalu_types_in_pic_flag so as to allow mixing of CRA NAL units with non-IRAP NAL units with maintained properties of the CRA NAL units.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).