| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0273 |
| --- | --- |

| Title: | AHG9: Editorial changes related to picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following editorial changes to the VVC draft specification:

- In the picture header there is a mix of syntax elements with ‘ph_’ and ‘pic_’ prefixes. It is proposed to replace ‘pic_’ with ‘ph_’ for all picture header syntax elements starting with ‘pic_’, except for pic_output_flag.

- The syntax element for SAO luma enabled flag in the picture header is called pic_sao_luma_enabled_flag in the syntax table and pic_sao_luma_flag in the semantics. It is proposed to rename it to ph_sao_luma_enabled_flag. Similarly, the SAO chroma flag is called pic_sao_chroma_enabled_flag in the syntax table and pic_sao_chroma_flag in the semantics. It is proposed to rename it ph_sao_chroma_enabled_flag.

- The reference picture lists syntax is identical in the picture header and the slice header apart from the prefixes ‘ph_’ and ‘slice_’ of the syntax elements. It is proposed to create a ref_pic_lists( ) syntax structure that may be signalled in either the picture header or the slice header. It is asserted by the proponents that the proposed change simplifies the syntax and specification text of reference picture lists and that it reduces the number of pages in the VVC draft specification by around 1.5 pages. The proponents note that the pic_rpl_present_flag syntax element in the current picture header controls whether the reference picture lists syntax is present in the slice header or the picture header and therefore claim that it is straightforward to use a common structure with a common name.

Proposed specification text relative to JVET-Q0041-v2 is provided with the contribution.

# Introduction

At the 16th JVET meeting in Geneva the picture header was adopted to the VVC draft specification. This contribution proposes some editorial changes to the specification text related to the picture header.

# Proposals

## Rename ‘pic_’ to ‘ph_’

In the picture header there is a mix of syntax elements with ‘ph_’ and ‘pic_’ prefixes. We think it is preferred to align the prefixes in the picture header unless there is a specific reason for having them different. The scope for all syntax elements starting with ‘pic_’ seems to be the picture header, except for the pic_output_flag which specifies whether the decoded picture should be output or not.

It is therefore proposed to replace ‘pic_’ with ‘ph_’ for all picture header syntax elements starting with ‘pic_’, except for pic_output_flag.

## Fix name for SAO enable flag

The syntax element for SAO luma enabled flag in the picture header is called pic_sao_luma_enabled_flag in the syntax table and pic_sao_luma_flag in the semantics. It is proposed to fix this by renaming both to ph_sao_luma_enabled_flag. The corresponding fix is also proposed for the SAO chroma enabled flag syntax element in the picture header. We therefore propose to rename pic_sao_chroma_enabled flag (syntax) and pic_sao_chroma_flag (semantics) to ph_sao_chroma_enabled_flag.

## Common ref_pic_lists( ) syntax structure for picture header and slice header

The syntax for the reference picture lists is identical in the picture header and the slice header apart from the ‘ph_’ and ‘slice_’ prefixes of the syntax elements. It is therefore proposed to create a common ref_pic_lists( ) syntax structure that may be signalled in either the picture header or the slice header.

We believe the proposed change improves readability and simplifies the syntax and the specification text of reference picture lists and that it reduces the number of pages in the VVC draft specification by around 1.5 pages.

The proponents note that the pic_rpl_present_flag syntax element in the current picture header controls whether the reference picture lists syntax is present in the slice header or the picture header and therefore claim that it is straightforward to use a common structure with a common name.

### Proposed syntax and semantics

Proposed changes on top of the VVC draft (JVET-Q0041-v2) are marked in yellow with removed text in red strike-through.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| ref_pic_lists( ) |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pic_rpl_present_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| ref_pic_lists( ) |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| if( slice_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| slice_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( slice_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| slice_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| ref_pic_lists( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| rpl_sps_flag[ i ] | u(1) |
| if( rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

### 7.4.3.6	Picture header RBSP semantics

…

pic_rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i for the picture associated with the PH is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. ref_pic_list_sps_flag[ i ] equal to 0 specifies that reference picture list i of the picture is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in the PH.

When pic_rpl_sps_flag[ i ] is not present, the following applies:

- If num_ref_pic_lists_in_sps[ i ] is equal to 0, the value of pic_rpl_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is greater than 0 and rpl1_idx_present_flag is equal to 0, the value of pic_rpl_sps_flag[ 1 ] is inferred to be equal to pic_rpl_sps_flag[ 0 ].

- Otherwise, the value of pic_rpl_sps_flag[ i ] is inferred to be equal to pps_ref_pic_list_sps_idc[ i ] − 1.

pic_rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element pic_rpl_idx[ i ] is represented by Ceil( Log2( num_ref_pic_lists_in_sps[ i ] ) ) bits. When not present, the value of pic_rpl_idx[ i ] is inferred to be equal to 0. The value of pic_rpl_idx[ i ] shall be in the range of 0 to num_ref_pic_lists_in_sps[ i ] − 1, inclusive. When pic_rpl_sps_flag[ i ] is equal to 1 and num_ref_pic_lists_in_sps[ i ] is equal to 1, the value of pic_rpl_idx[ i ] is inferred to be equal to 0. When pic_rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of pic_rpl_idx[ 1 ] is inferred to be equal to pic_rpl_idx[ 0 ].

The variable PicRplsIdx[ i ] is derived as follows:

PicRplsIdx[ i ] = pic_rpl_sps_flag[ i ] ? pic_rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]	(82)

pic_poc_lsb_lt[ i ][ j ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th LTRP entry in the i-th reference picture list for the picture associated with the PH. The length of the pic_poc_lsb_lt[ i ][ j ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

The variable PicPocLsbLt[ i ][ j ] is derived as follows:

PicPocLsbLt[ i ][ j ] = ltrp_in_slice_header_flag[ i ][ PicRplsIdx[ i ] ] ?	(83)
			pic_poc_lsb_lt[ i ][ j ] : rpls_poc_lsb_lt[ listIdx ][ PicRplsIdx[ i ] ][ j ]

pic_delta_poc_msb_present_flag[ i ][ j ] equal to 1 specifies that pic_delta_poc_msb_cycle_lt[ i ][ j ] is present. pic_delta_poc_msb_present_flag[ i ][ j ] equal to 0 specifies that pic_delta_poc_msb_cycle_lt[ i ][ j ] is not present.

Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id the same as the PH, has TemporalId equal to 0, and is not a RASL or RADL picture. Let setOfPrevPocVals be a set consisting of the following:

–	the PicOrderCntVal of prevTid0Pic,

–	the PicOrderCntVal of each picture that is referred to by entries in RefPicList[ 0 ] or RefPicList[ 1 ] of prevTid0Pic and has nuh_layer_id the same as the current picture,

–	the PicOrderCntVal of each picture that follows prevTid0Pic in decoding order, has nuh_layer_id the same as the current picture, and precedes the current picture in decoding order.

When there is more than one value in setOfPrevPocVals for which the value modulo MaxPicOrderCntLsb is equal to PicPocLsbLt[ i ][ j ], the value of pic_delta_poc_msb_present_flag[ i ][ j ] shall be equal to 1.

pic_delta_poc_msb_cycle_lt[ i ][ j ] specifies the value of the variable PicFullPocLt[ i ][ j ] as follows:

if( j  = =  0 )
	deltaPocMsbCycleLt[ i ][ j ] = pic_delta_poc_msb_cycle_lt[ i ][ j ]
else					(84)
	deltaPocMsbCycleLt[ i ][ j ] = pic_delta_poc_msb_cycle_lt[ i ][ j ] + deltaPocMsbCycleLt[ i ][ j − 1 ]
PicFullPocLt[ i ][ j ] = PicOrderCntVal – deltaPocMsbCycleLt[ i ][ j ] * MaxPicOrderCntLsb −
		( PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) ) + PicPocLsbLt[ i ][ j ]

The value of pic_delta_poc_msb_cycle_lt[ i ][ j ] shall be in the range of 0 to 2(32 − log2_max_pic_order_cnt_lsb_minus4 − 4 ), inclusive. When not present, the value of pic_delta_poc_msb_cycle_lt[ i ][ j ] is inferred to be equal to 0.

### 7.4.8	Slice header semantics

…

slice_rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i of the current slice is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. slice_rpl_sps_flag[ i ] equal to 0 specifies that reference picture list i of the current slice is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in the slice headers of the current picture.

When slice_rpl_sps_flag[ i ] is not present, the following applies:

- If pic_rpl_present_flag is equal to 1, the value of slice_rpl_sps_flag[ i ] is inferred to be equal to pic_rpl_sps_flag[ i ].

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is equal to 0, the value of ref_pic_list_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is greater than 0 and if rpl1_idx_present_flag is equal to 0, the value of slice_rpl_sps_flag[ 1 ] is inferred to be equal to slice_rpl_sps_flag[ 0 ].

slice_rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element slice_rpl_idx[ i ] is represented by Ceil( Log2( num_ref_pic_lists_in_sps[ i ] ) ) bits. When not present, the value of slice_rpl_idx[ i ] is inferred to be equal to 0. The value of slice_rpl_idx[ i ] shall be in the range of 0 to num_ref_pic_lists_in_sps[ i ] − 1, inclusive. When slice_rpl_sps_flag[ i ] is equal to 1 and num_ref_pic_lists_in_sps[ i ] is equal to 1, the value of slice_rpl_idx[ i ] is inferred to be equal to 0. When slice_rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of slice_rpl_idx[ 1 ] is inferred to be equal to slice_rpl_idx[ 0 ].

The variable RplsIdx[ i ] is derived as follows:

if( pic_rpl_present_flag )
	RplsIdx[ i ] = PicRplsIdx[ i ]		(141)
else
	RplsIdx[ i ] = slice_rpl_sps_flag[ i ] ? slice_rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]

slice_poc_lsb_lt[ i ][ j ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th LTRP entry in the i-th reference picture list. The length of the slice_poc_lsb_lt[ i ][ j ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

The variable PocLsbLt[ i ][ j ] is derived as follows: [Ed. (YK): Variable name subsetting issue: PocLsbLt is a subset of PicPocLsbLt.]

if( pic_rpl_present_flag )
	PocLsbLt[ i ][ j ] = PicPocLsbLt[ i ][ j ]		(142)
else
	PocLsbLt[ i ][ j ] = ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ?	(143)
			slice_poc_lsb_lt[ i ][ j ] : rpls_poc_lsb_lt[ listIdx ][ RplsIdx[ i ] ][ j ]

slice_delta_poc_msb_present_flag[ i ][ j ] equal to 1 specifies that slice_delta_poc_msb_cycle_lt[ i ][ j ] is present. slice_delta_poc_msb_present_flag[ i ][ j ] equal to 0 specifies that slice_delta_poc_msb_cycle_lt[ i ][ j ] is not present.

Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id the same as the current picture, has TemporalId equal to 0, and is not a RASL or RADL picture. Let setOfPrevPocVals be a set consisting of the following:

–	the PicOrderCntVal of prevTid0Pic,

–	the PicOrderCntVal of each picture that is referred to by entries in RefPicList[ 0 ] or RefPicList[ 1 ] of prevTid0Pic and has nuh_layer_id the same as the current picture,

–	the PicOrderCntVal of each picture that follows prevTid0Pic in decoding order, has nuh_layer_id the same as the current picture, and precedes the current picture in decoding order.

When pic_rpl_present_flag is equal to 0 and there is more than one value in setOfPrevPocVals for which the value modulo MaxPicOrderCntLsb is equal to PocLsbLt[ i ][ j ], the value of slice_delta_poc_msb_present_flag[ i ][ j ] shall be equal to 1.

slice_delta_poc_msb_cycle_lt[ i ][ j ] specifies the value of the variable FullPocLt[ i ][ j ] as follows:

if( pic_rpl_present_flag )
	FullPocLt[ i ][ j ] = PicFullPocLt[ i ][ j ]
else {
	if( j  = =  0 )
		DeltaPocMsbCycleLt[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ]
	else				(144)
		DeltaPocMsbCycleLt[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ] + DeltaPocMsbCycleLt[ i ][ j − 1 ]
	FullPocLt[ i ][ j ] = PicOrderCntVal − DeltaPocMsbCycleLt[ i ][ j ] * MaxPicOrderCntLsb  −
		( PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) ) + PocLsbLt[ i ][ j ]
}

The value of slice_delta_poc_msb_cycle_lt[ i ][ j ] shall be in the range of 0 to 2(32 − log2_max_pic_order_cnt_lsb_minus4 − 4 ), inclusive. When not present, the value of slice_delta_poc_msb_cycle_lt[ i ][ j ] is inferred to be equal to 0.

### 7.4.9	Reference picture lists syntax

The ref_pic_lists( ) syntax may be present in a picture header or slice header.

rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i in ref_pic_lists( ) is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. rpl_sps_flag[ i ] equal to 0 specifies that reference picture list i of the picture is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in ref_pic_lists( ).

When rpl_sps_flag[ i ] is not present, the following applies:

- If num_ref_pic_lists_in_sps[ i ] is equal to 0, the value of rpl_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise, if num_ref_pic_lists_in_sps[ i ] is greater than 0 and rpl1_idx_present_flag is equal to 0, the value of rpl_sps_flag[ 1 ] is inferred to be equal to rpl_sps_flag[ 0 ].

- Otherwise, the value of rpl_sps_flag[ i ] is inferred to be equal to pps_ref_pic_list_sps_idc[ i ] − 1.

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element rpl_idx[ i ] is represented by Ceil( Log2( num_ref_pic_lists_in_sps[ i ] ) ) bits. When not present, the value of rpl_idx[ i ] is inferred to be equal to 0. The value of rpl_idx[ i ] shall be in the range of 0 to num_ref_pic_lists_in_sps[ i ] − 1, inclusive. When rpl_sps_flag[ i ] is equal to 1 and num_ref_pic_lists_in_sps[ i ] is equal to 1, the value of rpl_idx[ i ] is inferred to be equal to 0. When rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ].

The variable RplsIdx[ i ] is derived as follows:

RplsIdx[ i ] = rpl_sps_flag[ i ] ? rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]	(82)

poc_lsb_lt[ i ][ j ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th LTRP entry in the i-th reference picture list in ref_pic_lists( ). The length of the poc_lsb_lt[ i ][ j ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

The variable PocLsbLt[ i ][ j ] is derived as follows:

PocLsbLt[ i ][ j ] = ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ?	(83)
			poc_lsb_lt[ i ][ j ] : rpls_poc_lsb_lt[ listIdx ][ RplsIdx[ i ] ][ j ]

delta_poc_msb_present_flag[ i ][ j ] equal to 1 specifies that delta_poc_msb_cycle_lt[ i ][ j ] is present. delta_poc_msb_present_flag[ i ][ j ] equal to 0 specifies that delta_poc_msb_cycle_lt[ i ][ j ] is not present.

Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id the same as the slice or picture header referring to ref_pic_lists( ), has TemporalId equal to 0, and is not a RASL or RADL picture. Let setOfPrevPocVals be a set consisting of the following:

–	the PicOrderCntVal of prevTid0Pic,

–	the PicOrderCntVal of each picture that is referred to by entries in RefPicList[ 0 ] or RefPicList[ 1 ] of prevTid0Pic and has nuh_layer_id the same as the current picture,

–	the PicOrderCntVal of each picture that follows prevTid0Pic in decoding order, has nuh_layer_id the same as the current picture, and precedes the current picture in decoding order.

When there is more than one value in setOfPrevPocVals for which the value modulo MaxPicOrderCntLsb is equal to PocLsbLt[ i ][ j ], the value of delta_poc_msb_present_flag[ i ][ j ] shall be equal to 1.

delta_poc_msb_cycle_lt[ i ][ j ] specifies the value of the variable FullPocLt[ i ][ j ] as follows:

if( j  = =  0 )
	deltaPocMsbCycleLt[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ]
else					(84)
	deltaPocMsbCycleLt[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ] + deltaPocMsbCycleLt[ i ][ j − 1 ]
FullPocLt[ i ][ j ] = PicOrderCntVal – deltaPocMsbCycleLt[ i ][ j ] * MaxPicOrderCntLsb −
		( PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) ) + PocLsbLt[ i ][ j ]

The value of delta_poc_msb_cycle_lt[ i ][ j ] shall be in the range of 0 to 2(32 − log2_max_pic_order_cnt_lsb_minus4 − 4 ), inclusive. When not present, the value of delta_poc_msb_cycle_lt[ i ][ j ] is inferred to be equal to 0.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).