| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0278 |
| --- | --- |

| Title: | AHG9/AHG8: On random access related flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

To fix a bug with the random access related flags (HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag), and to make the support of the layer-wise random access clear in VVC, the following specification text improvements are proposed:

- Proposal 1 (bug fix): The values of HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are set equal to 0, by default, for each non-RAP AU.

- Proposal 2 (functionality): The values of HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are manipulated for each (IRAP) PU (for each layer).

Introduction

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), a coded video sequence (CVS) starts with an IRAP AU, in which there is a PU for each layer in the CVS and the coded picture in each PU is an IRAP picture. For each IRAP AU, the values of HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are set, to deal with the associated RASL pictures, which are not avaible when decoding. For each GDR AU, the values of HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are set, similarly.

On the other hand, the VVC draft allows that a non-IRAP AU contains an IRAP PU, in which the coded picture is an IRAP picture. Fig. 1 illustrates the case. However, the values of HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are not set equal to specific values in a non-IRAP AU. Those flags are set equal to specific values only when the currrent AU is an IRAP AU, and do not have default values. So, the flags are referred to without the value setting, when an IRAP PU in a non-IRAP AU is decoded. This a bug in the curent specification.

Figure 1. An example of bitstream structure with IRAP AU and non-IRAP AU

In addition, to support a layer-wise random access, it is possible to perform random access at the position of an IRAP PU by discarding all PUs before the IRAP PU in the current VVC specficiation. To enable such a layer-wise random access by removing some prior PUs before the IRAP PU in a non-IRAP AU, the values of HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag need to be manipulated for each IRAP PU of each layer.

To fix a bug with the flags (HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag) in a non-IRAP AU, and to make the support of the layer-wise random access clear in VVC, the following specification text improvements are proposed:

- Proposal 1 (bug fix): The values of HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are set equal to 0, by default, for each non-RAP AU.

- Proposal 2 (functionality): The values of HandleCraAsCvsStartFlag, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are manipulated for each IRAP PU.

Specification text for Proposal 1

Option-1

- 8. 1 General decoding process - General … For each IRAP AU in the bitstream, the following applies: – If the AU is the first AU in the bitstream in decoding order, each picture is an IDR picture, or each picture is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the AU, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag. – Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. For each IRAP AU in the bitstream, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. For each GDR AU in the bitstream, the following applies: – If the AU is the first AU in the bitstream in decoding order or each picture is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleGdrAsCvsStartFlag to a value for the AU, HandleGdrAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleGdrAsCvsStartFlag. – Otherwise, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. NOTE – The above operations, for both IRAP pictures and GDR pictures, are needed for identification of the CVSs in the bitstream.

Option-2

- 8. 1 General decoding process - General … For each IRAP AU in the bitstream, the following applies: – If the AU is the first AU in the bitstream in decoding order, each picture is an IDR picture, or each picture is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the IRAP AU, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag. – Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. For each GDR AU in the bitstream, the following applies: – If the AU is the first AU in the bitstream in decoding order or each picture is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleGdrAsCvsStartFlag to a value for the AU, HandleGdrAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleGdrAsCvsStartFlag. – Otherwise, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. NOTE – The above operations, for both IRAP pictures and GDR pictures, are needed for identification of the CVSs in the bitstream.

- Specification text for Proposal 2

- 8. 1 General decoding process - General … For each IRAP AU in the bitstream, the following applies: – If the AU is the first AU in the bitstream in decoding order, each picture is an IDR picture, or each picture is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the AU, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag. – Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. For each IRAP PU (of an non-IRAP AU) in the bitstream, the following applies: – If the picture of the IRAP PU is an IDR picture or the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag for the IRAP PU is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the IRAP PU, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag for the IRAP PU. – Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0 for the IRAP PU. For each GDR AU in the bitstream, the following applies: – If the AU is the first AU in the bitstream in decoding order or each picture is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleGdrAsCvsStartFlag to a value for the AU, HandleGdrAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleGdrAsCvsStartFlag. – Otherwise, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0. For each GDR PU (of an non-IRAP AU) in the bitstream, the following applies: – If the picture of the GDR PU is the first picture of the layer that follows an EOS NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set the variable HandleGdrAsCvsStartFlag to a value for the GDR PU, HandleGdrAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleGdrAsCvsStartFlag for the GDR PU. – Otherwise, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0 for the GDR PU.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).