| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0279 |
| --- | --- |

| Title: | AHG8/AHG9: On alignment across layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

The following alignments across layers are proposed:

- If the current picture is RASL, a picture referencing the current picture in the same AU shall be equal to RASL.

- If the current picture is STSA, a picture referencing the current picture in the same AU shall be equal to STSA.

- The chroma_format_idc value of a layer is the same as the value of a (direct) reference layer.

- The subpics_present_flag and sps_subpic_id_present_flag values of a layer is the same as that of a (direct) reference layer.

Introduction

To address some issues with multiple layers in the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), the following alignments across layers are proposed.

Proposal 1

When a RASL picture in a layer A is referenced by a picture in a direct reference layer (layer B) of the layer A in the same AU, the picture referring to the RASL shall be a RASL picture. If not, the picture cannot be correctly decoded. For example, when each picture in the layer B references only the picture in the layer A in the same AU, without temporal reference, the picture of the layer B that references the RASL picture of the layer A shall be a RASL picture.

Fig. 1. An example of RASL pictures alignment across layers

Proposed text:

When a RASL NAL unit in a layer A is referenced by a VCL NAL unit in a direct reference layer of the layer A in the same AU, the nal_unit_type value of the VCL NAL unit referring to the RASL NAL unit shall be equal to RASL_NUT.

Proposal 2

When a STSA picture in a layer A is referenced by a picture in a direct reference layer of the layer A in the same AU, the picture referring to the STSA shall be a STSA picture. If not, the temporal sub-layer up-switching cannot be synchronized across layers. When a layer A is a reference layer of a layer B, if a temporal sub-layer is switched up with the target temporal ID only in the layer B, a coded picture in the layer A with the target temporal ID may not be present in the bitstream and cannot be referenced for decoding the picture in the layer B. In that case, the inter-layer motion compensation is broken.

Proposed text:

When a STSA NAL unit in a layer A is referenced by a VCL NAL unit in a direct reference layer of the layer A in the same AU, the nal_unit_type value of the VCL NAL unit referring to the STSA NAL unit shall be equal to STSA_NUT.

Proposal 3

The chroma_format_idc value of a SPS referred to by one or more coded pictures in a layer A shall be equal to the chroma_format_idc value in a SPS referred to by one or more coded pictures in a layer B, where the layer A is a direct reference layer of the layer B. This is because any coded picture shall have the same chroma_format_idc value with its reference picture.

Proposed text:

The chroma_format_idc value of a SPS referred to by one or more coded pictures in a layer A shall be equal to the chroma_format_idc value in a SPS referred to by one or more coded pictures in a direct reference layer of the layer A, in a CVS.

Proposal 4

The subpics_present_flag and sps_subpic_id_present_flag values of a SPS referred to by one or more coded pictures in a layer A shall be equal to the subpics_present_flag and sps_subpic_id_present_flag values in a SPS referred to by one or more coded pictures in a layer B, where the layer A is a direct reference layer of the layer B. This is because the sub-picture layout needs to be aligned or associated across layers. If not, the sub-picture with multiple layers may not be correctly extractible.

Proposed text:

The subpics_present_flag and sps_subpic_id_present_flag values of a SPS referred to by one or more coded pictures in a layer A shall be equal to the subpics_present_flag and sps_subpic_id_present_flag values in a SPS referred to by one or more coded pictures in a direct reference layer of the layer A, in a CVS.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

[1]	B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019