| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0282 |
| --- | --- |

| Title: | AHG9: On temporal sub-layer switching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

To guarantee the presence of the referenced PPS/APS NAL unit with TemporalId greater than 0 in the bitstream after temporal up-switching, the following constraint is proposed:

When a non-VCL NAL unit with nal_unit_type equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT and TemporalId equal to k (k is greater than 0) referred to by a VCL NAL unit with TemporalId equal to k is included in an AU, the non-VCL NAL unit shall be included in an AU with TemporalId equal to k between the nearest STSA prior to the VCL NAL unit in decoding order and the VCL NAL unit referring to the non-VCL NAL unit inclusive, or included in an AU with TemporalId less than k prior to the VCL NAL unit in decoding order.

Problem statement

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), a temporal sub-layer up-switching is supported with STSA pictures, and APS and PPS may have TemporalId greater than 0. Fig. 1 illustrate an example of temporal up-switching, with APS referencing. When a VCL NAL unit references an APS NAL unit prior to an STSA NAL unit in decoding order, the referenced APS NAL unit may not be present in the bitstream that is an output bitstream with a temporal up-switching at the STSA NAL unit.

Proposal

To guarantee the presence of the referenced PPS/APS NAL unit with TemporalId greater than 0 in the bitstream after temporal up-switching, the referenced PPS/APS NAL unit shall be included in an AU with the same TemporalId between the nearest STSA and the VCL NAL unit referring to the PPS/APS NAL unit inclusive, or included in an AU with TemporalId less than k prior to the VCL NAL unit in decoding order.

Figure 1. An example of temporal up-switching with APS referencing.

Proposed spec text

When a non-VCL NAL unit with nal_unit_type equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT and TemporalId equal to k (k is greater than 0) referred to by a VCL NAL unit with TemporalId equal to k is included in an AU, the non-VCL NAL unit shall be included in an AU with TemporalId equal to k between the nearest STSA prior to the VCL NAL unit in decoding order and the VCL NAL unit referring to the non-VCL NAL unit inclusive, or included in an AU with TemporalId less than k prior to the VCL NAL unit in decoding order.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).