| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0284 |
| --- | --- |

| Title: | AHG9: On mixed NAL unit type support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution proposes solutions to address several issues with the mixed NAL unit type:

Issue 1: The picture type is ambiguous when the picture is composed of mixed NAL unit types.

Proposal 1-1: It is proposed to individually define the picture type depending on the nal_unit_type, to maintain the full functionality.

Proposal 1-2:It is proposed that the picture with mixed_nalu_types_in_pic_flag equal to 1 is set equal to a trailing picture.

Issue 2: When NAL unit types are mixed in the picture, TemporalId contraints are coflict.

Proposal 2-1: It is proposed to disallow the mixture of STSA NAL units and IRAP NAL units in the same PU.

Proposal 2-2: It is proposed to remove the TemporalId constraint on STSA_NUT.

Issue 3: If mixed_nalu_types_in_pic_flag is signalled in PPS, at least two PPS NAL units shall be referred to by slice NAL units in a CLVS.

Proposal 3: It is proposed to move the syntax element mixed_nalu_types_in_pic_flag to PH.

Issue 4: The current design cannot support the coexistence of RADL/RASL NAL units with Trail NAL units in a PU.

Proposal 4: It is proposed to replace the flag mixed_nalu_types_in_pic_flag with an indicator mixed_nalu_types_in_pic_idc, to indicate more mixture types.

Issue 5: When a picture in a layer references another picture in a different layer in the same AU, the values of mixed_nalu_types_in_pic_flag shall be aligned across those layers.

Proposal 5: It is proposed to align the value of mixed_nalu_types_in_pic_flag between a dependent layer and its reference layer.

Problem statement

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), a mixed NAL unit type feature is supported, which enable to have one or more slice NAL units with NAL unit type equal to IRAP or CRA and one or more slice NAL units with NAL unit type equal to non-IRAP. The feature may be used for merging two different bitstreams into one or for supporting different random access periods for each local region (sub-picture). Currently, the following syntax and semantics are defined to support the functionality:

In JVET-P2001:

- Picture parameter set RBSP syntax pic_parameter_set_rbsp( ) {; Descriptor. …. mixed_nalu_types_in_pic_flag; u(1). …. }. mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type. When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0. For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies: – The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1. – The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA. – For all the following PUs in the CLVS in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry. For VCL NAL units of any particular picture, the following applies: – If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU. – Otherwise (mixed_nalu_types_in_pic_flag is equal to 1), one or more of the VCL NAL units shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GDR_NUT.

The current design of the mixed NAL unit types may have the following problems:

Issue 1: The picture type is ambiguous when the picture is composed of mixed NAL unit types. It should not be an IRAP picture, but the non-IRAP picture type is not clearly defined.

Issue 2: When NAL unit types are mixed in the picture, TemporalId contraints are coflict. For example, an IDR NAL unit can not coexist with an STSA NAL unit in the same PU, due to the following contraints:

- When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_12, inclusive, TemporalId shall be equal to 0.

- When nal_unit_type is equal to STSA_NUT, TemporalId shall not be equal to 0.

Issue 3: If mixed_nalu_types_in_pic_flag is signalled in PPS, at least two PPS NAL units shall be referred to by slice NAL units in a CLVS. If the number of PPS increases, the number of APS may increase in proportial.

Issue 4: The current design cannot support the coexistence of RADL/RASL NAL units with Trail NAL units in a PU. This case may happen when an IRAP NAL unit associated with a RADL/RASL NAL unit coexist with a Trail NAL unit in the same PU.

Issue 5: When a picture in a layer references another picture in a different layer in the same AU, the values of mixed_nalu_types_in_pic_flag shall be aligned across those layers.

Issue 1

The picture type is ambiguous when the picture is composed of mixed NAL unit types. It should not be an IRAP picture, but the non-IRAP picture type is not clearly defined.

Proposal 1-1 to address Issue-1

It is proposed to individually define the picture type depending on the nal_unit_type, to keep the full functionality.

Proposed text:

When mixed_nalu_types_in_pic_flag of a PPS referred to by a coded picture is equal to 1, the picture type of the coded picture is determined as follows: - When a nal_unit_type is equal to TRAIL_NUT and the other nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, the picture is a trailing picture. - When a nal_unit_type is equal to RADL_NUT and the other nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, the picture is a RADL picture. - When a nal_unit_type is equal to STSA_NUT and the other nal_unit_type is IDR_W_RADL or IDR_N_LP, the picture is a STSA picture. - When a nal_unit_type is equal to STSA _NUT and the other nal_unit_type is CRA_NUT, the picture is a trailing picture. - When a nal_unit_type is equal to GDR_NUT and the other nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, the picture is a trailing picture.

Proposal 1-2 to address Issue-1

It is proposed that the picture with mixed_nalu_types_in_pic_flag equal to 1 is set equal to a trailing picture.

Proposed text:

When mixed_nalu_types_in_pic_flag of a PPS referred to by a coded picture is equal to 1, the coded picture is a trailing picture

Issue 2

When NAL unit types are mixed in the picture, TemporalId contraints are coflict. For example, an IDR NAL unit can not coexist with an STSA NAL unit in the same PU, due to the following contraints:

- When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_12, inclusive, TemporalId shall be equal to 0.

- When nal_unit_type is equal to STSA_NUT, TemporalId shall not be equal to 0.

Note that all VCL NAL units in an AU shall have the same TemporalId value.

Proposal 2-1 to address Issue-2

It is proposed to disallow the mixture of STSA NAL units and IRAP NAL units in the same PU.

Proposed text:

For VCL NAL units of any particular picture, the following applies: – If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU. – Otherwise (mixed_nalu_types_in_pic_flag is equal to 1), one or more of the VCL NAL units shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of RADL_NUT to RSV_VCL_6, inclusive, or equal to GDR_NUT or TRAIL_NUT.

Proposal 2-2 to address Issue-2

It is proposed to remove the TemporalId constraint on STSA_NUT.

Proposed text:

When nal_unit_type is equal to STSA_NUT, TemporalId shall not be equal to 0.

Issue 3

If mixed_nalu_types_in_pic_flag is signalled in PPS, at least two PPS NAL units shall be referred to by slice NAL units in a CLVS. If the number of PPS increases, the number of APS may increase in proportial.

Proposal 3-1 to address Issue-3

It is proposed to move the syntax element mixed_nalu_types_in_pic_flag to PH.

Proposed text:

picture_header_rbsp( ) {; Descriptor. …. mixed_nalu_types_in_pic_flag; u(1). …. }. mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture associated with the PH has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture associated with the PH has one or more VCL NAL units and the VCL NAL units of each picture associated with the PH have the same value of nal_unit_type. When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

Proposal 3-1 to address Issue-3

It is proposed to move the syntax element mixed_nalu_types_in_pic_flag to PH and add a flag in SPS to indicate the presence of the mixed NAL unit type in the CLVS.

Proposed text:

seq_parameter_set_rbsp( ) {; Descriptor. …. sps_mixed_nalu_types_present_flag; u(1). …. }. picture_header_rbsp( ) {; Descriptor. …. if(sps_mixed_nalu_types_present_flag). mixed_nalu_types_in_pic_flag; u(1). …. }. sps_mixed_nalu_types_present_flag equal to 1 specifies that zero or more picture referring to the SPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. sps_mixed_nalu_types_present_flag equal to 0 specifies that each picture referring to the SPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type. When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of sps_mixed_nalu_types_present_flag shall be equal to 0. mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture associated with the PH has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture associated with the PH has one or more VCL NAL units and the VCL NAL units of each picture associated with the PH have the same value of nal_unit_type. When not present, the value of mixed_nalu_types_in_pic_flag is inferred to be equal to 0.

Issue 4

The current design cannot support the coexistence of RADL/RASL NAL units with Trail NAL units in a PU. This case may happen when an IRAP NAL unit associated with a RADL/RASL NAL unit coexist with a Trail NAL unit in the same PU.

Proposal 4 to address Issue-4

It is proposed to replace the flag mixed_nalu_types_in_pic_flag with an indicator mixed_nalu_types_in_pic_idc, to indicate more mixture types.

Proposed text:

pic_parameter_set_rbsp( ) {; Descriptor. …. mixed_nalu_types_in_pic_flagidc; u(2). …. }. mixed_nalu_types_in_pic_flagidc equal to 1 or 2 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_idc equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type. Other values for mixed_nalu_types_in_pic_idc are reserved for future use by ITU-T ¦ ISO/IEC. When no_mixed_nalu_types_in_pic_constraint_idc is equal to 1, the value of mixed_nalu_types_in_pic_idc shall be equal to 0. For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_idc for the picture picA is equal to 1), the following applies: – The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1. – The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA. – For all the following PUs in the CLVS in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry. For VCL NAL units of any particular picture, the following applies: – If mixed_nalu_types_in_pic_idc is equal to 1, one or more of the VCL NAL units shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GDR_NUT. – If mixed_nalu_types_in_pic_idc is equal to 2, one or more of the VCL NAL units shall all have a particular value of nal_unit_type equal to RASL_NUT or RADL_NUT, inclusive, or equal to GDR_NUT, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GDR_NUT, where the nal_unit_type is different than the other nal_unit_type. – Otherwise (mixed_nalu_types_in_pic_idc is equal to 0), the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU. When mixed_nalu_types_in_pic_idc of a PPS referred to by a coded picture is equal to 1or 2, the picture is a trailing picture.

Issue 5

When a picture in a layer references another picture in a different layer in the same AU, the values of mixed_nalu_types_in_pic_flag shall be aligned across those layers. If not, the mixed NAL unit type feature may not work with multiple layers with dependencies.

Proposed text:

When mixed_nalu_types_in_pic_flag of a picture in a layer A is equal to 1, mixed_nalu_types_in_pic_flag of a picture in a layer B, which is a reference layer of the layer A, shall be equal to 1 in the same AU.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).