| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0291-v2 |
| --- | --- |

| Title: | CE2-related: On maximum palette size of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Yan Ye, Jiancong Luo, Ru-Ling Liao 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In VVC7, the maximum predictor size for both joint and separate palettes are set to 63 and the maximum palette size for both joint and separate palettes are set to 31. However, the complexity of the palette generation and predictor update of separate tree is roughly double that of single palette since the separate tree need to process two palette tables for luma and chroma. This contribution proposes to reduce the maximum predictor and palette size of separate palette to the half of that of joint palette.

Following results are reported as compared to VTM-7.0.

- YUV444 (dual tree ON):

          - Overall: -0.03% (AI), 0.05% (RA), 0.01% (LB) BD-rate saving

# Introduction

In VVC draft 7, the palette mode is used in 4:4:4 color format. Based on the tree type (either single or dual tree) of a slice, two types of palette tables are maintained.

- Joint palette: For slices of single tree, each entry of the palette table contains Y, Cb, Cr color components and all color components of a coding unit (CU) are coded jointly (with the exception of local dual tree).

- Separate palette: For slices with dual luma/chroma tree, two palette tables are maintained. Luma (Y) and chroma (CbCr) components maintain different palette tables and are coded separately. Two index maps (one for luma and one for chroma) are signaled.

For coding of the palette, a palette predictor is maintained. For each entry in the palette predictor, a reuse flag is signaled to indicate whether it will be included in the current palette of the current CU. After decoding each palette coded CU, the palette predictor is updated. Following steps are performed to update the predictor:

- Step 1: Copy all of the entries of the current palette tables at the beginning of the predictor

- Step 2: Sequentially add un-used entries (i.e. entries with reuse_flag = 0) of the previous predictor until total number of entries reach to the maximum predictor size. In worst case scenario, the number of times encoder/decoder need to check reuse flags is equal to the maximum predictor size. Since this process is sequential, the larger the predictor size is,  the more flag checks the encoder/decoder needs to perform during palette predictor update process. This situation becomes worse in separate tree case since  two palette tables, one for luma and the other for chroma, need to be updated.

# Proposed method

In order to reduce the complexity of palette mode in separate tree cases, this contribution proposes to set both the maximum predictor size and maximum palette size of separate tree to the half of that of the single tree. The following six variables are defined to represent maximum predictor and palette size.

- maxPltPredictorSizeJoint: maximum predictor size of joint palette

- maxPltPredictorSizeLuma: maximum predictor size of the luma component if separate palette is used.

- maxPltPredictorSizeChroma: maximum predictor size of the chroma components if separate palette is used.

- maxPltSizeJoint: maximum palette size of joint palette

- maxPltSizeLuma: maximum palette size of the luma component if separate palette is used.

- maxPltSizeChroma: maximum palette size of the chroma components if separate palette is used.

In the proposed method,

maxPltPredictorSizeLuma = maxPltPredictorSizeChroma = (maxPltPredictorSizeJoint >> 1)

maxPltSizeLuma = maxPltSizeChroma = (maxPltPredictorSizeJoint >> 1)

To set the values of maxPltPredictorSizeJoint and maxPltSizeJoint, following two methods are proposed.

## Method 1

In method 1, same as VVC7, the maximum predictor and palette size are set to a set of predefined values as follows:

maxPltPredictorSizeJoint = 63

maxPltSizeJoint = 31

maxPltPredictorSizeLuma = maxPltPredictorSizeChroma = 31

maxPltSizeLuma = maxPltSizeChroma = 15

## Method 2

In method 2, the maximum palette size of joint palette and the difference between the maximum palette size and maximum predictor size of joint palette is signaled to the decoder through SPS. Following are the semantics:

sps_max_plt_size_joint specifies the maximum allowed palette size of joint palette table. The value of sps_max_plt_size_joint shall be in the range of 0 to 63, inclusive.

sps_delta_max_plt_predictor_size_joint specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size of joint palette. The value of sps_delta_max_plt_predictor_size_joint shall be in the range of 0 to 63, inclusive.

The maximum palette size and predictor size of separate luma/chroma palette are not signaled, instead those are derived from the maximum predictor and palette value of the joint palette. The following is the derivation of palette sizes and palette predictor sizes from the above mentioned syntax.

maxPltSizeJoint = sps_max_plt_size_joint

maxPltSizeLuma = maxPltSizeJoint >> 1

maxPltSizeChroma = maxPltSizeJoint >> 1

maxPltPredictorSizeJoint = maxPltSizeJoint  + sps_delta_max_plt_predictor_size_joint

maxPltPredictorSizeLuma = maxPltPredictorSizeJoint >> 1

maxPltPredictorSizeChroma = maxPltPredictorSizeJoint  >> 1

Table 1: SPS signaling of maximum predictor and palette size

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| if( chroma_format_idc = = 3 ) { |  |
| sps_palette_enabled_flag | u(1) |
| sps_act_enabled_flag | u(1) |
| } |  |
| if(sps_palette_enabled_flag){ |  |
| sps_max_plt_size_joint | u(6) |
| sps_delta_max_plt_predictor_size_joint | u(6) |
| } |  |

Table 1 shows the SPS syntax of the signaling of the maximum predictor and palette size.

# Simulation results

The method 1 of this contribution is implemented on top of VTM-7.0. Each test is evaluated under common test conditions described in CE2 [1]. Table 2 shows the results of the proposed method in case of YUV444 sequences with dual tree ON. Since there is no change in single tree, the results of YUV444 with dual tree OFF should be essentially the same as VTM-7.0, except the slight difference in HLS signaling cost.

Table 2: results of the proposed method ( YUV444, dual tree ON)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.15% | 0.26% | 0.22% | 99% | 100% |
| TGM 720p | 0.05% | -0.03% | -0.03% | 100% | 99% |
| Animation | -0.02% | 0.02% | 0.01% | 100% | 100% |
| Mixed content | -0.03% | 0.12% | 0.12% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | -0.03% | 0.08% | 0.07% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.05% | 0.23% | 0.23% | 99% | 100% |
| TGM 720p | 0.01% | -0.06% | -0.05% | 100% | 99% |
| Animation | -0.01% | 0.01% | -0.01% | 99% | 99% |
| Mixed content | 0.18% | 0.25% | 0.32% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.05% | 0.09% | 0.10% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.12% | 0.00% | 0.04% | 99% | 95% |
| TGM 720p | 0.13% | 0.01% | 0.08% | 99% | 100% |
| Animation | 0.03% | -0.03% | 0.01% | 100% | 100% |
| Mixed content | -0.01% | -0.12% | 0.14% | 100% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.01% | -0.02% | 0.06% | 100% | 99% |

# Conclusion

In order to reduce the complexity of palette mode of separate tree, this contribution proposes to reduce the predictor size and palette size of separate tree to the half of the joint tree.

# References

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “ Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).