| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0292-v4 |
| --- | --- |

| Title: | Cleanup of intra reference sample filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Junghak Nam Hyeongmun Jang Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, jangwon84.choi, junghak.nam, hm.jang, moonmo.koo jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC draft, the intra reference sample filter decision process relies on the size-dependent threshold to select intra reference sample filters, and the intra reference sample filtering process requires multiple checks because intra mode information should be considered to select intra reference sample filters. This contribution introduces two methods to simplify this intra reference sample filter decision process. In Method 1, the selection of filters for both the reference sample filter and the interpolation filter is directly derived according to the size-dependent threshold. In Method 2, the reference sample filter process is completely removed in the intra coding process and the interpolation filter is selected based on the size-dependent threshold. Method 1 provides the coding performance changes of 0.00%, 0.00%, and -0.04% BD-rate in AI, RA, and LD configuration, respectively. Method 2 provides the coding performance changes of 0.01%, 0.02%, and 0.03% BD-rate in AI, RA, and LD configuration, respectively.

Introduction

Since the intra reference sample filter decision process uses the size-dependent threshold to select intra reference sample filters, intra reference sample filters should be selected in difference process according to the size-dependent threshold. Moreover, the process of determining intra reference sample filters has still multiple checks, as intra mode information should be considered to select intra reference sample filters. This contribution proposes to simplify the intra reference sample filter selection.

Proposed method

This contribution proposes to determine intra reference sample filter selection without the intra mode checking process. Table 1 and Table 2 describe the proposed intra reference sample filter selection process for Method 1 and Method 2, respectively.

In Table 1 and Table 2, the refFilterFlag and the interpolationFlag represent the reference sample filter flag and the interpolation filter flag, respectively. If the refFilterFlag is set equal to true or false, the intra reference sample smoothing filter is enabled or disabled. The intraMode, HorMode, and VerMode represent the current intra mode, horizontal mode, and vertical mode. If the current intra mode is one of {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80} (the intra prediction mode refers to integer reconstructed boundary samples), isIntegerSlop is true. Otherwise, isIntegerSlop returns false. If the interpolationFlag is set equal to true or false, smoothing filter or DCT-IF filter is applied, respectively.

The only difference between Method 1 and Method 2 is the reference sample filter selection process for Planar.

Table 1. Proposed intra reference sample filter selection process for Method 1

Table 2. Proposed intra reference sample filter selection process for Method 2

In Method 1, intra reference sample filters are selected as the same way as VTM-7.0 except for the angular mode. Intra reference sample filters for the angular mode are selected as follows.

- If diff is greater than size-dependent threshold (MDIS condition is true) - refFilterFlag is set to false. - interpolationFlag is set to true (smoothing filter). - Otherwise, - refFilterFlag is set to false. - interpolationFlag is set to false (DCT-IF).

Since intra reference sample filters are directly determined according to the size-dependent threshold, intra mode information is unnecessary for Method 1.

Method 2 proposes to remove the reference sample filter flag decision process. This means in all intra prediction the reference sample filter process is completely removed in intra coding process.

Experimental result

The proposed method has been implemented using VTM-7.0 and under the common test conditions [1].

- Method 1: intra reference sample filters are directly derived according to the MDIS condition.

Table 3. Coding performance of the proposed method (AI)

Table 4. Coding performance of the proposed method (RA)

Table 5. Coding performance of the proposed method (LD)

- Method 2: reference sample filtering process for planar mode is disabled based on Method 1.

Table 6. Coding performance of the proposed method (AI)

Table 7. Coding performance of the proposed method (RA)

Table 8. Coding performance of the proposed method (LD)

Working Draft Text

The changes in the VVC working draft [2] is highlight below.

- Method 1: intra reference sample filters are directly derived according to the MDIS condition.

        - General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables refW and refH are derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

refW = nTbW * 2		(301)

refH = nTbH * 2		(302)

- Otherwise ( IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 ), the following applies:

refW = nCbW + nTbW		(303)

refH = nCbH + nTbH		(304)

The variable refIdx specifying the intra prediction reference line index is derived as follows:

refIdx = ( cIdx  = =  0 )  ?  IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ]  :  0	(305)

The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

The variable refFilterFlag is derived as follows:

- If predModeIntra is equal to 0 one of the following values: 0, −14, −12, −10, −6, 2, 34, 66, 72, 76, 78, 80, then refFilterFlag is set equal to 1.

- Otherwise, refFilterFlag is set equal to 0.

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.4.5.2.7 is invoked with the sample location ( xTbCmp, yTbCmp ), the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = − refIdx..refW − 1, y = −1 − refIdx as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.5.2.8 is invoked with the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

- The reference sample filtering process as specified in clause 8.4.5.2.9 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the reference filter flag refFilterFlag, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.4.5.2.10 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.4.5.2.11 is invoked with the transform block width nTbW, the transform block height nTbH, the intra prediction reference line index refIdx, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_LT_CCLM, INTRA_L_CCLM or INTRA_T_CCLM, the corresponding intra prediction mode process specified in clause 8.4.5.2.13 is invoked with the intra prediction mode predModeIntra, the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, the colour component index cIdx, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.4.5.2.12 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the coding block width nCbW and height nCbH, the reference filter flag refFilterFlag, the colour component index cIdx, and the reference sample array p as inputs, and the predicted sample array predSamples as outputs.

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, and the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as inputs, and the output is the modified predicted sample array predSamples:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

        - Reference sample filtering process

Inputs to this process are:

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable refFilterFlag specifying the value of reference filter flag,

- the (unfiltered) neighbouring samples refUnfilt[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the reference samples p[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- refFilterFlag is equal to 1

- Otherwise, filterFlag is set equal to 0.

For the derivation of the reference samples p[ x ][ y ] the following applies:

- If filterFlag is equal to 1, the filtered sample values p[ x ][ y ] with x = −1, y = −1.. nTbH refH − 1 and x = 0.. nTbW refW − 1, y = −1 are derived as follows:

p[ −1 ][ −1 ] = ( refUnfilt[ −1 ][ 0 ] + 2 * refUnfilt[ −1 ][ −1 ] + refUnfilt[ 0 ][ −1 ] + 2 )  >>  2	(313)

p[ −1 ][ y ] = ( refUnfilt[ −1 ][ y + 1 ] + 2 * refUnfilt[ −1 ][ y ] + refUnfilt[ −1 ][ y − 1 ] + 2 )  >>  2
	for y = 0..refH − 2		(314)

p[ −1 ][ refH − 1 ] = refUnfilt[ −1 ][ refH − 1 ]		(315)

p[ x ][ −1 ] = ( refUnfilt[ x − 1 ][ −1 ] + 2 * refUnfilt[ x ][ −1 ] + refUnfilt[ x + 1 ][ −1 ] + 2 )  >>  2
	for x = 0..refW − 2 		(316)

p[ refW − 1 ][ −1 ] = refUnfilt[ refW − 1 ][ −1 ]		(317)

- Otherwise, the reference samples values p[ x ][ y ] are set equal to the unfiltered sample values refUnfilt[ x ][ y ] with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

        - Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable refFilterFlag specifying the value of reference filter flag,

- a variable cIdx specifying the colour component of the current block,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0:

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 25.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 25 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 24 | 14 | 2 | 0 | 0 | 0 |

Figure 9 – Intra prediction directions (informative)

Figure 9 illustrates the 93 prediction directions, where the dashed directions are associated with the wide-angle modes that are only applied to non-square blocks.

Table 26 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Table 26 – Specification of intraPredAngle

| predModeIntra | −14 | −13 | −12 | −11 | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | 512 | 341 | 256 | 171 | 128 | 102 | 86 | 73 | 64 | 57 | 51 | 45 | 39 | 35 | 32 | 29 | 26 |
| predModeIntra | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| intraPredAngle | 23 | 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 | 3 | 2 | 1 | 0 | −1 | −2 | −3 |
| predModeIntra | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| intraPredAngle | −4 | −6 | −8 | −10 | −12 | −14 | −16 | −18 | −20 | −23 | −26 | −29 | −32 | −29 | −26 | −23 | −20 |
| predModeIntra | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| intraPredAngle | −18 | −16 | −14 | −12 | −10 | −8 | −6 | −4 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 4 | 6 |
| predModeIntra | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 |
| intraPredAngle | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 |
| predModeIntra | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |  |  |  |  |  |  |  |  |  |
| intraPredAngle | 73 | 86 | 102 | 128 | 171 | 256 | 341 | 512 |  |  |  |  |  |  |  |  |  |

The inverse angle parameter invAngle is derived based on intraPredAngle as follows:

invAngle = Round512*32intraPredAngle		(325)

The interpolation filter coefficients fC[ phase ][ j ] and fG[ phase ][ j ] with phase = 0..31 and j = 0..3 are specified in Table 27.

Table 27 – Specification of interpolation filter coefficients fC and fG

| Fractional sample position p | fC interpolation filter coefficients |  |  |  | fG interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | −1 | 63 | 2 | 0 | 16 | 32 | 16 | 0 |
| 2 | −2 | 62 | 4 | 0 | 15 | 31 | 17 | 1 |
| 3 | −2 | 60 | 7 | −1 | 15 | 31 | 17 | 1 |
| 4 | −2 | 58 | 10 | −2 | 14 | 30 | 18 | 2 |
| 5 | −3 | 57 | 12 | −2 | 14 | 30 | 18 | 2 |
| 6 | −4 | 56 | 14 | −2 | 13 | 29 | 19 | 3 |
| 7 | −4 | 55 | 15 | −2 | 13 | 29 | 19 | 3 |
| 8 | −4 | 54 | 16 | −2 | 12 | 28 | 20 | 4 |
| 9 | −5 | 53 | 18 | −2 | 12 | 28 | 20 | 4 |
| 10 | −6 | 52 | 20 | −2 | 11 | 27 | 21 | 5 |
| 11 | −6 | 49 | 24 | −3 | 11 | 27 | 21 | 5 |
| 12 | −6 | 46 | 28 | −4 | 10 | 26 | 22 | 6 |
| 13 | −5 | 44 | 29 | −4 | 10 | 26 | 22 | 6 |
| 14 | −4 | 42 | 30 | −4 | 9 | 25 | 23 | 7 |
| 15 | −4 | 39 | 33 | −4 | 9 | 25 | 23 | 7 |
| 16 | −4 | 36 | 36 | −4 | 8 | 24 | 24 | 8 |
| 17 | −4 | 33 | 39 | −4 | 8 | 24 | 24 | 8 |
| 18 | −4 | 30 | 42 | −4 | 7 | 23 | 25 | 9 |
| 19 | −4 | 29 | 44 | −5 | 7 | 23 | 25 | 9 |
| 20 | −4 | 28 | 46 | −6 | 6 | 22 | 26 | 10 |
| 21 | −3 | 24 | 49 | −6 | 6 | 22 | 26 | 10 |
| 22 | −2 | 20 | 52 | −6 | 5 | 21 | 27 | 11 |
| 23 | −2 | 18 | 53 | −5 | 5 | 21 | 27 | 11 |
| 24 | −2 | 16 | 54 | −4 | 4 | 20 | 28 | 12 |
| 25 | −2 | 15 | 55 | −4 | 4 | 20 | 28 | 12 |
| 26 | −2 | 14 | 56 | −4 | 3 | 19 | 29 | 13 |
| 27 | −2 | 12 | 57 | −3 | 3 | 19 | 29 | 13 |
| 28 | −2 | 10 | 58 | −2 | 2 | 18 | 30 | 14 |
| 29 | −1 | 7 | 60 | −2 | 2 | 18 | 30 | 14 |
| 30 | 0 | 4 | 62 | −2 | 1 | 17 | 31 | 15 |
| 31 | 0 | 2 | 63 | −1 | 1 | 17 | 31 | 15 |

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = 0..nTbW + refIdx + 1	(326)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbH ) ],
	with x = −nTbH..−1	(327)

- Otherwise,

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = nTbW + 2 + refIdx..refW + refIdx	(328)

- The additional samples ref[ refW + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1 ) are derived as follows:

ref[ refW + refIdx + x ] = p[ −1 + refW ][ −1 − refIdx ]	(329)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( y + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(330)

iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31	(331)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(332)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(333)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(334)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(335)

- Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = 0..nTbH + refIdx + 1	(336)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

ref[ x ] = p[ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbW ) ][ −1 − refIdx ],
	with x = −nTbW..−1	(337)

- Otherwise,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = nTbH + 2 + refIdx..refH + refIdx	(338)

- The additional samples ref[ refH + refIdx +x ] with x = 1..( Max( 1, nTbH / nTbW ) * refIdx + 1) are derived as follows:

ref[ refH + refIdx +x ] = p[ −1 + refH ][ −1 − refIdx ]	(339)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( x + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(340)

iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31	(341)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(342)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 )  >>  6 )	(343)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(344)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(345)

- Method 2: reference sample filtering process for planar mode is disabled based on Method 1.

        - General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables refW and refH are derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

refW = nTbW * 2		(301)

refH = nTbH * 2		(302)

- Otherwise ( IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 ), the following applies:

refW = nCbW + nTbW		(303)

refH = nCbH + nTbH		(304)

The variable refIdx specifying the intra prediction reference line index is derived as follows:

refIdx = ( cIdx  = =  0 )  ?  IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ]  :  0	(305)

The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

The variable refFilterFlag is derived as follows:

- If predModeIntra is equal to one of the following values: 0, −14, −12, −10, −6, 2, 34, 66, 72, 76, 78, 80, then refFilterFlag is set equal to 1.

- Otherwise, refFilterFlag is set equal to 0.

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.4.5.2.7 is invoked with the sample location ( xTbCmp, yTbCmp ), the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = − refIdx..refW − 1, y = −1 − refIdx as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.5.2.8 is invoked with the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the modified reference samples p[ x ][ y ] refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

- The reference sample filtering process as specified in clause 8.4.5.2.9 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the reference filter flag refFilterFlag, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.4.5.2.10 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.4.5.2.11 is invoked with the transform block width nTbW, the transform block height nTbH, the intra prediction reference line index refIdx, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_LT_CCLM, INTRA_L_CCLM or INTRA_T_CCLM, the corresponding intra prediction mode process specified in clause 8.4.5.2.13 is invoked with the intra prediction mode predModeIntra, the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, the colour component index cIdx, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.4.5.2.12 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the coding block width nCbW and height nCbH, the reference filter flag refFilterFlag, the colour component index cIdx, and the reference sample array p as inputs, and the predicted sample array predSamples as outputs.

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, and the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as inputs, and the output is the modified predicted sample array predSamples:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

        - Reference sample substitution process

Inputs to this process are:

- a variable refIdx specifying the intra prediction reference line index,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx for intra sample prediction,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the modified reference samples p[ x ][ y ] refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx for intra sample prediction.

The values of the samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx are modified as follows:

- If all samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx are marked as "not available for intra prediction", all values of refUnfilt[ x ][ y ] are set equal to 1  <<  ( BitDepth − 1 ).

- Otherwise (at least one but not all samples refUnfilt[ x ][ y ] are marked as "not available for intra prediction"), the following ordered steps apply:

- When refUnfilt[ −1 − refIdx ][ refH − 1 ] is marked as "not available for intra prediction", search sequentially starting from x = −1 − refIdx, y = refH − 1 to x = −1 − refIdx, y = −1 − refIdx, then from x = −refIdx, y = −1 − refIdx to x = refW − 1, y = −1 − refIdx, for a sample refUnfilt[ x ][ y ] that is marked as "available for intra prediction". Once a sample refUnfilt[ x ][ y ] marked as "available for intra prediction" is found, the search is terminated and the value of refUnfilt[ −1 − refIdx ][ refH − 1 ] is set equal to the value of refUnfilt[ x ][ y ].

- For x = −1 − refIdx, y = refH − 2..−1 − refIdx, when refUnfilt[ x ][ y ] is marked as "not available for intra prediction", the value of refUnfilt[ x ][ y ] is set equal to the value of refUnfilt[ x ][ y + 1 ].

- For x = −refIdx..refW − 1, y = −1 − refIdx, when refUnfilt[ x ][ y ] is marked as "not available for intra prediction", the value of refUnfilt[ x ][ y ] is set equal to the value of refUnfilt[ x − 1 ][ y ].

All samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx are marked as "available for intra prediction".

The reference sample values p[ x ][ y ] are set equal to the sample values refUnfilt[ x ][ y ] with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

        - Reference sample filtering process

Inputs to this process are:

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable refFilterFlag specifying the value of reference filter flag,

- the (unfiltered) neighbouring samples refUnfilt[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the reference samples p[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- refFilterFlag is equal to 1

- Otherwise, filterFlag is set equal to 0.

For the derivation of the reference samples p[ x ][ y ] the following applies:

- If filterFlag is equal to 1, the filtered sample values p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 are derived as follows:

p[ −1 ][ −1 ] = ( refUnfilt[ −1 ][ 0 ] + 2 * refUnfilt[ −1 ][ −1 ] + refUnfilt[ 0 ][ −1 ] + 2 )  >>  2	(313)

p[ −1 ][ y ] = ( refUnfilt[ −1 ][ y + 1 ] + 2 * refUnfilt[ −1 ][ y ] + refUnfilt[ −1 ][ y − 1 ] + 2 )  >>  2
	for y = 0..refH − 2		(314)

p[ −1 ][ refH − 1 ] = refUnfilt[ −1 ][ refH − 1 ]		(315)

p[ x ][ −1 ] = ( refUnfilt[ x − 1 ][ −1 ] + 2 * refUnfilt[ x ][ −1 ] + refUnfilt[ x + 1 ][ −1 ] + 2 )  >>  2
	for x = 0..refW − 2 		(316)

p[ refW − 1 ][ −1 ] = refUnfilt[ refW − 1 ][ −1 ]		(317)

- Otherwise, the reference samples values p[ x ][ y ] are set equal to the unfiltered sample values refUnfilt[ x ][ y ] with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

        - Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable refFilterFlag specifying the value of reference filter flag,

- a variable cIdx specifying the colour component of the current block,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0:

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 25.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 25 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 24 | 14 | 2 | 0 | 0 | 0 |

Figure 9 – Intra prediction directions (informative)

Figure 9 illustrates the 93 prediction directions, where the dashed directions are associated with the wide-angle modes that are only applied to non-square blocks.

Table 26 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Table 26 – Specification of intraPredAngle

| predModeIntra | −14 | −13 | −12 | −11 | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | 512 | 341 | 256 | 171 | 128 | 102 | 86 | 73 | 64 | 57 | 51 | 45 | 39 | 35 | 32 | 29 | 26 |
| predModeIntra | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| intraPredAngle | 23 | 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 | 3 | 2 | 1 | 0 | −1 | −2 | −3 |
| predModeIntra | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| intraPredAngle | −4 | −6 | −8 | −10 | −12 | −14 | −16 | −18 | −20 | −23 | −26 | −29 | −32 | −29 | −26 | −23 | −20 |
| predModeIntra | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| intraPredAngle | −18 | −16 | −14 | −12 | −10 | −8 | −6 | −4 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 4 | 6 |
| predModeIntra | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 |
| intraPredAngle | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 |
| predModeIntra | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |  |  |  |  |  |  |  |  |  |
| intraPredAngle | 73 | 86 | 102 | 128 | 171 | 256 | 341 | 512 |  |  |  |  |  |  |  |  |  |

The inverse angle parameter invAngle is derived based on intraPredAngle as follows:

invAngle = Round512*32intraPredAngle		(325)

The interpolation filter coefficients fC[ phase ][ j ] and fG[ phase ][ j ] with phase = 0..31 and j = 0..3 are specified in Table 27.

Table 27 – Specification of interpolation filter coefficients fC and fG

| Fractional sample position p | fC interpolation filter coefficients |  |  |  | fG interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | −1 | 63 | 2 | 0 | 16 | 32 | 16 | 0 |
| 2 | −2 | 62 | 4 | 0 | 15 | 31 | 17 | 1 |
| 3 | −2 | 60 | 7 | −1 | 15 | 31 | 17 | 1 |
| 4 | −2 | 58 | 10 | −2 | 14 | 30 | 18 | 2 |
| 5 | −3 | 57 | 12 | −2 | 14 | 30 | 18 | 2 |
| 6 | −4 | 56 | 14 | −2 | 13 | 29 | 19 | 3 |
| 7 | −4 | 55 | 15 | −2 | 13 | 29 | 19 | 3 |
| 8 | −4 | 54 | 16 | −2 | 12 | 28 | 20 | 4 |
| 9 | −5 | 53 | 18 | −2 | 12 | 28 | 20 | 4 |
| 10 | −6 | 52 | 20 | −2 | 11 | 27 | 21 | 5 |
| 11 | −6 | 49 | 24 | −3 | 11 | 27 | 21 | 5 |
| 12 | −6 | 46 | 28 | −4 | 10 | 26 | 22 | 6 |
| 13 | −5 | 44 | 29 | −4 | 10 | 26 | 22 | 6 |
| 14 | −4 | 42 | 30 | −4 | 9 | 25 | 23 | 7 |
| 15 | −4 | 39 | 33 | −4 | 9 | 25 | 23 | 7 |
| 16 | −4 | 36 | 36 | −4 | 8 | 24 | 24 | 8 |
| 17 | −4 | 33 | 39 | −4 | 8 | 24 | 24 | 8 |
| 18 | −4 | 30 | 42 | −4 | 7 | 23 | 25 | 9 |
| 19 | −4 | 29 | 44 | −5 | 7 | 23 | 25 | 9 |
| 20 | −4 | 28 | 46 | −6 | 6 | 22 | 26 | 10 |
| 21 | −3 | 24 | 49 | −6 | 6 | 22 | 26 | 10 |
| 22 | −2 | 20 | 52 | −6 | 5 | 21 | 27 | 11 |
| 23 | −2 | 18 | 53 | −5 | 5 | 21 | 27 | 11 |
| 24 | −2 | 16 | 54 | −4 | 4 | 20 | 28 | 12 |
| 25 | −2 | 15 | 55 | −4 | 4 | 20 | 28 | 12 |
| 26 | −2 | 14 | 56 | −4 | 3 | 19 | 29 | 13 |
| 27 | −2 | 12 | 57 | −3 | 3 | 19 | 29 | 13 |
| 28 | −2 | 10 | 58 | −2 | 2 | 18 | 30 | 14 |
| 29 | −1 | 7 | 60 | −2 | 2 | 18 | 30 | 14 |
| 30 | 0 | 4 | 62 | −2 | 1 | 17 | 31 | 15 |
| 31 | 0 | 2 | 63 | −1 | 1 | 17 | 31 | 15 |

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = 0..nTbW + refIdx + 1	(326)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbH ) ],
	with x = −nTbH..−1	(327)

- Otherwise,

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = nTbW + 2 + refIdx..refW + refIdx	(328)

- The additional samples ref[ refW + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1 ) are derived as follows:

ref[ refW + refIdx + x ] = p[ −1 + refW ][ −1 − refIdx ]	(329)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( y + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(330)

iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31	(331)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(332)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(333)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(334)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(335)

- Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = 0..nTbH + refIdx + 1	(336)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

ref[ x ] = p[ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbW ) ][ −1 − refIdx ],
	with x = −nTbW..−1	(337)

- Otherwise,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = nTbH + 2 + refIdx..refH + refIdx	(338)

- The additional samples ref[ refH + refIdx +x ] with x = 1..( Max( 1, nTbH / nTbW ) * refIdx + 1) are derived as follows:

ref[ refH + refIdx +x ] = p[ −1 + refH ][ −1 − refIdx ]	(339)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( x + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(340)

iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31	(341)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(342)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 )  >>  6 )	(343)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(344)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(345)

Conclusion

This contribution proposes to simplify the intra reference sample filter selection. The proposed method provides the minor coding performance changes. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

- B. Bross, et al., “Versatile Video Coding (Draft 7),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P2001(JVET-P2001-vE), 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).