| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels,BE, 7–17January 2020 | Document: JVET-Q0296 |
| --- | --- |

| Title: | Non-CE5: Adaptive precision for CCALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Jie Yao, Jianqing Zhu, Kimihiko Kazui | Tel: Email: | +86-21-63350606 xuluhang@cn.fujitsu.com yaojie@cn.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution proposes a method to increase the precision of CCALF coefficients without computational complexity or coefficients length increment. It is proposed to use an Adaptive bitshift instead of constant bitshift to fix the floatingfilter coefficients when encoding. Simulation results show 0.01%(Y), -0.29%(U), -0.24%(V) BD-rates change for AI and 0.01%(Y), -0.40%(U), -0.41(V) BD-rates change for RA.

Introduction

In Geneva meeting, CCALF is agreed to study in CE5.

CCALF extracts the information from Luma to Chroma. (However, ALF extracts the information from the component itself.) This makes the range of CCALF coefficients different from ALF.

The CCALF coefficient length is limited to 6 bits for each (ranges from -32 to 31). In current CCALF design, the encoder will train the floating coefficients of CCALF and fix them into integer by multiplication with 128 and rounding process. It is observed that in some sequence fixed coefficient is greater than 31 but in some sequence fixed integer number of max coefficients are quite small.

Shifting with aninappropriate bit number will make coefficients less precise so the PSNR will also be affected.

Proposed method

This contribution proposes to use an adaptive bitshift for each CCALF filter to make fixing point process appropriate. This Adaptbitshiftdependson the maximum absolute coefficient in each filter and calculated by encoder with 3 bits limited range. (In experiment, Adaptbitshift ranges from 5 to 12, signal (Adaptbitshift - 5) for each CCALF filter).

The derivation of Adaptbitshift is shown as follow:

Adaptbitshift=Clip30,  7,  floorLog2round1÷MaxAbsValue+ Baseshift,      (1)

MaxAbsValue denotes the coefficient with maximum absolute value in each CCALF filter, Baseshift is set to '5' in experiment. By this Adaptbitshift, the fixed length (6 bits for current CCALF) will be fully utilized to increase the precision.

Table 1. Syntax specification of Adaptbitshift

| if ( alf_cross_component_cb_filter_signal_flag ) { |  |
| --- | --- |
| alf_cross_component_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cb_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 8; j++ ) |  |
| alf_cross_component_cb_coeff_plus32[ k ][ j ] | u(6) |
| alf_cross_component_cb_coeff_shift_minus5[k] | u(3) |
| } |  |
| } |  |
| if ( alf_cross_component_cr_ filter_signal_flag ) { |  |
| alf_cross_component_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cr_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 8; j++ ) |  |
| alf_cross_component_cr_coeff_plus32[ k ][ j ] | u(6) |
| alf_cross_component_cr_coeff_shift_minus5[k] | u(3) |
| } |  |
| } |  |

Decoder gets the Adaptbitshift for filtering instead using a fixed bitshift:

Oxc,ycCb=if'ui,viCb.Ix+ui,y+vi+1≪Adaptbitshift-1≫Adaptbitshift,	(2)

where Adaptbitshift is the number of bits for the decimal precision (5~12 bits in experiment) of the fixed point representation of the filter coefficients f'ui,viCb; and Ix+ui,y+vi is integer sample value with a integer precision equal to the internal bit depth for Luma (IBDL) samples (10 bits in CTC).

Test results

Test Results of AI and RA are shown as follows:

Table 2. AI results over CE5 anchor

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | -0.51% | -0.14% | -0.05% | 98% | 100% |
| Class A2 | 0.01% | -0.75% | -0.44% | -0.11% | 93% | 100% |
| Class B | 0.01% | -0.12% | -0.19% | -0.03% | 105% | 100% |
| Class C | 0.01% | -0.10% | -0.16% | -0.02% | 108% | 100% |
| Class E | 0.02% | -0.14% | -0.31% | -0.03% | 100% | 100% |
| Overall | 0.01% | -0.29% | -0.24% | -0.04% | 102% | 100% |
| Class D | 0.01% | -0.10% | -0.15% | -0.02% | 88% | #NUM! |
| Class F | 0.03% | -0.29% | -0.34% | -0.04% | 105% | #NUM! |

Table 3. RA results over CE5 anchor

|  |  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.03% | -1.02% | -0.50% | -0.13% | 102% | 100% |
| Class A2 | 0.02% | -0.40% | -0.24% | -0.05% | 102% | 100% |
| Class B | 0.01% | -0.22% | -0.65% | -0.08% | 101% | 100% |
| Class C | -0.02% | -0.15% | -0.16% | -0.05% | 103% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | -0.40% | -0.41% | -0.07% | 102% | 100% |
| Class D | -0.02% | -0.02% | -0.05% | -0.02% | 101% | 100% |
| Class F | 0.00% | -0.48% | -0.48% | -0.10% | 99% | 100% |

Conclusions

This method makes CCALF more flexible and improves the coding efficiency without computational complexity increment. It could also cooperate with other contributions in CE5. It is proposed to adopt this contribution with CCALF.

References

- B. Bross, J. Chen, and etc, “Versatile Video Coding (Draft 7),” JVET-P2001, Nov 2019.

- K. Misra, F. Bossen, A. Segall, N. Hu, J. Dong, V. Seregin, M. Karczewicz, P. Onno, C. Gisquet, G. Laroche, J. Li, C.S. Lim, C.-W. Kuo, J. Nam, J. Choi, J. Lim, S. Kim, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, "CE5-related: On the design of CC-ALF, " JVET-P1008, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019.

- F. Bossen, J. Boyce, and etc, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

Fujitsu may have current or pendingpatent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).