| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0303-v1 |
| --- | --- |

| Title: | LFNST signaling simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Yao, J.-Q. Zhu, K. Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

In current VVC, LFNST signaling constraint on LfnstDcOnly is ISP condition dependent. This contribution proposes to remove the ISP condition check for LFNST signaling so that making LFNST signaling consistent and simpler. The results reportedly show that the proposed method has negligable impact on coding efficiency and complexity.

The results reportedly show the average BDR gains are respectively 0.00% -0.03% 0.01% for All Intra configuration, and 0.00% 0.00% 0.02% for Random Access configuration over VTM-7.0.

Introduction

In current VVC [1], the LFNST (low-frequency non-separable transform) can be used in combination with ISP. When ISP mode is not selected, LFNST is restricted to be applicable only if one or more scan positions of the last significant coefficient (lastScanPos) within the CU are no less than 1, ie LfnstDcOnly == 0. In other word, If all lastScanPos within the CU are less than 1, LFNST index is not signaled and is inferred to 0. However when ISP mode is selected, the lastScanPos constraint is ignored and LFNST is always applicable and signaling.

Obviously, the LFNST signaling depends on ISP mode. It’s desirable to make LFNST signaling consistent and simpler.

7.3.9.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| … |  |
| } |  |

7.3.9.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| … |  |
| } |  |

Table 1. Syntax of LFNST signaling.

Proposed method

This contribution proposes to remove the ISP condition check for LFNST signaling. LFNST is restricted to be applicable only if one or more scan positions of the last significant coefficient (lastScanPos) within the CU are no less than 1, ie LfnstDcOnly == 0. The proposed spec text changes are shown in Table 2 highlighted in yellow.

Following above constraint, If ISP is selected, LFNST is not signaled and infer to be 0 when all lastScanPos are less than 1, ie LfnstDcOnly == 1.

7.3.9.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| … |  |
| } |  |

Table 2. Proposed syntax of LFNST signaling

Results

The proposed signaling is implemented upon VTM-7.0 and is evaluated for all-intra and random-access configuration according to the common test conditions in JVET-N1010 [2].

Table 3 shows summarized experimental results of the proposed method compared to VTM-7.0 with AI and RA configurations. The proposed method has no impact on coding efficiency and complexity. Encoding time is not reliable according to CPU resources.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | 0.08% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | -0.07% | 100% | 100% |
| Class B | 0.00% | 0.03% | -0.06% | 102% | 101% |
| Class C | -0.01% | 0.03% | 0.10% | 101% | 99% |
| Class E | 0.00% | -0.05% | 0.01% | 99% | 99% |
| Overall | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class D | 0.02% | -0.05% | -0.12% | 102% | 102% |
| Class F | 0.01% | 0.04% | 0.12% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | -0.01% | 99% | 100% |
| Class A2 | -0.05% | -0.05% | -0.07% | 104% | 100% |
| Class B | 0.03% | 0.04% | 0.11% | 99% | 100% |
| Class C | 0.00% | -0.03% | 0.00% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class D | -0.03% | 0.06% | -0.16% | 75% | 100% |
| Class F | 0.04% | 0.06% | 0.07% | 98% | 100% |

Table 3. Proposed method over VTM-7.0

Conclusions

This contribution proposes to remove the ISP condition check for LFNST signaling so that making LFNST signaling consistent and simpler. It is reportedly shown that the proposed method has no impact on coding efficiency and complexity. It is proposed to adopt this contribution.

Acknowledgments

The contribution has been kindly cross-checked by HHI. The cross-check results are available in the contribution JVET-Q0578.

References

- B. Bross, J. Chen, and etc, “Versatile Video Coding (Draft 7),” JVET-P2001, Nov 2019.

- F. Bossen, J. Boyce, and etc, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).