| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0305-v2 |
| --- | --- |

| Title: | Disallowing JCCR mode for ACT coded CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huan Dou Lidong Xu Yi-jen Chiu 8F Raycom Infotech Park A, Haidian District, Beijing, 100190, China | Tel: Email: | +86-10-8217-1034 huan.dou@intel.com lidong.xu@intel.com yi-jen.chiu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

In 4:4:4 mode of Work Draft 7 (VTM7.1), ACT could convert a CU’s luma and chroma channel residuals into YCgCo color space and then encode on the converted residual in YCgCo space. Followed by ACT on a CU, JCCR could be further applied to jointly encode the two converted chroma channel residuals in YCgCo color space. The combination of enabling both ACT and JCCR within a CU increases undesirably the length of residual construction pipe at video decoder side. This contribution proposes not to enable JCCR for the ACT coded CUs to improve the worst-case residual reconstruction pipe delay at decoder side along with the benefit to reduce the RDO checks at encoder side. With HEVC SCC test clips, the experiments report the BD rate of the proposal has a very small impact in RGB test clips, and a very small improvement in YUV test clips (in LDB case).

- When dual-tree is off

  - For RGB test clips, the average {G, B, R} BD-rates are {0.15%, 0.16%, 0.10%}, {0.25%, 0.20%, 0.09%} and {0.11%, 0.26%, -0.13%} for AI, RA and LDB, respectively

  - For YUV test clips, the average {Y, U, V} BD-rates are {0.00%, 0.01%, 0.00%}, {0.02%, 0.02%, 0.03%} and {-0.02%, -0.03%, -0.04%} for AI, RA and LDB, respectively

- When dual-tree is on

  - For RGB test clips, the average {G, B, R} BD-rates are {0.14%, 0.11%, 0.01%} and {0.02%, 0.25%, -0.09%} for RA and LDB, respectively

  - For YUV test clips, the average {Y, U, V} BD-rates are {0.00%, 0.01%, -0.01%} and {0.00%, -0.11%, -0.10%} for RA and LDB, respectively

The second version of this document updated the RA quality data (the data in the first version double counted the I-frame bitrate/PSNR of the parallel segments) and increased the precision of the AI/LDB quality data.

# Introduction

Adaptive color transform (ACT) was proposed in [1] and was adopted into VVC Work Draft 7. At encoder side, when the input chroma format is 4:4:4, ACT could convert a CU’s luma and chroma channel residuals into YCgCo color space and then encode the converted residuals in YCgCo. When ACT is applied to a CU, JCCR could be further applied to jointly encode the two converted chroma channel residuals in YCgCo color space into one converted joint chroma channel residual. The combination of enabling both ACT and JCCR within a CU increases undesirably the length of residual construction pipe at video decoder side.

# Proposal

This contribution proposes not to enable JCCR for the ACT coded CUs to improve the worst-case residual reconstruction pipe delay at video decoder side along with the benefit to reduce the RDO checks at video encoder side for the case that both ACT and JCCR are applied.

With this proposal, when ACT is applied to a CU, i.e., cu_act_enabled_flag is true, it becomes unnecessary to signal the corresponding TU JCCR flag tu_joint_cbcr_residual_flag in bitstream. The flag tu_joint_cbcr_residual_flag is inferred to be false. The change of Spec description is proposed as below.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ¦ ¦ ( tu_cbf_cb[ xC ][ yC ] && tu_cbf_cr[ xC ][ yC ] ) ) && chromaAvailable && !cu_act_enabled_flag) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| …… |  |
| } |  |

# Simulation results

The proposal was implemented on VTM-7.1 [2] and the experiments were simulated under the ACT test conditions used in [3] with the test sequences used in HEVC SCC [4]. Both dual-tree on and dual-tree off cases were tested. In dual-tree on case, ACT is turned off by default and thus there is no impact to BD rate in All Intra case. The experiments report the BD rate of the proposal has a very small impact in RGB test clips, and a very small improvement in YUV test clips (in LDB case).

- When dual-tree is off

  - For RGB test clips, the average {G, B, R} BD-rates are {0.15%, 0.16%, 0.10%}, {0.25%, 0.20%, 0.09%} and {0.11%, 0.26%, -0.13%} for AI, RA and LDB, respectively

  - For YUV test clips, the average {Y, U, V} BD-rates are {0.00%, 0.01%, 0.00%}, {0.02%, 0.02%, 0.03%} and {-0.02%, -0.03%, -0.04%} for AI, RA and LDB, respectively

- When dual-tree is on

  - For RGB test clips, the average {G, B, R} BD-rates are {0.14%, 0.11%, 0.01%} and {0.02%, 0.25%, -0.09%} for RA and LDB, respectively

  - For YUV test clips, the average {Y, U, V} BD-rates are {0.00%, 0.01%, -0.01%} and {0.00%, -0.11%, -0.10%} for RA and LDB, respectively

## BD-rate quality impact in dual-tree on case

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.22% | 0.12% | 0.06% |
| RGB, mixed content, 1440p & 1080p | 0.00% | 0.05% | -0.02% |
| RGB, Animation, 720p | 0.04% | 0.10% | 0.02% |
| RGB, camera captured, 1080p | 0.12% | 0.18% | -0.18% |
| YUV, text & graphics with motion, 1080p & 720p | 0.01% | 0.00% | -0.02% |
| YUV, mixed content, 1440p & 1080p | 0.01% | 0.03% | 0.01% |
| YUV, Animation, 720p | -0.01% | -0.01% | -0.02% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.01% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.11% | 0.25% | 0.10% |
| RGB, mixed content, 1440p & 1080p | -0.12% | 0.06% | -0.11% |
| RGB, Animation, 720p | 0.05% | 0.31% | 0.00% |
| RGB, camera captured, 1080p | -0.13% | 0.49% | -0.83% |
| YUV, text & graphics with motion, 1080p & 720p | 0.05% | -0.11% | -0.02% |
| YUV, mixed content, 1440p & 1080p | -0.07% | -0.12% | -0.26% |
| YUV, Animation, 720p | -0.11% | -0.24% | -0.35% |
| YUV, camera captured, 1080p | -0.03% | -0.06% | -0.05% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

## BD-rate quality impact in dual-tree off case

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.21% | 0.19% | 0.13% |
| RGB, mixed content, 1440p & 1080p | 0.05% | 0.11% | 0.05% |
| RGB, Animation, 720p | 0.07% | 0.18% | 0.05% |
| RGB, camera captured, 1080p | 0.11% | 0.11% | 0.07% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.00% | 0.02% | 0.01% |
| YUV, Animation, 720p | 0.00% | 0.01% | 0.00% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.32% | 0.24% | 0.14% |
| RGB, mixed content, 1440p & 1080p | 0.07% | 0.21% | 0.06% |
| RGB, Animation, 720p | -0.02% | 0.12% | -0.03% |
| RGB, camera captured, 1080p | 0.36% | 0.12% | -0.01% |
| YUV, text & graphics with motion, 1080p & 720p | 0.01% | 0.01% | 0.02% |
| YUV, mixed content, 1440p & 1080p | 0.05% | 0.04% | 0.11% |
| YUV, Animation, 720p | 0.03% | 0.04% | -0.04% |
| YUV, camera captured, 1080p | 0.02% | 0.03% | 0.00% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.27% | 0.27% | 0.02% |
| RGB, mixed content, 1440p & 1080p | -0.02% | 0.07% | -0.13% |
| RGB, Animation, 720p | -0.15% | 0.33% | -0.02% |
| RGB, camera captured, 1080p | -0.17% | 0.49% | -0.77% |
| YUV, text & graphics with motion, 1080p & 720p | 0.01% | 0.00% | 0.02% |
| YUV, mixed content, 1440p & 1080p | -0.18% | -0.04% | -0.18% |
| YUV, Animation, 720p | 0.11% | -0.16% | -0.34% |
| YUV, camera captured, 1080p | 0.03% | -0.09% | 0.09% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 97% |  |  |

# Reference

X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, “Support of adaptive color transform for 444 video coding in VVC”, JVET-P0517, Oct. 2019, Geneva, Switzerland.

VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.1.

X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, July 2019, Gothenburg, Sweden.

H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common test conditions for screen content coding”, JCTVC-U1015, June 2015, Warsaw, Poland

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).