| -Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0313-v2 |
| --- | --- |

| Title: | Non-CE4: Constraints on block size for ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Current ATMVP applies on blocks which height or width not less than 8, and was put in the first place in sub-block merge list if it is available after checking the related spatial and temporal motion information. However, after the adoption of remove 4x4 sub-blocks for ATMVP mode. For an 8x8 block, the ATMVP is no longer a sub-block mode which has only one sub-block. This makes 8x8 ATMVP mode degenerates into a regular temporal merge mode. This contribution presents size constrains on ATMVP mode to remove 8x8 ATMVP candidate when construct sub-block merge list, only one line modification in the spec. After applying this modification, the spatial and temporal check is no longer needed for 8x8 block’s sub-block merge list construction. The test results reportedly show that apply this modification won’t influence the coding performance of ATMVP.

•	RA: 0.01% (Y), 0.01% (U), 0.03% (V), 100% (EnT), 100% (DeT)

•	LD: 0.00% (Y), -0.01% (U), -0.13% (V), 100% (EnT), 99% (DeT)

Proposed modification

This contribution presents removing 8x8 block size for ATMVP mode. When applying ATMVP on 8x8 block, ATMVP mode degenerates into a special regular merge mode.

Spec modification:

- If one or more of the following conditions is true, availableFlagSbCol is set equal to 0.

- slice_temporal_mvp_enabled_flag is equal to 0.

- sps_sbtmvp_enabled_flag is equal to 0.

- cbWidth is less than 8.

- cbHeight is less than 8.

- cbWidth is equal to 8 and cbHeight is equal to 8.

Performance results

The performance result is summarised in Table 1 and Table 2. The tests were performed using the JVET common test conditions with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 7.0.

Table 1 - Performance of RA.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.05% | 0.03% | 100% | 99% |
| Class A2 | -0.01% | -0.01% | 0.03% | 100% | 102% |
| Class B | 0.01% | -0.02% | 0.02% | 100% | 98% |
| Class C | 0.02% | 0.03% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.03% | 100% | 100% |
| Class D | 0.01% | -0.10% | 0.06% | 100% | 100% |
| Class F | 0.01% | 0.00% | 0.00% | 100% | 100% |

Table 2 - Performance of LD.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.10% | 0.14% | 100% | 101% |
| Class C | 0.05% | -0.19% | -0.05% | 100% | 97% |
| Class E | -0.09% | 0.37% | -0.68% | 99% | 99% |
| Overall | 0.00% | -0.01% | -0.13% | 100% | 99% |
| Class D | -0.01% | 0.25% | 0.25% | 100% | 100% |
| Class F | 0.02% | -0.42% | -0.11% | 100% | 100% |

Conclusion

In this contribution, block size constraint on ATMVP is tested. Compare to VTM 7.0, no coding performance change was found. It is recommended to adopt this modification.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).