| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0315 |
| --- | --- |

| Title: | Non-CE4: Triangular prediction mode with motion vector difference |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Zhipin Deng, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, Triangular prediction mod with Motion Vector Difference (TMVD) is proposed. With TMVD, the MV of a triangular partition in TPM is refined as a sum of a MV derived from a merge candidate and a signalled MVD. The MVD is signalled in a similar way as MMVD, wherein indices of a direction and a distance are coded. Simulation results reported as below:

RA: -0.18%, 103%, 98%

LB: -0.29%, 105%, 100%

# Introduction

With Triangular Prediction Mode (TPM) in VVC, a block is split into two triangular partitions. The MV of a triangular partition is set to be a MV derived from a merge candidate. The MV may not be accurate enough since there is no motion estimation for the coded triangular region.

# Triangular Prediction with Motion Vector Difference

To address the problem in TPM, Triangular prediction mode with Motion Vector Difference (TMVD) is proposed. Each triangular partition in TPM can decide to use TMVD or not. If TMVD is chosen for a triangular region, the MV of the region is calculated as a sum of the MV of a merge candidate and an MVD. All other processing is kept the same as in TPM.

With TMVD, an MVD is signaled as a pair of direction and distance, similar as in MMVD. There are nine candidate distances (¼-pel, ½-pel, 1-pel, 2-pel, 3-pel, 4-pel, 6-pel, 8-pel, 16-pel), and eight candidate directions (four horizontal/vertical directions and four diagonal directions) involved in TMVD.

When pic_fpel_mmvd_enabled_flag is equal to 1, the MVD in TMVD is left shifted by 2 as in MMVD.

# Experimental Results

The proposed method has been implemented on top of VTM-7.0. Results under RA/LB configurations are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results of TMVD

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.17% | -0.35% | 103% | 99% |  |  |  |  |  |
| Class A2 | -0.21% | -0.34% | -0.34% | 103% | 98% |  |  |  |  |  |
| Class B | -0.13% | -0.22% | -0.20% | 104% | 99% | -0.21% | -0.54% | -0.35% | 105% | 100% |
| Class C | -0.28% | -0.56% | -0.55% | 103% | 98% | -0.43% | -0.95% | -0.79% | 106% | 99% |
| Class E |  |  |  |  |  | -0.25% | -0.23% | -0.95% | 105% | 100% |
| Overall | -0.18% | -0.33% | -0.35% | 103% | 98% | -0.29% | -0.60% | -0.65% | 105% | 100% |
| Class D | -0.29% | -0.56% | -0.52% | 104% | 101% | -0.49% | -0.52% | -1.01% | 107% | 102% |
| Class F | -0.10% | -0.13% | -0.12% | 103% | 100% | -0.21% | -0.81% | -0.32% | 104% | 99% |

# Conclusions

With the proposed method can achieve 0.18% (RA)/0.29% (LB) BD-rate reduction with almost no burden at decoder side. It is suggested adopting TMVD to the next version of VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.