| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0316 |
| --- | --- |

| Title: | AHG9: On signaling of the wraparound offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Ye-Kui Wang, Hongbin Liu, Jizheng Xu, Zhipin Deng 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VVC, the wraparound offset is signaled in SPS by sps_ref_wraparound_offset_minus1. However, the picture width and height in a CLVS may vary when RPR is applied. In this contribution, it is proposed to signal the wraparound offset in PPS, coinciding with the picture width and height.

# Introduction

In VVC, a wraparound offset is signaled in SPS for the 360 applications. However, picture resolutions in a CLVS may vary when RPR is applied. A fixed wraparound offset may not be suitable for pictures with different resolutions. Furthermore, this makes the SPS semantics dependent on PPS syntax, like the following, making it difficult for checking SPS syntax conformance while parsing SPS (and before parsing PPS):

When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0.

# Proposed methods

In this contribution, it is proposed to signal the wraparound offset in PPS, coinciding with the picture width and height.

The working draft changes based on JVET-P2001-v14 are as follows, with the deleted text and newly added text highlighted in turquois and yellow, respectively.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( sps_bdpcm_enabled_flag && chroma_format_idc = = 3 ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| … |  |

#### 7.3.2.3 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pps_ref_wraparound_enabled_flag | u(1) |
| if( pps_ref_wraparound_enabled_flag ) |  |
| pps_ref_wraparound_offset_minus1 | ue(v) |
| conformance_window_flag | u(1) |
| … |  |

pps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. pps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), the value of pps_ref_wraparound_enabled_flag shall be equal to 0.

pps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of pps_ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

#### 7.4.4.2 General constraint information semantics

…

no_ref_wraparound_constraint_flag equal to 1 specifies that sps_ref_wraparound_enabled_flag pps_ref_wraparound_enabled_flag shall be equal to 0. no_ref_wraparound_constraint_flag equal to 0 does not impose such a constraint.

##### 8.5.3.2.2 Luma sample bilinear interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i )	(642)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )	(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag pps_ref_wraparound_enabled_flag ?
		ClipH( ( pps_ref_wraparound_offset_minus1 pps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(645)

…

##### 8.5.6.3.2 Luma sample interpolation filtering process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i − 3 )	(955)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )	(956)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag pps_ref_wraparound_enabled_flag ?
		ClipH( ( pps_ref_wraparound_offset_minus1 pps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(957) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(958)

…

##### 8.5.6.3.3 Luma integer sample fetching process

…

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL )	(966)

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL )	(967)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag pps_ref_wraparound_enabled_flag ?	(968)
		ClipH( ( pps_ref_wraparound_offset_minus1 pps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(969)

…

##### 8.5.6.3.4 Chroma sample interpolation process

…

The variable xOffset is set equal to ( pps_ref_wraparound_offset_minus1 pps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, xIntL + i )	(971)

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntL + i )	(972)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag pps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )

…

# Conclusions

Since the proposed method harmonizes the design of RPR and picture wraparound, it is suggested adopting this proposal to VVC.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.