| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0324-v1 |
| --- | --- |

| Title: | Simplification of MV Derivation for Affine Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution proposes to simplify the MV derivation process for chroma sub-block of an affine CU. (1) For 4:2:0 format, the motion vector of a chroma 4x4 sub-block is proposed to use only top-left luma sub-block’s MV; (2) For 4:2:2 format, the motion vector of a chroma 4x4 sub-block is proposed to use only the left luma sub-block’s MV. Experimental results of the two tests are summarized as follows:

Test 1: Simplification of MV derivation for 420 format:

RA: 0.04%, 0.24%, 0.25%

LDB: 0.01%, 0.01%, 0.04%

Test 2: Simplification of MV derivation for 422 format:

RA: 0.00%, -0.02%, -0.01%

LDB:

Introduction

In current design[1], for 4:2:0 format, the MV of a chroma 4x4 sub-block is defined based on the average of the top-left sub-block’s MV and the right-bottom sub-block’s MV. For 4:2:2 format, the MV of a chroma 4x4 sub-block is defined based on the two horizontally-adjacent luma sub-blocks, as shown in Fig.1.

- (b)

Fig. 1 MV derivation for affine chroma sub-blocks:

(a) 4:2:0 format (b) 4:2:2 format

To simplify the MV derivation process for affine chroma sub-block, this contribution proposes several options.

Proposed method

In this contribution, average process is proposed to be removed from MV derivation process for affine chroma.

Method 1: for 4:2:0 format, chroma MV is identical to the corresponding top-left luma sub-block’s MV.

Method 2: for 4:2:2 format, chroma MV is identical to the corresponding left luma sub-block’s MV.

(a)                        (b)

Fig. 2 Proposed method: (a) 4:2:0 format (b) 4:2:2 format

Experimental results

The experiments were performed based on VTM-7.0 software[2] with VTM configuration files under JVET common test condition[3] for 4:2:0 test sequences and JCT-VC RExt CTC test set [4] for 4:2:2 sequences. The experiments were conducted under Ubuntu Linux. The encoding time is unreliable. Two tests are conducted,

Test 1: 4:2:0 format, use top-left luma sub-block’s MV for affine chroma MV derivation

Table 1. Experimental result of the proposed method (Test1)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.14% | 113% | 100% |
| Class A2 | 0.13% | 0.80% | 0.56% | 105% | 96% |
| Class B | 0.02% | 0.18% | 0.24% | 104% | 89% |
| Class C | 0.03% | 0.13% | 0.10% | 97% | 84% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.24% | 0.25% | 104% | 91% |
| Class D | 0.00% | 0.29% | 0.24% | 84% | 69% |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.08% | 0.05% | 120% | 99% |
| Class C | -0.02% | -0.03% | -0.01% | 119% | 103% |
| Class E | 0.03% | 0.22% | 0.11% | 115% | 91% |
| Overall | 0.01% | 0.01% | 0.04% | 119% | 98% |
| Class D | 0.04% | 0.68% | 1.03% | 125% | 128% |
| Class F | 0.06% | 0.13% | 1.11% | 119% | 99% |

Test 2: 4:2:2 format, use top luma sub-block’s MV for affine chroma MV derivation

Table 2. Experimental result of the proposed method (Test2)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncR | DecR |
| Traffic | 0.01% | -0.06% | -0.07% | 100% | 102% |
| Kimono | -0.04% | 0.00% | 0.06% | 100% | 95% |
| Seeking | 0.01% | 0.00% | 0.04% | 99% | 97% |
| EBUHorse | 0.00% | 0.07% | -0.01% | 100% | 94% |
| EBUGraphics | 0.00% | 0.02% | 0.04% | 99% | 98% |
| EBUWaterRocksClose | 0.01% | -0.06% | 0.00% | 79% | 85% |
| EBUKidsSoccer | -0.01% | -0.10% | -0.13% | 90% | 91% |
| Overall | 0.00% | -0.02% | -0.01% | 95% | 95% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncR | DecR |
| Traffic |  |  |  |  |  |
| Kimono |  |  |  |  |  |
| Seeking |  |  |  |  |  |
| EBUHorse | -0.09% | 0.02% | -0.17% | 97% | 99% |
| EBUGraphics |  |  |  |  |  |
| EBUWaterRocksClose |  |  |  |  |  |
| EBUKidsSoccer |  |  |  |  |  |
| Overall |  |  |  |  |  |

Proposed Working Draft Changes

Method 1:

#### 8.5.5.9  Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the reference index refIdxLX and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1 and X being 0 or 1,

- the prediction refinement utilization flag cbProfFlagLX and X being 0 or 1,

- the motion vector difference array diffMvLX[ xIdx ][ yIdx ] with xIdx = 0..cbWidth / numSbX − 1, yIdx = 0..cbHeight / numSbY – 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(878)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(879)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(880)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(881)

log2CbH = Log2( cbHeight )		(882)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(883)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(884)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(885)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(886)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(887)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(888)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(889)

dVerY = dHorX		(890)

The variable fallbackModeTriggered is set equal to 1 and modified as follows:

- The variables bxWX4, bxHX4, bxWXh, bxHXh, bxWXvand bxHXv are derived as follows:

maxW4 = Max( 0, Max( 4 * ( 2048 + dHorX ), 	
			Max( 4*dHorY, 4 * ( 2048 + dHorX ) + 4 * dHorY ) ) ) 	(891)

minW4 = Min( 0, Min( 4 * ( 2048 + dHorX ), 	
			Min( 4*dHorY, 4 * ( 2048 + dHorX ) + 4 * dHorY ) ) ) 	(892)

maxH4 = Max( 0, Max( 4 * dVerX, 		
			Max( 4* ( 2048 + dVerY ), 4 * dVerX + 4 * ( 2048 + dVerY ) ) ) ) 	(893)

minH4 = Min( 0, Min( 4 * dVerX, 		
			Min( 4* ( 2048 + dVerY ), 4 * dVerX + 4 * ( 2048 + dVerY ) ) ) ) 	(894)

bxWX4 = ( ( maxW4 − minW4 ) >> 11 ) + 9		(895)

bxHX4 = ( ( maxH4 − minH4 ) >> 11 ) + 9		(896)

bxWXh = ( (Max( 0, 4 * ( 2048 + dHorX ) ) − Min( 0, 4 * ( 2048 + dHorX ) ) ) >> 11 ) + 9	(897)

bxHXh = ( ( Max( 0, 4 * dVerX ) − Min( 0, 4 * dVerX ) ) >> 11 ) + 9	(898)

bxWXv = ( ( Max( 0, 4 * dHorY ) − Min( 0, 4 * dHorY ) ) >> 11 ) + 9	(899)

bxHXv =  ( ( Max( 0, 4 * ( 2048 + dVerY ) ) − Min( 0, 4 * ( 2048 + dVerY ) ) ) >> 11 ) + 9	(900)

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_BI and bxWX4 * bxHX4 is less than or equal to 225,  fallbackModeTriggered is set equal to 0.

- Otherwise, if both bxWXh * bxHXh is less than or equal to 165 and bxWXv * bxHXv is less than or equal to 165, fallbackModeTriggered is set equal to 0.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xPosCb and yPosCb are derived as follows

- If fallbackModeTriggered is equal to 1, the following applies:

xPosCb = ( cbWidth >> 1 )		(901)

yPosCb = ( cbHeight >> 1 )		(902)

- Otherwise (fallbackModeTriggered is equal to 0), the following applies:

xPosCb = 2 + ( xSbIdx << 2 )		(903)

yPosCb = 2 + ( ySbIdx << 2 )		(904)

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(905)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(906)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

- The motion vectors mvLX[ xSbIdx ][ ySbIdx ] are clipped as follows:

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 0 ] )	(907)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 1 ] )	(908)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

  - If both SubWidthC and SubHeightC are equal to 1, the following applies:

mvAvgLX = mvLX[ xSbIdx ][ ySbIdx ]		(909)

  - Otherwise, the following applies:

    - If both SubWidthC and SubHeightC are equal to 2, the following applies:

mvAvgLX = mvLX[ ( xSbIdx >> ( SubWidthC − 1 ) << ( SubWidthC − 1 ) ) ]

[ (ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) ]	                 (910)

    - Otherwise, the following applies:

mvAvgLX = mvLX[ ( xSbIdx >> ( SubWidthC − 1 ) << ( SubWidthC − 1 ) ) ]	
			[ (ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) ] + 	
			mvLX[ ( xSbIdx >> (SubWidthC − 1 ) << ( SubWidthC − 1) ) + ( SubWidthC − 1 ) ]	(911)
			[ ( ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) + ( SubHeightC − 1 ) ]

mvAvgLX[ 0 ] = ( mvAvgLX[ 0 ] + 1 − ( mvAvgLX[ 0 ] >= 0 ) )  >>  1	(912)

mvAvgLX[ 1 ] = ( mvAvgLX[ 1 ] + 1 − ( mvAvgLX[ 1 ] >= 0 ) )  >>  1	(913)

- The derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with mvAvgLX and refIdxLX as inputs, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

Method 2:

#### 8.5.5.9  Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the reference index refIdxLX and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1 and X being 0 or 1,

- the prediction refinement utilization flag cbProfFlagLX and X being 0 or 1,

- the motion vector difference array diffMvLX[ xIdx ][ yIdx ] with xIdx = 0..cbWidth / numSbX − 1, yIdx = 0..cbHeight / numSbY – 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(878)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(879)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(880)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(881)

log2CbH = Log2( cbHeight )		(882)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(883)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(884)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(885)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(886)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(887)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(888)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(889)

dVerY = dHorX		(890)

The variable fallbackModeTriggered is set equal to 1 and modified as follows:

- The variables bxWX4, bxHX4, bxWXh, bxHXh, bxWXvand bxHXv are derived as follows:

maxW4 = Max( 0, Max( 4 * ( 2048 + dHorX ), 	
			Max( 4*dHorY, 4 * ( 2048 + dHorX ) + 4 * dHorY ) ) ) 	(891)

minW4 = Min( 0, Min( 4 * ( 2048 + dHorX ), 	
			Min( 4*dHorY, 4 * ( 2048 + dHorX ) + 4 * dHorY ) ) ) 	(892)

maxH4 = Max( 0, Max( 4 * dVerX, 		
			Max( 4* ( 2048 + dVerY ), 4 * dVerX + 4 * ( 2048 + dVerY ) ) ) ) 	(893)

minH4 = Min( 0, Min( 4 * dVerX, 		
			Min( 4* ( 2048 + dVerY ), 4 * dVerX + 4 * ( 2048 + dVerY ) ) ) ) 	(894)

bxWX4 = ( ( maxW4 − minW4 ) >> 11 ) + 9		(895)

bxHX4 = ( ( maxH4 − minH4 ) >> 11 ) + 9		(896)

bxWXh = ( (Max( 0, 4 * ( 2048 + dHorX ) ) − Min( 0, 4 * ( 2048 + dHorX ) ) ) >> 11 ) + 9	(897)

bxHXh = ( ( Max( 0, 4 * dVerX ) − Min( 0, 4 * dVerX ) ) >> 11 ) + 9	(898)

bxWXv = ( ( Max( 0, 4 * dHorY ) − Min( 0, 4 * dHorY ) ) >> 11 ) + 9	(899)

bxHXv =  ( ( Max( 0, 4 * ( 2048 + dVerY ) ) − Min( 0, 4 * ( 2048 + dVerY ) ) ) >> 11 ) + 9	(900)

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_BI and bxWX4 * bxHX4 is less than or equal to 225,  fallbackModeTriggered is set equal to 0.

- Otherwise, if both bxWXh * bxHXh is less than or equal to 165 and bxWXv * bxHXv is less than or equal to 165, fallbackModeTriggered is set equal to 0.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xPosCb and yPosCb are derived as follows

- If fallbackModeTriggered is equal to 1, the following applies:

xPosCb = ( cbWidth >> 1 )		(901)

yPosCb = ( cbHeight >> 1 )		(902)

- Otherwise (fallbackModeTriggered is equal to 0), the following applies:

xPosCb = 2 + ( xSbIdx << 2 )		(903)

yPosCb = 2 + ( ySbIdx << 2 )		(904)

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(905)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(906)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

- The motion vectors mvLX[ xSbIdx ][ ySbIdx ] are clipped as follows:

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 0 ] )	(907)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 1 ] )	(908)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

  - If both SubWidthC and SubHeightC are equal to 1, the following applies:

mvAvgLX = mvLX[ xSbIdx ][ ySbIdx ]		(909)

  - Otherwise, the following applies:

    - If SubWidthC is equal to 2 and SubHeightC is equal to 1 , the following applies:

mvAvgLX = mvLX[ ( xSbIdx >> ( SubWidthC − 1 ) << ( SubWidthC − 1 ) ) ]

[ (ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) ]	                 (910)

    - Otherwise, the following applies:

mvAvgLX = mvLX[ ( xSbIdx >> ( SubWidthC − 1 ) << ( SubWidthC − 1 ) ) ]	
			[ (ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) ] + 	
			mvLX[ ( xSbIdx >> (SubWidthC − 1 ) << ( SubWidthC − 1) ) + ( SubWidthC − 1 ) ]	(911)
			[ ( ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) + ( SubHeightC − 1 ) ]

mvAvgLX[ 0 ] = ( mvAvgLX[ 0 ] + 1 − ( mvAvgLX[ 0 ] >= 0 ) )  >>  1	(912)

mvAvgLX[ 1 ] = ( mvAvgLX[ 1 ] + 1 − ( mvAvgLX[ 1 ] >= 0 ) )  >>  1	(913)

- The derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with mvAvgLX and refIdxLX as inputs, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

Method 1&2

#### 8.5.5.9  Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the reference index refIdxLX and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1 and X being 0 or 1,

- the prediction refinement utilization flag cbProfFlagLX and X being 0 or 1,

- the motion vector difference array diffMvLX[ xIdx ][ yIdx ] with xIdx = 0..cbWidth / numSbX − 1, yIdx = 0..cbHeight / numSbY – 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(878)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(879)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(880)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(881)

log2CbH = Log2( cbHeight )		(882)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(883)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(884)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(885)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(886)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(887)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(888)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(889)

dVerY = dHorX		(890)

The variable fallbackModeTriggered is set equal to 1 and modified as follows:

- The variables bxWX4, bxHX4, bxWXh, bxHXh, bxWXvand bxHXv are derived as follows:

maxW4 = Max( 0, Max( 4 * ( 2048 + dHorX ), 	
			Max( 4*dHorY, 4 * ( 2048 + dHorX ) + 4 * dHorY ) ) ) 	(891)

minW4 = Min( 0, Min( 4 * ( 2048 + dHorX ), 	
			Min( 4*dHorY, 4 * ( 2048 + dHorX ) + 4 * dHorY ) ) ) 	(892)

maxH4 = Max( 0, Max( 4 * dVerX, 		
			Max( 4* ( 2048 + dVerY ), 4 * dVerX + 4 * ( 2048 + dVerY ) ) ) ) 	(893)

minH4 = Min( 0, Min( 4 * dVerX, 		
			Min( 4* ( 2048 + dVerY ), 4 * dVerX + 4 * ( 2048 + dVerY ) ) ) ) 	(894)

bxWX4 = ( ( maxW4 − minW4 ) >> 11 ) + 9		(895)

bxHX4 = ( ( maxH4 − minH4 ) >> 11 ) + 9		(896)

bxWXh = ( (Max( 0, 4 * ( 2048 + dHorX ) ) − Min( 0, 4 * ( 2048 + dHorX ) ) ) >> 11 ) + 9	(897)

bxHXh = ( ( Max( 0, 4 * dVerX ) − Min( 0, 4 * dVerX ) ) >> 11 ) + 9	(898)

bxWXv = ( ( Max( 0, 4 * dHorY ) − Min( 0, 4 * dHorY ) ) >> 11 ) + 9	(899)

bxHXv =  ( ( Max( 0, 4 * ( 2048 + dVerY ) ) − Min( 0, 4 * ( 2048 + dVerY ) ) ) >> 11 ) + 9	(900)

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_BI and bxWX4 * bxHX4 is less than or equal to 225,  fallbackModeTriggered is set equal to 0.

- Otherwise, if both bxWXh * bxHXh is less than or equal to 165 and bxWXv * bxHXv is less than or equal to 165, fallbackModeTriggered is set equal to 0.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xPosCb and yPosCb are derived as follows

- If fallbackModeTriggered is equal to 1, the following applies:

xPosCb = ( cbWidth >> 1 )		(901)

yPosCb = ( cbHeight >> 1 )		(902)

- Otherwise (fallbackModeTriggered is equal to 0), the following applies:

xPosCb = 2 + ( xSbIdx << 2 )		(903)

yPosCb = 2 + ( ySbIdx << 2 )		(904)

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(905)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(906)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

- The motion vectors mvLX[ xSbIdx ][ ySbIdx ] are clipped as follows:

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 0 ] )	(907)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 1 ] )	(908)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

  - If both SubWidthC and SubHeightC are equal to 1, the following applies:

mvAvgLX = mvLX[ xSbIdx ][ ySbIdx ]		(909)

  - Otherwise, the following applies:

mvAvgLX = mvLX[ ( xSbIdx >> ( SubWidthC − 1 ) << ( SubWidthC − 1 ) ) ]	
			[ (ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) ] + 	
			mvLX[ ( xSbIdx >> (SubWidthC − 1 ) << ( SubWidthC − 1) ) + ( SubWidthC − 1 ) ]	(910)
			[ ( ySbIdx >> ( SubHeightC − 1 ) << ( SubHeightC − 1 ) ) + ( SubHeightC − 1 ) ]

mvAvgLX[ 0 ] = ( mvAvgLX[ 0 ] + 1 − ( mvAvgLX[ 0 ] >= 0 ) )  >>  1	(911)

mvAvgLX[ 1 ] = ( mvAvgLX[ 1 ] + 1 − ( mvAvgLX[ 1 ] >= 0 ) )  >>  1	(912)

- The derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with mvAvgLX and refIdxLX as inputs, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

Conclusion

This contribution proposes to simplify the MV derivation process for chroma sub-block of an affine CU by removing the average process. The proposed methods show marginal coding performance change and it is suggested to adopt the proposed method into the next VVC working draft text.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

- Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006-N1028, January 2014

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).