| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0327-v4 |
| --- | --- |

| Title: | On IBC/ATMVP candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng1, Xuewei Meng2, Suhong Wang2, Shanshe Wang2, Siwei Ma2 SZ DJI Technology Co., Ltd.1, Peking University2 | Email: | suhong.wang@pku.edu.cn xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

In this contribution, IBC and ATMVP candidate list construction process is modified to better harmonize with normal merge candidate list. As the first two merge candidates are swapped in the current VTM-7.0, we propose to swap the first two IBC merge candidate and IBC inter candidate. And change the derivation process of motion shift in ATMVP to only check the top block of the current block. Experimental results of three tests are summarized as follows,

Test1: Modification of IBC candidate list construction

AI:   Class F, 0.00%; Class TGM, -0.06%

RA:  Class F, -0.03%; Class TGM, 0.05%

LDB: Class F, -0.11%; Class TGM, -0.05%

Test2: Modification of ATMVP

RA:  0.01%, 0.03%, 0.03%

LDB: -0.01%, -0.07%, -0.16%

Test3: Combination of Test1 and Test2.

RA:  Class F, -0.02%; Class TGM, 0.03%

LDB: Class F, 0.02%; Class TGM, -0.10%

Introduction

The derivation of spatial merge candidates in the current VVC-7.0[1] is same to that in HEVC except the positions of first two merge candidates are swapped because of the adoption of JVET-P0325[2], as shown in Fig. 1. The order of derivation is B1, A1, B0, A0 and B2.

Figure 1.  Position of spatial merge candidate

However, the order of derivation process for IBC spatial merge candidate and IBC AMVP candidate is A1, B1, which is still the same as before.

Figure 2.  Position of IBC merge/inter candidate

For ATMVP, at the very beginning, the motion shift is derived by scanning the spatial merge list. During the previous meetings, this process has been simplified to check the first candidate in the spatial merge list, also known as block A1. However, due the the change introduced by JVET-P0325, the first candidate is no longer A1, but block B1.

Figure 3.  Current design on ATMVP motion shift derivation

Proposed Method

To resolve these problems mentioned above, the proposed method is as follows,

IBC candidate list construction

To align the derivation process in IBC with spatial merge candidate list construction process, we propose to swap the two spatial candidates. The order of derivation process for IBC spatial merge candidate and IBC AMVP candidate is B1, A1.

ATMVP

In order to align the motion derivation process in ATMVP with spatial merge candidate list construction process, we propose to check the top block of the current block, also known as block B1.

Figure 4.  Proposed method on ATMVP motion shift derivation

Simulation results

The experiments were performed based on VTM-7.0 software [3] with VTM configuration files under JVET common test condition [4]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable. Three tests are conducted,

Test1: Modification of IBC merge/IBC inter candidate list construction.

Test2: Modification of ATMVP

Test3: Combination of Test1 and Test2.

Table 1. Experimental result of the proposed method (Test1: IBC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.07% | 0.18% | 165% | 100% |
| Class A2 | -0.79% | -1.01% | -0.95% | 173% | 100% |
| Class B | -0.79% | -0.65% | -0.88% | 225% | 100% |
| Class C | -0.30% | -0.24% | -0.36% | 225% | 100% |
| Class E | -1.28% | -1.31% | -1.25% | 231% | 100% |
| Overall | -0.64% | -0.63% | -0.66% | 206% | 100% |
| Class D | -0.62% | -0.66% | -0.58% | 205% | 99% |
| Class F | 0.00% | -0.04% | 0.14% | 122% | 100% |
| Class TGM | -0.06% | -0.02% | 0.00% | 98% | 100% |
|  | Random Access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.01% | 0.25% | 107% | 100% |
| Class A2 | -0.10% | -0.12% | -0.07% | 108% | 100% |
| Class B | -0.10% | -0.19% | -0.36% | 115% | 100% |
| Class C | 0.21% | 0.04% | 0.11% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.07% | -0.05% | 111% | 100% |
| Class D | 0.11% | -0.10% | -0.13% | 114% | 100% |
| Class F | -0.03% | -0.06% | -0.09% | 103% | 100% |
| Class TGM | 0.05% | 0.06% | 0.08% | 98% | 100% |

|  | Low DelayB Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.13% | 0.06% | 115% | 100% |
| Class C | 0.09% | -0.32% | 0.16% | 120% | 100% |
| Class E | -0.06% | 0.55% | 1.14% | 109% | 100% |
| Overall | 0.02% | -0.03% | 0.37% | 115% | 100% |
| Class D | 0.02% | 0.20% | -0.09% | 122% | 100% |
| Class F | -0.11% | -0.38% | 0.26% | 118% | 100% |
| Class TGM | -0.05% | -0.12% | -0.11% | 101% | 100% |

Table 2. Experimental result of the proposed method (Test2: ATMVP)

|  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V |  | EncT | DecT |
| Class A1 | 0.03% |  | 0.06% |  | 0.04% | 121% | 100% |
| Class A2 | 0.00% |  | 0.00% |  | 0.02% | 115% | 98% |
| Class B | 0.00% |  | 0.05% |  | 0.02% | 99% | 85% |
| Class C | 0.01% |  | 0.02% |  | 0.06% | 97% | 85% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% |  | 0.03% |  | 0.03% | 105% | 90% |
| Class D | -0.02% |  | -0.03% |  | -0.04% | 85% | 71% |
| Class F | 0.01% |  | -0.04% |  | 0.00% | 98% | 81% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Y |  | U | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.15% |  |  | 0.10% | 121% | 101% |
| Class C | -0.03% | -0.14% |  |  | -0.06% | 119% | 106% |
| Class E | 0.01% | 0.16% |  |  | -0.72% | 135% | 105% |
| Overall | -0.01% | -0.07% |  |  | -0.16% | 124% | 104% |
| Class D | -0.07% | 0.78% |  |  | 0.25% | 121% | 113% |
| Class F | 0.04% | -0.37% |  |  | -0.12% | 118% | 98% |

Table 3. Experimental result of the proposed method (Test3: IBC + ATMVP)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | -0.01% | 0.17% | 108% | 100% |
| Class A2 | -0.11% | -0.04% | -0.05% | 109% | 100% |
| Class B | -0.10% | -0.15% | -0.28% | 109% | 100% |
| Class C | 0.25% | 0.09% | 0.07% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.03% | -0.05% | 110% | 100% |
| Class D | 0.12% | -0.09% | -0.11% | 114% | 100% |
| Class F | -0.02% | -0.09% | -0.09% | 99% | 100% |
| Class TGM | 0.03% | 0.03% | 0.10% | 99% | 100% |

|  | Low DelayB Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.08% | -0.34% | 115% | 100% |
| Class C | 0.07% | -0.13% | -0.02% | 119% | 100% |
| Class E | -0.11% | 0.25% | 0.78% | 108% | 100% |
| Overall | -0.02% | -0.01% | 0.05% | 115% | 100% |
| Class D | 0.05% | -0.11% | 0.15% | 122% | 100% |
| Class F | 0.02% | -0.27% | 0.53% | 100% | 100% |
| Class TGM | -0.10% | -0.08% | -0.14% | 101% | 100% |

Conclusion

In this proposal, modification of IBC and ATMVP candidate list construction was proposed. With the proposed modifications, IBC/ATMVP candidate derivation processes can be better aligned with spatial merge candidate derivation process. Simulation results for all tests shows minor performance change on average compared to VTM 7.0 anchor, it is recommended to adopt the proposed methods to the next version of VVC working draft.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019

- Z. Wang, Y. Yan, J. Luo (Alibaba), “Non-CE4: Construction of spatial merge candidates”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0325, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).