| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0328 |
| --- | --- |

| Title: | On LFNST signalling and transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

The VVC WD7 text includes conditioning of the LFNST index, which applies across all colour channels of a CU, on the luma transform skip flag indicating application of a transform. This contribution proposes to extend the conditioning to the chroma transform skip flags, including in chroma branch of dual-tree coding trees.

# Introduction

VVC WD7 [1] includes support for transform skip of chroma transform blocks (TBs). When a TB is skipped, there is no opportunity to apply LFNST. The current design conditions signalling of lfnst_idx only on the luma transform skip flag.

When a single coding tree is in use, it is still possible to apply LFNST to chroma TBs when the luma TB is transform skipped. Additionally, in the chroma branch of a dual-tree coding tree, signalling of lfnst_idx should be conditioned on the chroma transform skip flags. Otherwise it is possible to signal application of LFNST to CUs that include only transform skipped TBs.

# Proposal

This contribution proposes to add additional condition checks on signalling of lfnst_idx such that it is only signalled when at least one TB in the CU is subject to transformation. Here, the CU includes a single tree CU having TBs for luma and chroma and a chroma branch CU, having TBs for the chroma channels (in addition to a luma TB of a CU in a luma branch, as currently handled in VVC WD7). As a consequence, lfnst_idx is inferred to be zero in all cases where it is not possible to apply LFNST to a CU.

# Coding performance measurement

Coding performance results are shown in Table 1 under JVET SDR common test conditions (CTC) [2].

Table 1 – CTC coding result.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.04% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | -0.03% | 0.02% | 101% | 99% |
| Class E | 0.00% | 0.00% | -0.02% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.01% | -0.04% | 101% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution adds condition checks on signalling of lfnst_idx to prevent signalling application of LFNST to CUs for which all TBs are transform skipped, considering CUs belonging to single coding trees and chroma branches of separate coding trees.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October, 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March, 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

#### 7.3.9.5   Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| ….. |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| transformUsed = treeType = = SINGLE_TREE ? transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) : treeType = = DUAL_TREE_LUMA ? transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 : transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transformUsedtransform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |