| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0339 |
| --- | --- |

| Title: | CE4-related: Adjustment of the distance on the GEO mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the previous meeting, GEO mode was proposed and common base code for CE was provided. In the GEO common base, a CU is split to two partitions by angle index and distance index. However, there are some irregular policies in block splitting. Firstly, the direction of the movement of the edge may be different according to the mode and block shape, even though same distance index is being considered. Secondly, some modes have narrow interval between edges and some other modes have wide interval between edges.

In this contribution, TEST1 and TEST2 are proposed to adjust the direction of the movement of edges and edge position with regular policies for each mode. In the both tests, angles with narrow interval are adjusted to have wide interval. In the TEST1, the direction of the movement of the edge is adjusted to move clock-wise, while in the TEST2, the direction of the movement of the edge is adjusted to move count-clock-wise. The proposed methods are applied to the storing process of the motion for GEO with same manner.

The experimental results of the TEST1 reportedly show -0.01%, 0.02% BD-rate changes with negligible changes in encoding and decoding run-time compared to VTM7.0 common base as anchor in RA, LB configurations. And, the experimental results of the TEST2 reportedly show -0.01%, -0.01% BD-rate changes with negligible changes in encoding and decoding run-time compared to VTM7.0 common base as anchor in RA, LB configurations.

Introduction

Figure 1. Distance (weightIdx) between edge and each sample position (x, y)

In the common base for GEO mode, “wedge_partition_idx” is parsed to represent the angle and distance. In order to blend the edge area, which is partitioned by the angle index and distance index, distance (i. e., weightIdx) between edge and each sample position (x, y) in a CU is calculated as follows:

weightIdx = (((x * subW + offsetX)<<1) + 1)*Dis[displacementX]

+ (((y * subH + offsetY )<<1) + 1))*Dis[displacementY] − rho.

,where displacementX is set to angleIdx and displacementY is set to (displacementX + 6)%24. The look-up table of the Dis is defined as Table 1 and rho is set to the (Dis[displacementX]<< 8) + (Dis[displacementY] << 8) using the Table 1. subW and subH are both set to 1 when cIdx is equal to 0, and subW and subH are set equal to SubWidthC and SubHeightC, respectively when cIdx is not equal to 0.

Table 1. Look-up table Dis for derivation of wedgemetric partitioning distance.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 8 | 8 | 8 | 8 | 4 | 2 | 0 | −2 | −4 | −8 | −8 | −8 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | −8 | −8 | −8 | −8 | −4 | −2 | 0 | 2 | 4 | 8 | 8 | 8 |

In the equation, offsetX and offsetY are derived as follows.

If one of the following conditons is true, variable shiftHor is set equal to 0: angleIdx % 12 is equal to 6 angleIdx % 12 is not equal to 0 and hwRatio ≥ 1 Otherwise, shiftHor is set equal to 1. If shiftHor is equal to 0, offsetX and offsetY are derived as follows: offsetX = ( 256 − nW ) >> 1 offsetY = ( 256 − nH ) >> 1 + angleIdx < 12 ? (distanceIdx * nH) >> 3 : −((distanceIdx * nH) >> 3) Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows: offsetX = ( 256 − nW ) >> 1 + angleIdx < 12 ? (distanceIdx * nW) >> 3 : −((distanceIdx * nW) >> 3) offsetY = ( 256 − nH ) >> 1

From the distance between edge and each sample position, the weight value of the each sample position for blending, sampleWeight is derived as follows using the Table 2.

sampleWeight = weightIdx <= 0 ? WedgeFilter[weightIdxAbs] : 8 − WedgeFilter[weightIdxAbs]

Table 2. Filter weight look-up table WedgeFilter for derivation of wedge partitioning filter weights.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WedgeFilter[idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| WedgeFilter[idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

Problem statement

In above weightIdx equation, offsetX and offsetY are derived considering the distance index. And distance index is affected to offsetX or offsetY according to the variable “shiftHor” which is decided from angle index and the shape of the block as Table 3.

Table 3 Variable "shiftHor" in the GEO common base

| Angle | GEO Common base |  |
| --- | --- | --- |
|  | hwRatio>=1 | hwRatio < 1 |
| 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 | 1 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 1 1 1 0 1 1 1 1 1 |

Problem #1.

Figure 2 shows the example of the movement of the edge. The direction of the edge movement of each modes are different according to the mode and block shape, i.e., the direction of the movement of the edge by the distance index is decided by the irregular policy. If hwRatio is greater than or equal to 1, edge moves clock-wise except angle index 0 and 12. Otherwise, if hwRatio is less than 1, edge moves counter-clock-wise except angle index 6 and 18.

Figure 2 Examples of the direction of the movement of the edge for (a) hwRatio >= 1 and (b) hwRatio < 1

The variable “shiftHor” is decided by the block shape. As a example, if angle index is 1 and hwRatio is greater than or equal to 1, “shiftHor” is set to 0. In that case, edge position is shifted by offsetY as Figure 3(a). Otherwise, if angle index is 1 and hwRatio is less than 1, “shiftHor” is set to 1. In that case, edge position is shifted by offsetX as Figure 3(b). Therefore, the edge position is decided differently according to the block shape, even though same distance index is being considered.

Figure 3 Examples of the edge position for (a) hwRatio >= 1 and (b) hwRatio < 1 when angle index equal to 1.

Problem #2

Some modes have narrow interval between edges and some other modes have wide interval between edges. Figure 4 shows the example of the edge position which is derived with offsetY. Since the both cases, angleIdx 1 and 5 has same “shiftHor” value, offsetY is derived considering (distanceIdx * nH) >> 3. It is show that the interval between edges at Figure 4(a) is narrow and the interval between edges at Figure 4 (b) is wide.

Figure 4 Examples of the edge position for (a) angleIdx equal to 1 and (b) angleIdx equal to 5.

Proposed method

In the above problems, edge positioning dependent to block shape causes the irregular block partitioning in design. Therefore, block shape condition is removed and adjusted to have same rule for all mode. Also, In the GEO common base, angles close to vertical edge has narrow and angles close to horizontal edge has wide. To fix the irregular partitioning policies, angles with narrow interval are adjusted to have wide interval. Figure 5(a) and Figure 6(a) shows the example of the angle close to vertical edge to have wide interval as Figure 5(b) and Figure 6(b), respectively.

Figure 5 Example of the adjusted angles (angleIdx == 1) with clock-wise movement

Figure 6 Example of the adjusted angles (angleIdx == 1) with count-clock-wise movement

For the abovementioned fix, simple modification has applied as follows. Edge of the block is moved to x-axis when angleIdx is { 0, 1, 2, 10, 11, 12, 13, 14, 22, 23 } and edges is moved to y-axis when anglIdx is { 3, 4, 5, 6, 7, 8, 9, 15, 16, 17, 18, 19, 20, 21 }. Also, the direction of the movement of the edge is adjusted to move clock-wise / count-clock-wise. The Proposed method makes that all modes have same rule in the direction of the edge.

In the TEST1, angles with narrow interval are adjusted to have wide interval. And, the direction of the movement of the edge is adjusted to move clock-wise shown as Figure 5 by “angleIdx % 18 >= 6” condition of offsetX. The changes in spec text is as follows:

If one of the following conditons is true, variable shiftHor is set equal to 01: angleIdx % 12 is equal to 6less than 3. angleIdx % 12 is not equal to 0 and hwRatio ≥ 1greater than 9 Otherwise, shiftHor is set equal to 10. If shiftHor is equal to 0, offsetX and offsetY are derived as follows: offsetX = ( 256 − nW ) >> 1 offsetY = ( 256 − nH ) >> 1 + angleIdx < 12 ? (distanceIdx * nH) >> 3 : −((distanceIdx * nH) >> 3) Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows: offsetX = ( 256 − nW ) >> 1 + angleIdx < 12 angleIdx % 18 >= 6 ? (distanceIdx * nW) >> 3 : −((distanceIdx * nW) >> 3) offsetY = ( 256 − nH ) >> 1

In the TEST2, angles with narrow interval are adjusted to have wide interval. And, the direction of the movement of the edge is adjusted to move count-clock-wise shown as Figure 6 by “angleIdx >= 12” and “angleIdx % 18 < 6” condition for offsetY and offsetX, respectively. The changes in spec text is as follows:

If one of the following conditons is true, variable shiftHor is set equal to 01: angleIdx % 12 is equal to 6less than 3. angleIdx % 12 is not equal to 0 and hwRatio ≥ 1greater than 9 Otherwise, shiftHor is set equal to 10. If shiftHor is equal to 0, offsetX and offsetY are derived as follows: offsetX = ( 256 − nW ) >> 1 offsetY = ( 256 − nH ) >> 1 + angleIdx <>= 12 ? (distanceIdx * nH) >> 3 : −((distanceIdx * nH) >> 3) Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows: offsetX = ( 256 − nW ) >> 1 + angleIdx < 12 angleIdx % 18 < 6 ? (distanceIdx * nW) >> 3 : −((distanceIdx * nW) >> 3) offsetY = ( 256 − nH ) >> 1

The proposed methods are applied to the storing process of the motion for GEO with same manner. Also, the variable “shiftHor” is decided as following Table 4 without considering the block shape.

Table 4 Modified variable "shiftHor"

| Angle | VTM7.0 Common base |  | Proposed method |
| --- | --- | --- | --- |
|  | hwRatio>=1 | hwRatio < 1 | - |
| 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 | 1 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 1 1 1 0 1 1 1 1 1 | 1 1 1 0 0 0 0 0 0 0 1 1 1 1 1 0 0 0 0 0 0 0 1 1 |

Experimental results

The proposed method has been implemented on top of the VTM7.0 common base for GEO. The experimental results of the TEST1 reportedly show -0.01%, 0.02 BD-rate changes with negligible changes in encoding and decoding run-time compared to VTM7.0 common base as anchor in RA, LB configurations. And, the experimental results of the TEST2 reportedly show -0.01%, -0.01 BD-rate changes with negligible changes in encoding and decoding run-time compared to VTM7.0 common base as anchor in RA, LB configurations.

TEST1:

|  | Over VTM 7.0 Common base |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.02% | 0.01% | -0.07% | 101% | 100% |  |  |  |  |  |
| Class B | -0.02% | -0.02% | 0.00% | 100% | 101% | -0.02% | 0.17% | 0.32% | 100% | 100% |
| Class C | -0.04% | 0.04% | 0.01% | 100% | 100% | -0.01% | 0.07% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  | 0.15% | 0.22% | 0.80% | 100% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 100% | 0.02% | 0.15% | 0.30% | 100% | 100% |
| Class D | -0.03% | -0.08% | 0.07% | 100% | 99% | -0.08% | -0.33% | 0.17% | 100% | 97% |
| Class F | 0.02% | 0.07% | 0.03% | 100% | 100% | -0.03% | -0.15% | 0.14% | 100% | 98% |

TEST2:

|  | Over VTM 7.0 Common base |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | -0.03% | 101% | 100% |  |  |  |  |  |
| Class A2 | 0.01% | 0.08% | -0.01% | 101% | 100% |  |  |  |  |  |
| Class B | -0.02% | -0.03% | 0.04% | 100% | 100% | -0.01% | 0.06% | 0.34% | 101% | 101% |
| Class C | -0.02% | 0.02% | -0.02% | 100% | 100% | 0.01% | 0.04% | 0.08% | 100% | 100% |
| Class E |  |  |  |  |  | -0.04% | -0.19% | 0.06% | 100% | 100% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 100% | -0.01% | -0.01% | 0.19% | 100% | 100% |
| Class D |  |  |  |  |  | -0.02% | -0.57% | 0.19% | 99% | 100% |
| Class F | 0.02% | 0.08% | 0.06% | 100% | 100% | -0.01% | 0.02% | 0.02% | 100% | 99% |

Conclusion

In current GEO common base code, there are irregular policies in block splitting. Firstly, the direction of the movement of the edge by the distance index is decided by the irregular policy, even though having same distance index. Secondly, some modes have narrow interval between edges and some other modes have wide interval between edges. In this contribution, the adjusting methods to have regular policies in each mode are proposed. The experimental results of TEST1 and TEST2 reportedly show negligible changes in BD-rate and encoding / decoding run-time. It is recommended to consider this method in the next VTM.

Reference

- Han Gao, et al., “CE4 Common Base: Geometric inter prediction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0079, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).