| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0340 |
| --- | --- |

| Title: | Non-CE4: Cleanup of the MMVD offset derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, cleanup of the MMVD offset derivation process is proposed. When the current picture refers short-term reference pictures for both directions and distance between current picture and reference picture of each direction is different, mmvd offset is scaled. However, since the maximum value of the scaled MVD cannot exceed the range of the MVD, clipping operation is not necessary. The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0 as anchor in CTC[2].

Proposed method

In this contribution, cleanup of the MMVD offset derivation process is proposed. In the VVC 7.0 [1], MMVD offset is scaled when current picture refers short-term reference pictures for both directions and the distance between current picture and reference picture of the each direction is different. The MMVD distance is set to maximum 512 by the following Table 1. And, the direction of the offset is assigned by following Table 2.

Table 1 Specification of MmvdDistance[ x0 ][ y0 ] based on mmvd_distance_idx[ x0 ][ y0 ]

| mmvd_distance_idx[ x0 ][ y0 ] | MmvdDistance[ x0 ][ y0 ] |  |
| --- | --- | --- |
|  | pic_fpel_mmvd_enabled_flag = = 0 | pic_fpel_mmvd_enabled_flag = = 1 |
| 0 | 1 | 4 |
| 1 | 2 | 8 |
| 2 | 4 | 16 |
| 3 | 8 | 32 |
| 4 | 16 | 64 |
| 5 | 32 | 128 |
| 6 | 64 | 256 |
| 7 | 128 | 512 |

Table 2 Specification of MmvdSign[ x0 ][ y0 ] based on mmvd_direction_idx[ x0 ][ y0 ]

| mmvd_direction_idx[ x0 ][ y0 ] | MmvdSign[ x0 ][ y0 ][0] | MmvdSign[ x0 ][ y0 ][1] |
| --- | --- | --- |
| 0 | +1 | 0 |
| 1 | −1 | 0 |
| 2 | 0 | +1 |
| 3 | 0 | −1 |

The MMVD offset, MmvdOffset is derived as follows and finally, the MmvdOffset is set to range of [-2048, 2048].

MmvdOffset[ x0 ][ y0 ][ 0 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][0]

MmvdOffset[ x0 ][ y0 ][ 1 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][1]

Also, in the scaling process, MMVD offset is scaled by distScaleFactor, which is clipped within [-4096, 4095]. If the distance between current picture and L0 reference picture (currPocDiffL0) is greater than or equal to distance between current picture and L1 reference picture (currPocDiffL1), the motion vector difference of the L0, mMvdL0 is set to MmvdOffset and motion vector difference of the L1 is scaled as follows.

mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] mMvdL1[ 0 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL0[ 0 ] + 128 − (distScaleFactor * mMvdL0[ 0 ] >= 0) ) >> 8 ) mMvdL1[ 1 ] = Clip3( −217, 217 − 1, , (distScaleFactor * mMvdL0[ 1 ] + 128 − (distScaleFactor * mMvdL0[ 1 ] >= 0) ) >> 8 ) )

Otherwise, if currPocDiffL0 is less than currPocDiffL1, the motion vector difference of the L1, mMvdL1 is set to MmvdOffset and motion vector difference of the L0 is scaled as follows.

mMvdL1[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] mMvdL1[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] mMvdL0[ 0 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL1[ 0 ] + 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) ) >> 8 ) mMvdL0[ 1 ] = Clip3( −217, 217 − 1, , (distScaleFactor * mMvdL1[ 1 ] + 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) ) >> 8 ) )

According to the above derivation process, scaled MVD is always in range of [ −215, 215 ]. Therefore, in this proposal, unnecessary clipping operation of the MMVD mode is removed.

Proposed text changes

In the VVC7.0 spec text [1], modified texts are marked as stricken-red-font text.

#### 8.5.2.7 Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1.

The variable currPic specifies the current picture.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] )	(564)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] )	(565)

- If currPocDiffL0 is equal to currPocDiffL1, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(566)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(567)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(568)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(569)

- Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(570)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(571)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(572)

tb = Clip3( −128, 127, currPocDiffL1 )		(573)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(574)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(575)

mMvdL1[ 0 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL0[ 0 ] + 	(576)
	128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) )  >>  8 )

mMvdL1[ 1 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL0[ 1 ] + 	(577)
	128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) )  >>  8 )

    - Otherwise, the following applies:

mMvdL1[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL0[ 0 ]  :  −mMvdL0[ 0 ]	(578)

mMvdL1[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL0[ 1 ]  :  −mMvdL0[ 1 ]	(579)

- Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies:

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(580)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(581)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(582)

tb = Clip3( −128, 127, currPocDiffL0 )		(583)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(584)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(585)

mMvdL0[ 0 ] = Clip3( −217, 217 − 1,  (distScaleFactor * mMvdL1[ 0 ] +	(586)
	 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) )  >>  8 )

mMvdL0[ 1 ] = Clip3( −217, 217 − 1, , (distScaleFactor * mMvdL1[ 1 ] +	(587)
	 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) )  >>  8 ) )

    - Otherwise, the following applies:

mMvdL0[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL1[ 0 ]  :  −mMvdL1[ 0 ]	(588)

mMvdL0[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL1[ 1 ]  :  −mMvdL1[ 1 ]	(589)

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 0 ]  :  0	(590)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 1 ]  :  0	(591)

Conclusion

In this contribution, cleanup of the MMVD offset derivation process is proposed. In the VVC7.0, MMVD offset is scaled when current picture refers short-term reference pictures for both directions and the distance between current picture and reference picture of the each direction is different. However, since the maximum value of the scaled MVD cannot exceed the range of the MVD, clipping operation is not necessary. The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0 as anchor in CTC. It is recommended to consider this method in the next VTM.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 7)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P2001(JVET-P2001-vE), 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).