| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0347 |
| --- | --- |

| Title: | Non-CE7: Fixed rice parameters in transform residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Chen, F. Le Léannec, F. Galpin, T. Poirier | Tel: Email: | +33(0)256856325 ya.chen@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to replace per-sample adaptive codeword derivation based on the levels of the bottom and right neighboring residual samples with a fixed codeword in transform residual coding. It is proposed to use a fixed rice parameter=0 for the syntax element abs_remainder, and a fixed rice parameter=1 for the syntax element dec_abs_level.

The results of using fixed rice parameters show BD-rate change of 0.08% AI, 0.05% RA and -0.02% LB over VTM-7.0 for common test conditions.

# Introduction

In transform residual coding, for each coefficient, the remaining absolute levels abs_remainder and the intermediate values dec_abs_level are adaptively binarized using the rice parameters derived depending on the levels of the bottom and right neighboring residual samples.

The unified (same) rice parameter (RiceParam) derivation is used for Pass 2 and Pass 3 in residual coding for transform coefficients. The only difference is that baseLevel is set to 4 and 0 for Pass 2 and Pass 3, respectively. Rice parameter is determined not only based on the sum of absolute levels (sumAbs) of the five neighboring transform coefficients in a local neighbor template as shown in Figure 1, but the corresponding base level is also taken into consideration as follow:

RiceParam = RiceParamTable[ max(min( 31,sumAbs – 5 * baseLevel),0) ].

Figure 1: Local neighbor template used for rice parameter derivation. The black square specifies the current scan position and the blue squares represent the local neighborhood used.

Given the variable SumAbs, the Rice parameter RiceParam is derived as specified in Table 1.

Table 1. Specification of RiceParam based on SumAbs

| SumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| SumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| RiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

# Proposed method

In this contribution, it is proposed to use a fixed binary codeword (cRiceParam equal to 0) for the syntax element abs_remainder, and another fixed binary codeword (cRiceParam equal to 1) for the syntax element dec_abs_level in transform residual coding. The codeword binarization process is illustrated in the Table 2 below. With the proposed simplification, the per-sample codeword determination can be removed.

Table 2. Proposed syntax modification

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] #### Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1494) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 124. When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows: ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam (1495) Table 124 Specification of cRiceParam based on locSumAbs locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived as follows: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the Rice parameter cRiceParam is set equal to 1. - Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. - Otherwise, the rice parameter cRiceParam is set to 0. The variable cMax is derived from cRiceParam as: cMax = 6 << cRiceParam (1503) The binarization of the syntax element abs_remainder[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string. For the derivation of the prefix bin string, the following applies: - The prefix value of abs_remainder[ n ], prefixVal, is derived as follows: prefixVal = Min( cMax, abs_remainder[ n ] ) (1504) - The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs. When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows: - The suffix value of abs_remainder[ n ], suffixVal, is derived as follows: suffixVal = abs_remainder[ n ] − cMax (1505) - The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1, variable cRiceParam, variable log2TransformRange set equal to 15 and variable maxPreExtLen set equal to 11 as input. #### 9.3.3.12 Binarization process for dec_abs_level[ ] Input to this process is a request for a binarization of the syntax element dec_abs_level[ n ], the colour component cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for dec_abs_level[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 0, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. The rice parameter cRiceParam is set to 1. The variable ZeroPos[ n ] is derived as follows: ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam (1495) The variable cMax is derived from cRiceParam as: cMax = 6 << cRiceParam (1506) The binarization of dec_abs_level[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string. For the derivation of the prefix bin string, the following applies: - The prefix value of dec_abs_level[ n ], prefixVal, is derived as follows: prefixVal = Min( cMax, dec_abs_level[ n ] ) (1507) - The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs. When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows: - The suffix value of dec_abs_level[ n ], suffixVal, is derived as follows: suffixVal = dec_abs_level[ n ] − cMax (1508) - The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1, variable cRiceParam, variable log2TransformRange set equal to 15 and variable maxPreExtLen set equal to 11 as input.

# Experimental results

The proposed method was implemented on top of VTM-7.0 and tested under CE7 CTC.

The following tables show the performance obtained, compared to VTM-7.0. Standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are both tested and results are shown in Table 3 and Table 4, respectively.

Please note that the run time results may not be accurate. The detailed simulation results can be found in the attached spreadsheets.

Table 3: performances of the proposed method compared to VTM-7.0 over standard QPs 22, 27, 32, and 37.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.05% | 0.19% | 105% | 102% |
| Class A2 | 0.09% | 0.16% | 0.10% | 110% | 106% |
| Class B | 0.07% | 0.07% | -0.05% | 103% | 100% |
| Class C | 0.09% | 0.06% | 0.05% | 108% | 103% |
| Class E | 0.06% | -0.03% | 0.03% | 106% | 103% |
| Overall | 0.08% | 0.05% | 0.05% | 106% | 102% |
| Class D | 0.13% | 0.15% | 0.18% | 104% | 109% |
| Class F | 0.10% | 0.01% | 0.17% | 109% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.09% | 0.09% | 101% | 107% |
| Class A2 | 0.01% | 0.07% | 0.19% | 100% | 102% |
| Class B | 0.05% | 0.16% | 0.11% | 99% | 104% |
| Class C | 0.06% | -0.07% | 0.08% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.03% | 0.11% | 100% | 104% |
| Class D | 0.09% | -0.17% | 0.24% | 99% | 101% |
| Class F | 0.11% | 0.11% | -0.01% | 100% | 105% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.18% | 0.12% | 98% | 102% |
| Class C | 0.02% | -0.38% | -0.33% | 99% | 96% |
| Class E | -0.08% | 0.64% | 0.09% | 99% | 96% |
| Overall | -0.02% | -0.04% | -0.04% | 99% | 98% |
| Class D | 0.00% | 0.34% | -0.37% | 99% | 99% |
| Class F | 0.00% | -0.65% | 1.07% | 98% | 97% |

Table 4: performances of the proposed method compared to VTM-7.0 over low QPs 2, 7, 12, and 17.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.53% | 0.70% | 0.68% | 96% | 100% |
| Class A2 | 1.88% | 2.19% | 2.21% | 97% | 104% |
| Class B | 1.65% | 1.75% | 1.81% | 95% | 104% |
| Class C | 2.80% | 3.00% | 3.13% | 93% | 106% |
| Class E | 1.35% | 1.51% | 1.57% | 96% | 103% |
| Overall | 1.71% | 1.89% | 1.94% | 95% | 103% |
| Class D | 3.17% | 3.23% | 3.26% | 92% | 106% |
| Class F | 1.03% | 1.26% | 1.34% | 97% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.55% | 0.64% | 0.66% | 96% | 98% |
| Class C | 1.16% | 1.47% | 1.50% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 1.35% | 1.66% | 1.54% | 97% | 100% |
| Class F | 0.52% | 1.01% | 1.06% | 98% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.43% | 0.50% | 0.50% | 97% | 98% |
| Class C | 0.98% | 1.27% | 1.32% | 96% | 100% |
| Class E | 0.13% | 0.07% | 0.07% | 98% | 98% |
| Overall | 0.54% | 0.65% | 0.67% | 97% | 99% |
| Class D | 1.02% | 1.23% | 1.15% | 98% | 100% |
| Class F | 0.58% | 0.66% | 0.53% | 98% | 100% |

# Supplemental results with JVET-P0300

CABAC throughput issue will be a significant problem for the use cases of high bitrate and low delay conditions. In order to address this problem, “High throughput CABAC mode for VVC” was proposed in JVET-P0300. However, even with the proposed high throughput CABAC mode, determination of the Rice parameters could also be another throughput issue. Therefore, the proposed method was also implemented on top of VTM-7.0 with JVET-P0300.

The following tables show the performance obtained with JVET-P0300, compared to VTM-7.0. Standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are both tested and results are shown in Table 5 and Table 6, respectively. The losses are mostly due to JVET-P0300, which are 3.95% AI and 4.77% RA BD rate loss over VTM-7.0.

Please note that the run time results may not be accurate. The detailed simulation results can be found in the attached spreadsheets.

Table 5: performances of the proposed method with JVET-P0300 compared to VTM-7.0 over standard QPs 22, 27, 32, and 37.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 7.09% | 4.34% | 4.64% | 107% | 96% |
| Class A2 | 5.77% | 3.88% | 6.68% | 95% | 94% |
| Class B | 3.29% | 3.72% | 3.13% | 96% | 94% |
| Class C | 2.57% | 3.56% | 3.67% | 91% | 93% |
| Class E | 2.44% | 3.69% | 3.46% | 92% | 96% |
| Overall | 4.04% | 3.81% | 4.15% | 96% | 94% |
| Class D | 2.21% | 3.10% | 3.35% | 90% | 93% |
| Class F | 6.13% | 7.07% | 6.80% | 93% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 7.19% | 10.42% | 8.81% | 99% | 111% |
| Class A2 | 4.42% | 5.58% | 7.10% | 95% | 111% |
| Class B | 4.58% | 5.51% | 3.64% | 95% | 110% |
| Class C | 3.69% | 5.40% | 3.89% | 94% | 100% |
| Class E |  |  |  |  |  |
| Overall | 4.83% | 6.48% | 5.43% | 96% | 108% |
| Class D | 3.40% | 4.20% | 3.95% | 95% | 93% |
| Class F | 7.09% | 8.82% | 8.44% | 96% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 4.52% | 5.68% | -0.23% | 94% | 102% |
| Class C | 3.08% | 5.45% | 1.17% | 94% | 94% |
| Class E | 2.59% | 2.39% | 0.50% | 96% | 95% |
| Overall | 3.56% | 4.78% | 0.42% | 95% | 97% |
| Class D | 3.20% | 3.40% | -0.17% | 94% | 97% |
| Class F | 6.52% | 8.77% | 8.39% | 95% | 98% |

Table 6: performances of the proposed method with JVET-P0300 compared to VTM-7.0 over low QPs 2, 7, 12, and 17.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 9.03% | 7.37% | 8.17% | 103% | 90% |
| Class A2 | 7.54% | 7.95% | 8.11% | 92% | 87% |
| Class B | 8.36% | 8.51% | 9.63% | 97% | 95% |
| Class C | 6.95% | 7.06% | 7.68% | 88% | 97% |
| Class E | 6.91% | 7.28% | 9.08% | 91% | 95% |
| Overall | 7.78% | 7.70% | 8.61% | 94% | 93% |
| Class D | 6.27% | 6.09% | 6.56% | 87% | 98% |
| Class F | 8.32% | 6.71% | 7.23% | 94% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 8.67% | 11.65% | 10.63% | 102% | 99% |
| Class C | 7.12% | 10.49% | 8.46% | 90% | 91% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 6.35% | 10.02% | 7.67% | 91% | 94% |
| Class F | 10.27% | 9.36% | 8.33% | 94% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.36% | 10.71% | 10.15% | 104% | 91% |
| Class C | 7.05% | 10.26% | 8.62% | 91% | 90% |
| Class E | 6.91% | 8.00% | 7.24% | 94% | 85% |
| Overall | 7.56% | 9.88% | 8.91% | 97% | 89% |
| Class D | 5.88% | 9.26% | 7.33% | 92% | 94% |
| Class F | 7.85% | 10.09% | 8.18% | 94% | 97% |

For reference, JVET-P0300 on top of VTM-7.0 is tested and results under Standard QPs (22, 27, 32, 37) are shown in Table 7.

Table 7: performances of JVET-P0300 compared to VTM-7.0 over standard QPs 22, 27, 32, and 37.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.99% | 4.22% | 4.55% | 109% | 92% |
| Class A2 | 5.68% | 3.73% | 6.48% | 95% | 90% |
| Class B | 3.22% | 3.60% | 3.04% | 132% | 106% |
| Class C | 2.48% | 3.42% | 3.55% | 128% | 104% |
| Class E | 2.38% | 3.57% | 3.39% | 132% | 106% |
| Overall | 3.95% | 3.68% | 4.04% | 120% | 100% |
| Class D | 2.05% | 3.05% | 3.23% | 129% | 105% |
| Class F | 6.02% | 7.04% | 6.79% | 120% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 7.10% | 10.22% | 8.65% | 97% | 122% |
| Class A2 | 4.35% | 5.61% | 7.00% | 98% | 116% |
| Class B | 4.57% | 5.53% | 3.63% | 97% | 117% |
| Class C | 3.60% | 5.35% | 3.79% | 113% | 114% |
| Class E |  |  |  |  |  |
| Overall | 4.77% | 6.44% | 5.35% | 101% | 117% |
| Class D | 3.29% | 4.24% | 3.56% | 118% | 109% |
| Class F | 6.96% | 8.63% | 8.28% | 117% | 116% |

# Conclusion

This contribution proposes to replace per-sample adaptive codeword derivation with a fixed codeword. It is proposed to use a fixed rice parameter=0 for the syntax element abs_remainder, and a fixed rice parameter=1 for the syntax element dec_abs_level in transform residual coding. With the proposed simplification, the per-sample codeword determination can be removed with few losses.

Furthermore, this contribution can be implemented with the “High throughput CABAC mode for VVC” proposed in JVET-P0300, improving the throughput for the use cases of high bitrate and low delay conditions.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).