| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0352 |
| --- | --- |

| Title: | AHG9/AHG12: On subpicture boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the VVC draft specification, in-loop filter could be applicable on the subpicture boundary even when the subpicture is treated as a picture. If subpictures from different bitstreams are merged, undesirable mismatch could occur at the subpicture boundary. This error could be propagated into the following subpictures. Therefore, this contribution proposes three methods to prevent error propagation.

- loop_filter_across_subpic_enabled_flag[ i ] signaling depend on subpic_treated_as_pic_flag[ i ]

- Restrict pixel modification by the deblocking operation of following subpictures

- Restrict the motion compensation range within the subpicture except subpicture boundary area where reconstructed pixels could be modified by in-loop filter.

Introduction

The subpicture is proposed to support bitstream merging [1]. In current VVC specification, subpicture information is transmitted in SPS and also in-loop filtering is controlled by the transmitted syntax at the subpicture boundary. The flag which controls in-loop filter on subpicture boundary is signaled independently even when the subpicture is treated as a picture. Hence it is possible that the subpicture treated as a picture is extracted from the bitstream and merged with different bitstream. This case could cause reconstructed pixel mismatch at the subpicture boundary when the in-loop filter is enabledas described in Figure 1.

Figure 1 Mismatch caused by in-loop filtering at the subpicture boundary

Proposed method

Proposed method 1

The proposed method 1 suggest to signal loop_filter_across_subpic_enabled_flag[ i ] only when a subpicture is not treated as a picture.

| subpics_present_flag | u(1) |
| --- | --- |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if( ! subpic_treated_as_pic_flag[ i ] ) |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is set equal to 0.

Proposed method 2

The proposed method suggests to prevent pixel modification by deblock filtering operation of following subpicture at bottom and right subpicture boundary.

- 8.8.3.2 Deblocking filter process for one direction

…

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left or right boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The left boundary of the current coding block is one of the vertical virtual boundaries of the picture and VirtualBoundariesDisabledFlag is equal to 1.

- Otherwise, if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top or bottom boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The top boundary of the current coding block is one of the horizontal virtual boundaries of the picture and VirtualBoundariesDisabledFlag is equal to 1.

- Otherwise, filterEdgeFlag is set equal to 1.

        - Filtering process for a luma sample using short filters

When nDp is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1, nDp is set equal to 0

When nDq is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1, nDq is set equal to 0:

When loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index containing the sample pi and the sample pi belongs to different subpicture. the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

- 8.8.3.6.7 Filtering process for a luma sample using long filters

…

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1, the filtered sample value, qi′ is substituted by the corresponding input sample value qj with j = 0..maxFilterLengthQ − 1.

When loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index containing the sample pi and the sample pi belongs to a different subpicture. the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

- 8.8.3.6.9 Filtering process for a chroma sample

…

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1, the filtered sample value, qi′ is substituted by the corresponding input sample value qi with i = 0..maxFilterLengthQ − 1:

When loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index containing the sample pi and the sample pi belongs to a different subpicture. the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

Proposed method 3

The proposed method suggests that the motion compensation to be applied within the area, which is not affected (or modified) by loop filtering when the subpicture is treated as a picture itself.

Figure 2. Proposed motion compensation range for subpicture which is treated as a picture

The proposed motion compensation ranges is defined with considering modified pixels range from long-tap deblocking filter as Figure 3 and Figure 4.

Figure 3. worst case luma pixel modification by long-tap deblocking filter at subpicture boundary

Figure 4. worst case chroma pixel modification by long-tap deblocking filter at subpicture boundary for 4:2:0 color format

It is from subpictureLeftBoundaryPos + 7 to subpictureRightBoudnaryPos – 7 horizontally and subpictureTopBoundaryPos + 7 to subpictureBottomBoundaryPos – 3 vertically.

Figure 5. the proposed restricted motion compensation range for subpicture

The proposed syntax changes are below:

- Luma sample bilinear interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2, shift3, shift4, offset1, offset2 and offset3 are derived as follows:

shift1 = BitDepth − 6		(635)

offset1 = 1 << ( shift1 − 1 )		(636)

shift2 = 4					(637)

offset2 = 1 << ( shift2 − 1 )		(638)

shift3 = 10 − BitDepth		(639)

shift4 = BitDepth − 10		(640)

offset4 = 1 << ( shift4 − 1 )		(641)

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fbL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 28.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos+7, SubPicRightBoundaryPos-7, xIntL + i )	(642)

yInti = Clip3( SubPicTopBoundaryPos+7, SubPicBotBoundaryPos-3, yIntL + i )	(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(645)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = BitDepth  <=  10 ? (refPicLXL[ xInt0 ][ yInt0 ]  <<  shift3 ) :
					( ( refPicLXL[ xInt0 ][ yInt0 ] + offset4 )  >>  shift4 )	(646)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = (  i=01fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt0 ]  + offset1 )  >>  shift1	(647)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = (  i=01fL[ yFracL ][ i ] * refPicLXL[ xInt0 ][ yInti ]  + offset1 )  >>  shift1	(648)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..1, is derived as follows:

temp[ n ] = (  i=01fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]  + offset1 )  >>  shift1	(649)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = (  i=01fL[ yFracL ][ i ] * temp[ i ]  + offset2 )  >>  shift2	(650)

- Luma sample interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- a luma location in full-sample units ( xSbIntL, ySbIntL ) specifying the top-left sample of the bounding block for reference sample padding relative to the top-left luma sample of the reference picture,

- the luma reference sample array refPicLXL,

- the half sample interpolation filter index hpelIfIdx,

- a variable sbWidth specifying the width of the current subblock,

- a variable sbHeight specifying the height of the current subblock,

- a fixedpoint representation of the horizontal scaling factor scalingRatio[ 0 ],

- a fixedpoint representation of the vertical scaling factor scalingRatio[ 1 ],

- a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

- The variable shift1 is set equal to Min( 4, BitDepth − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepth ).

- The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The horizontal luma interpolation filter coefficients fLH[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLH[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLH[ p ] are specified in Table 29 depending on hpelIfIdx.

The vertical luma interpolation filter coefficients fLV[ p ] for each 1/16 fractional sample position p equal to yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLV[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 1 ] is greater than 28672, luma interpolation filter coefficients fLV[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, luma interpolation filter coefficients fLV[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLV[ p ] are specified in Table 29 depending on hpelIfIdx.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos+7, SubPicRightBoundaryPos-7, xIntL + i − 3 )	(955)

yInti = Clip3( SubPicTopBoundaryPos+7, SubPicBotBoundaryPos-3, yIntL + i − 3 )	(956)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(957) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(958)

The luma locations in full-sample units are further modified as follows for i = 0..7:

xInti = Clip3( xSbIntL − 3, xSbIntL + sbWidth + 4, xInti )	(959)

yInti = Clip3( ySbIntL − 3, ySbIntL + sbHeight + 4, yInti )	(960)

The predicted luma sample value predSampleLXL is derived as follows:

- If both xFracLand yFracL are equal to 0, and both scalingRatio[ 0 ] and scalingRatio[ 1 ] are less than 20481, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(961)

- Otherwise, if yFracL is equal to 0 and scalingRatio[ 1 ] is less than 20481, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fLH[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(962)

- Otherwise, if xFracL is equal to 0 and scalingRatio[ 0 ] is less than 20481, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fLV[ yFracL ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(963)

- Otherwise, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fLH[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(964)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fLV[ yFracL ][ i ] * temp[ i ]   >>  shift2	(965)

- Luma integer sample fetching process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift is set equal to Max( 2, 14 − BitDepth ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos+7, SubPicRightBoundaryPos-7, xIntL )	(966)

yInt = Clip3( SubPicTopBoundaryPos+7, SubPicBotBoundaryPos-3, yIntL )	(967)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(968)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(969)

The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt ][ yInt ] << shift3		(970)

- Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	a chroma location in full-sample units ( xSbIntC, ySbIntC ) specifying the top-left sample of the bounding block for reference sample padding relative to the top-left chroma sample of the reference picture,

- a variable sbWidth specifying the width of the current subblock,

- a variable sbHeight specifying the height of the current subblock,

–	the chroma reference sample array refPicLXC,

- a fixedpoint representation of the horizontal scaling factor scalingRatio[ 0 ],

- a fixedpoint representation of the vertical scaling factor scalingRatio[ 1 ].

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepth − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepth ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The horizontal chroma interpolation filter coefficients fCH[ p ] for each 1/32 fractional sample position p equal to xFracC are derived as follows:

- If scalingRatio[ 0 ] is greater than 28672, chroma interpolation filter coefficients fCH[ p ] are specified in Table 35.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, chroma interpolation filter coefficients fCH[ p ] are specified in Table 60.

- Otherwise, chroma interpolation filter coefficients fCH[ p ] are specified in Table 33.

The vertical chroma interpolation filter coefficients fCV[ p ] for each 1/32 fractional sample position p equal to yFracC are derived as follows:

- If scalingRatio[ 1 ] is greater than 28672, chroma interpolation filter coefficients fCV[ p ] are specified in Table 35.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, chroma interpolation filter coefficients fCV[ p ] are specified in Table 60.

- Otherwise, chroma interpolation filter coefficients fCV[ p ] are specified in Table 33.

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( (SubPicLeftBoundaryPos+7) / SubWidthC, (SubPicRightBoundaryPos-7) / SubWidthC, xIntL + i )	(971)

yInti = Clip3( (SubPicTopBoundaryPos+7) / SubHeightC, (SubPicBotBoundaryPos-3) / SubHeightC, yIntL + i )	(972)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(974)

The chroma locations in full-sample units ( xInti, yInti ) are further modified as follows for i = 0..3:

xInti = Clip3( xSbIntC − 1, xSbIntC + sbWidth + 2, xInti )	(975)

yInti = Clip3( ySbIntC − 1, ySbIntC + sbHeight + 2, yInti )	(976)

The predicted chroma sample value predSampleLXC is derived as follows:

–	If both xFracC and yFracC are equal to 0, and both scalingRatio[ 0 ] and scalingRatio[ 1 ] are less than 20481, the value of predSampleLXC is derived as follows:

predSampleLXC = refPicLXC[ xInt1 ][ yInt1 ] << shift3	(977)

–	Otherwise, if yFracC is equal to 0 and scalingRatio[ 1 ] is less than 20481, the value of predSampleLXC is derived as follows:

predSampleLXC =  i=03fCH[ xFracC ][ i ] * refPicLXC[ xInti ][ yInt1 ]   >>  shift1	(978)

–	Otherwise, if xFracC is equal to 0 and scalingRatio[ 0 ] is less than 20481, the value of predSampleLXC is derived as follows:

predSampleLXC =  i=03fCV[ yFracC ][ i ] * refPicLXC[ xInt1 ][ yInti ]   >>  shift1 	(979)

–	Otherwise, the value of predSampleLXC is derived as follows:

- The sample array temp[ n ] with n = 0..3, is derived as follows:

temp[ n ] =  i=03fCH[ xFracC ][ i ] * refPicLXC[ xInti ][ yIntn ]   >>  shift1	(980)

- The predicted chroma sample value predSampleLXC is derived as follows:

predSampleLXC =( fCV[ yFracC ][ 0 ] * temp[ 0 ] +
					 fCV[ yFracC ][ 1 ] * temp[ 1 ] +
					 fCV[ yFracC ][ 2 ] * temp[ 2 ] +	(981)
					 fCV[ yFracC ][ 3 ] * temp[ 3 ] ) >> shift2

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).