| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0353 |
| --- | --- |

| Title: | Non-CE: Harmonization adaptive color transform with BDPCM chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The Adaptive color transform and BDPCM chroma were adopted in VVC draft version 7 simultaneously without considering interaction between them. In current VVC draft, chroma intra prediction mode is implicitly determined to DM when the ACT is enabled (i.e., ACT flag is equal to 1). However, BDPCM chroma flag is still signaled when ACT is enabled. Therefore, this contribution suggests signaling BDPCM chroma flag is to be removed so that BDPCM chroma flag is inherited from the corresponding luma. The experimental result shows 0.0X% and 0.0X% BD-rate saving for camera captured 4:4:4 RGB and YUV contents and also additional coding gain entirely for the other RGB and YUV 4:4:4 contents.

Proposed method

The proposed method suggests to signal BDPCM chroma flag when current CU is coded MODE_INTRA and ACT flag is equal to 0. The chroma intra prediction mode is forced to DM and BDPCM chroma flag is inherited from BDPCM luma flag of the corresponding luma coding block when the ACT flag is transmitted as 1.

Experimental result

The experimental results show additional coding gain for entire RGB and YUV 444 sequences.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| RGB, TGM 720p | -0.03% | -0.04% | -0.02% | 100% | 99% |
| RGB, Animation | -0.01% | 0.02% | -0.04% | 100% | 99% |
| RGB, Mixed content | -0.06% | -0.04% | -0.03% | 100% | 96% |
| RGB, Camera-Captured content | -2.01% | 0.36% | 0.07% | #DIV/0! | #DIV/0! |
| YUV, TGM 1080p | -0.21% | 0.11% | 0.14% | 100% | 96% |
| YUV, TGM 720p | -0.50% | 0.41% | 0.39% | 100% | 101% |
| YUV, Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| YUV, Mixed content | -0.34% | 0.69% | 0.66% | 100% | 97% |
| YUV, Camera-Captured content | -1.59% | 0.51% | 1.01% | 99% | 100% |
| Overall (RGB) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall (YUV) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

It is suggested to include the proposed harmonization between BDPCM chroma and ACT for next VTM.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).