| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0354 |
| --- | --- |

| Title: | Non-CE: Clean-up regarding syntaxes for prediction mode decision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution suggests to clean-up conditions on syntax signaling regarding with prediction mode decision. The proposed method clean-up both software and specification. The software which is modified to the proposed syntax clean-up generates identical bitstreams compare to VTM-7.0 and also VTM-7.0 bitstreams are decodable for all of the configuration.

In the current VVC draft and also VTM SW, there is some redundancies among 4x4 block, Slice type and prediction mode decision in terms of “available prediction mode for coding unit”. For example, inter prediction is not allowed for 4x4 block or I-Slice. But this block could be defined as MODE_TYPE_ALL in current VTM and specification.

In order to clean up syntax table by removing redundant meaning of signaling condition, this contribution suggests 2 clean-up methods on determining prediction mode.

- Method 1: define 4x4 block as MODE_TYPE_INTRA and remove 4x4 condition on prediction mode decision

- Method 2: define 4x4 block or coding block in I-Slice as MODE_TYPE_INTRA and remove related redundant conditions.

The proposed clean-up methods simplify the conditions without any bitstream changes for all of the configurations and also the syntax signaling condition in VTM software could be simple and easy to read.

Proposed method

Proposed clean-up 1

The proposed method 1 suggests to remove 4x4 condition. 4x4 block only can uses MODE_PLT and MODE_INTRA. This means that current CU can be defined as MODE_TYPE_INTRA. This condition is redundant somehow.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = (cbWidth = = 4 && cbHeight = = 4) ? MODE_TYPE_INTRA : modeType |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |

Proposed clean-up 2

The 4x4 coding block or coding block in I slice could uses MODE_INTRA, MODE_PLT and MODE_IBC. Hence in this proposed method 2 it is suggested to define 4x4 and I-Slice condition as MODE_TYPE_INTRA. From the proposed syntax clean-up, the conditions which are redundant each other can be removed.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = (slice_type = = I ¦¦ (cbWidth = = 4 && cbHeight = = 4)) ? MODE_TYPE_INTRA : modeType |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |

The proposed above syntax clean-up could be rewritten as below table.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = (slice_type = = I ¦¦ (cbWidth = = 4 && cbHeight = = 4)) ? MODE_TYPE_INTRA : modeType |  |
| if(treeType != DUAL_TREE_CHROMA && (modeType != MODE_TYPE_INTRA ¦ ¦ ( sps_ibc_enabled_flag && Max(cbWidth,cbHeight) <= 64) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( modeType = = MODE_TYPE_ALL && cu_skip_flag[ x0 ][ y0 ] == 0) |  |
| pred_mode_flag | ae(v) |
| if( sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA && Max(cbWidth, cbHeight) <= 64 && (( modeType != MODE_TYPE_INTER && cuPredMode[chType][x0][y0] == MODE_INTER) ¦¦ ( modeType == MODE_TYPE_INTRA && cu_skip_flag[x0][y0] == 0) ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( sps_palette_enabled_flag && cuPredMode[chType][x0][y0] == MODE_INTRA && Max(cbWidth, cbHeight) <= 64) |  |
| pred_mode_plt_flag | ae(v) |

Experimental result

The experimental results are identical with VTM-7.0 software for both method 1 and method 2. The test are experimented under CTC and 4:4:4 color format including SCC.

Conclusion

It is suggest to adopt the proposed syntax clean-up for VVC draft version 8 and Next VTM software

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).