| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0354_r1 |
| --- | --- |

| Title: | Non-CE: Clean-up regarding syntaxes for prediction mode decision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution suggests to clean-up conditions regarding prediction modes syntaxes. The proposed clean-up condition do not change bitstream. In order to verify whether bitstream changing, the SW is also clean-up based on the suggested syntax table.

- Method 1: remove “cu_skip_flag == 0 && modeType != MODE_TYPE_INTER” condition for pred_mode_plt_flag.

- Method 2: define 4x4 block or coding block in I-Slice as MODE_TYPE_INTRA and remove related redundant conditions.

The proposed clean-up methods simplify the conditions without any bitstream changes.

In revision1, the semantic is attached.

Proposed method

Proposed clean-up 1

The proposed method 1 suggests to remove redundant condition on “pred_mode_plt_flag”. The condition cuPredMode[][] == MODE_INTRA, cu_skip_flag == 0 and modeType != MODE_TYPE_INTER are redundant. Hence, it is suggest to remove “cu_skip_flag == 0 and modeType != MODE_TYPE_INTER”.

Yellow part are suggested to remove

| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER ) |  |
| --- | --- |
| pred_mode_plt_flag | ae(v) |

Proposed clean-up 2

On top of proposed clean-up 1, the proposed method 2 suggested to define 4x4 and I-Slice condition as MODE_TYPE_INTRA and remove related unnecessary condition. The 4x4 coding block or coding block in I slice do not allow MODE_INTER prediction. Hence those block suggested to define MODE_TYPE_INTRA.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = (slice_type = = I ¦¦ (cbWidth = = 4 && cbHeight = = 4)) ? MODE_TYPE_INTRA : modeType |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( ( modeType != MODE_TYPE_INTER && cuPredMode[chType][x0][y0] == MODE_INTER )CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |

The proposed syntax clean-up method 2 could be rewritten as below table.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = (slice_type = = I ¦¦ (cbWidth = = 4 && cbHeight = = 4)) ? MODE_TYPE_INTRA : modeType |  |
| if(treeType != DUAL_TREE_CHROMA && (modeType != MODE_TYPE_INTRA ¦ ¦ ( sps_ibc_enabled_flag && Max(cbWidth,cbHeight) <= 64) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( modeType = = MODE_TYPE_ALL && cu_skip_flag[ x0 ][ y0 ] == 0) |  |
| pred_mode_flag | ae(v) |
| if( sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA && Max(cbWidth, cbHeight) <= 64 && (( modeType != MODE_TYPE_INTER && cuPredMode[chType][x0][y0] == MODE_INTER) ¦¦ ( modeType == MODE_TYPE_INTRA && cu_skip_flag[x0][y0] == 0) ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( sps_palette_enabled_flag && cuPredMode[chType][x0][y0] == MODE_INTRA && Max(cbWidth, cbHeight) <= 64) |  |
| pred_mode_plt_flag | ae(v) |

Related semantic also can be cleaned up as below

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode.

When pred_mode_flag is not present, it is inferred as follows:

- If cbWidth is equal to 4 and cbHeight is equal to 4, pred_mode_flag is inferred to be equal to 1.

- Otherwise, if modeType is equal to MODE_TYPE_INTRA, pred_mode_flag is inferred to be equal to 1.

- Otherwise, if modeType is equal to MODE_TYPE_INTER, pred_mode_flag is inferred to be equal to 0.

- Otherwise, pred_mode_flag is inferred to be equal to 0  1 when decoding an I slice, and equal to 0 when decoding a P or B slice, respectively.

The variable CuPredMode[ chType ][ x ][ y ] is derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- If pred_mode_flag is equal to 0, CuPredMode[ chType ][ x ][ y ] is set equal to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1), CuPredMode[ chType ][ x ][ y ] is set equal to MODE_INTRA.

pred_mode_ibc_flag equal to 1 specifies that the current coding unit is coded in IBC prediction mode. pred_mode_ibc_flag equal to 0 specifies that the current coding unit is not coded in IBC prediction mode.

When pred_mode_ibc_flag is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, and cbWidth is equal to 4, and cbHeight is equal to 4, pred_mode_ibc_flag is inferred to be equal 1.

- Otherwise, if cu_skip_flag[ x0 ][ y0 ] is equal to 1 and modeType is equal to MODE_TYPE_INTRA, pred_mode_ibc_flag is inferred to be equal 1.

- Otherwise, if either cbWidth or cbHeight are equal to 128, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if modeType is equal to MODE_TYPE_INTER, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if treeType is equal to DUAL_TREE_CHROMA, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, pred_mode_ibc_flag is infered to be equal to 0 the value of sps_ibc_enabled_flag when decoding an I slice, and 0 when decoding a P or B slice, respectively.

When pred_mode_ibc_flag is equal to 1, the variable CuPredMode[ chType ][ x ][ y ] is set to be equal to MODE_IBC for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

Experimental result

The experimental results are identical with VTM-7.0 software for both method 1 and method 2. The test are experimented under CTC and 4:4:4 color format including SCC.

Conclusion

It is suggest to adopt the proposed syntax clean-up for VVC draft version 8 and Next VTM software

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).