| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0372-v1 |
| --- | --- |

| Title: | On 6-tap interpolation filter for affine 4x4 blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhao Wang, Jiancong Luo, Yan Ye | Tel: Email: | (+86)13716516722 baixiu.wz@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In current VVC, 6-tap interpolation filters are used for the 4x4 affine motion compensation block, while 8-tap DCT-based interpolation filters are used for other blocks for the luma component. However, some anomalies of the current 6-tap filters have been observed. In this contribution, new 6-tap filter coefficients are derived by the DCT-based interpolation filter principle to fix such anomalies. Out of the sixteen phases, four phases have different filter coefficients in the proposed method compared to VVC draft 7. The BD rate savings are reported to be 0.02% (RA) and 0.04% (LD) under the CTC, and 0.05% (RA) and 0.21% (LD) for affine sequences.

Introduction

In current VVC, the DCT-based interpolation filters are used for motion compensation and extended to one-sixteenth sample accuracy [1]. For the luma component, the 8-tap coefficients are shown in Table 1. If half-pel adaptive motion vector resolution (AMVR) mode is selected and interpolated position is half-pel, a 6-tap filter [ 3, 9, 20, 20, 9, 3 ] is used. Otherwise, if motion compensated block size is 4x4 as in affine motion prediction, the 6-tap filters as shown in Table 2 are used to reduce the complexity [2].

Table 1. Luma interpolation filter in VVC

| Fractional positions | Interpolation filter coefficients for each reference sample position |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p[-3] | p[-2] | p[-1] | p[0] | p[1] | p[2] | p[3] | p[4] |
| 1/16 | 0 | 1 | -3 | 63 | 4 | -2 | 1 | 0 |
| 1/8 | -1 | 2 | -5 | 62 | 8 | -3 | 1 | 0 |
| 3/16 | -1 | 3 | -8 | 60 | 13 | -4 | 1 | 0 |
| 1/4 | -1 | 4 | -10 | 58 | 17 | -5 | 1 | 0 |
| 5/16 | -1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 3/8 | -1 | 3 | -9 | 47 | 31 | -10 | 4 | -1 |
| 7/16 | -1 | 4 | -11 | 45 | 34 | -10 | 4 | -1 |
| 1/2 | -1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9/16 | -1 | 4 | -10 | 34 | 45 | -11 | 4 | -1 |
| 5/8 | -1 | 4 | -10 | 31 | 47 | -9 | 3 | -1 |
| 11/16 | -1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 3/4 | 0 | 1 | -5 | 17 | 58 | -10 | 4 | -1 |
| 13/16 | 0 | 1 | -4 | 13 | 60 | -8 | 3 | -1 |
| 7/8 | 0 | 1 | -3 | 8 | 62 | -5 | 2 | -1 |
| 15/16 | 0 | 1 | -2 | 4 | 63 | -3 | 1 | 0 |

Table 2. Luma interpolation filter for 4x4 affine motion compensation in VVC

| Fractional positions | Interpolation filter coefficients for each reference sample position |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p[-3] | p[-2] | p[-1] | p[0] | p[1] | p[2] | p[3] | p[4] |
| 1/16 | 0 | 1 | -3 | 63 | 4 | -2 | 1 | 0 |
| 1/8 | 0 | 1 | -5 | 62 | 8 | -3 | 1 | 0 |
| 3/16 | 0 | 2 | -8 | 60 | 13 | -4 | 1 | 0 |
| 1/4 | 0 | 3 | -10 | 58 | 17 | -5 | 1 | 0 |
| 5/16 | 0 | 3 | -11 | 52 | 26 | -8 | 2 | 0 |
| 3/8 | 0 | 2 | -9 | 47 | 31 | -10 | 3 | 0 |
| 7/16 | 0 | 3 | -11 | 45 | 34 | -10 | 3 | 0 |
| 1/2 | 0 | 3 | -11 | 40 | 40 | -11 | 3 | 0 |
| 9/16 | 0 | 3 | -10 | 34 | 45 | -11 | 3 | 0 |
| 5/8 | 0 | 3 | -10 | 31 | 47 | -9 | 2 | 0 |
| 11/16 | 0 | 2 | -8 | 26 | 52 | -11 | 3 | 0 |
| 3/4 | 0 | 1 | -5 | 17 | 58 | -10 | 3 | 0 |
| 13/16 | 0 | 1 | -4 | 13 | 60 | -8 | 2 | 0 |
| 7/8 | 0 | 1 | -3 | 8 | 62 | -5 | 1 | 0 |
| 15/16 | 0 | 1 | -2 | 4 | 63 | -3 | 1 | 0 |

Observing the 6-tap filters, it is found that the 6-tap coefficients are obtained by adding p[-3] and p[-2] of 8-tap coefficients together to get p[-2], and setting p[-3] to 0, and adding p[4] and p[3] of 8-tap coefficients together to get p[3] and setting p[4] to 0. The 6-tap filters derived by this way are not aligned with the design principle of DCT-based interpolation filters. In Table 2, the highlighted rows show four phases where filter coefficients are somewhat abnormal.

Proposed method

In this proposed method, the 6-tap filters are derived following the principle of DCT-based filters [3]. The proposed coefficients are shown in Table 3, where the highlighted rows have coefficients different from the VVC draft 7, and where the filter coefficient anomalies have been fixed. Compared to the current 6-tap filters, the performance of the proposed 6-tap filters and the VVC 8-tap filters are shown in the following section.

Table 3. Proposed 6-tap luma interpolation filters for 4x4 affine motion compensation

| Fractional positions | Interpolation filter coefficients for each reference sample position |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p[-3] | p[-2] | p[-1] | p[0] | p[1] | p[2] | p[3] | p[4] |
| 1/16 | 0 | 1 | -3 | 63 | 4 | -2 | 1 | 0 |
| 1/8 | 0 | 1 | -5 | 62 | 8 | -3 | 1 | 0 |
| 3/16 | 0 | 2 | -8 | 60 | 13 | -4 | 1 | 0 |
| 1/4 | 0 | 3 | -10 | 58 | 17 | -5 | 1 | 0 |
| 5/16 | 0 | 3 | -10 | 53 | 24 | -8 | 2 | 0 |
| 3/8 | 0 | 3 | -10 | 49 | 29 | -9 | 2 | 0 |
| 7/16 | 0 | 3 | -11 | 45 | 34 | -10 | 3 | 0 |
| 1/2 | 0 | 3 | -11 | 40 | 40 | -11 | 3 | 0 |
| 9/16 | 0 | 3 | -10 | 34 | 45 | -11 | 3 | 0 |
| 5/8 | 0 | 2 | -9 | 29 | 49 | -10 | 3 | 0 |
| 11/16 | 0 | 2 | -8 | 24 | 53 | -10 | 3 | 0 |
| 3/4 | 0 | 1 | -5 | 17 | 58 | -10 | 3 | 0 |
| 13/16 | 0 | 1 | -4 | 13 | 60 | -8 | 2 | 0 |
| 7/8 | 0 | 1 | -3 | 8 | 62 | -5 | 1 | 0 |
| 15/16 | 0 | 1 | -2 | 4 | 63 | -3 | 1 | 0 |

Experimental results

Proposed 6-tap filters vs current 6-tap filters

|  | Random access Main10 Over VTM-7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | EncT | DecT |  |
| Class A1 | 0.00% |  | -0.08% |  | -0.02% | 100% |  | 100% |
| Class A2 | -0.03% |  | 0.02% |  | 0.01% | 100% |  | 100% |
| Class B | -0.01% |  | 0.03% |  | -0.05% | 100% |  | 100% |
| Class C | -0.04% |  | -0.01% |  | -0.01% | 100% |  | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.02% |  | -0.01% |  | -0.02% | 100% |  | 100% |
| Class D | -0.12% |  | 0.00% |  | 0.06% | 100% |  | 100% |
| Affine | -0.05% |  | -0.08% |  | 0.04% | 100% |  | 100% |

|  | Lowdelay Main10 Over VTM-7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% |  | -0.13% |  | -0.05% |  | 100% |  | 100% |
| Class C | -0.07% |  | -0.43% |  | -0.15% |  | 100% |  | 100% |
| Class E | -0.02% |  | 0.35% |  | -0.18% |  | 100% |  | 99% |
| Overall | -0.04% |  | -0.11% |  | -0.12% |  | 100% |  | 100% |
| Class D | -0.06% |  | 0.09% |  | 0.21% |  | 100% |  | 99% |
| Affine | -0.21% |  | -0.55% |  | -0.33% |  | 100% |  | 100% |

8-tap filters vs current 6-tap filters

|  | Random access Main10 Over VTM-7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% |  | -0.02% |  | 0.00% |  | 100% |  | 100% |
| Class A2 | -0.03% |  | -0.01% |  | -0.01% |  | 100% |  | 100% |
| Class B | -0.08% |  | -0.03% |  | -0.06% |  | 100% |  | 100% |
| Class C | -0.06% |  | -0.02% |  | -0.05% |  | 100% |  | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.06% |  | -0.02% |  | -0.03% |  | 100% |  | 100% |
| Class D | -0.09% |  | -0.07% |  | -0.06% |  | 100% |  | 100% |
| Affine | -0.18% |  | -0.12% |  | -0.13% |  | 100% |  | 100% |

|  | Lowdelay Main10 Over VTM-7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.08% |  | -0.28% |  | -0.13% | 100% |  | 100% |
| Class C | -0.20% |  | -0.37% |  | -0.17% | 100% |  | 100% |
| Class E | -0.07% |  | 0.56% |  | -0.36% | 100% |  | 99% |
| Overall | -0.12% |  | -0.10% |  | -0.20% | 100% |  | 100% |
| Class D | -0.94% |  | -0.66% |  | -1.01% | 100% |  | 99% |
| Affine | -0.67% |  | -0.98% |  | -0.64% | 100% |  | 100% |

Conclusion

This contribution proposes 6-tap interpolation filter coefficients derived using the principle of DCT-based interpolation filters. The BD rate savings are reported to be 0.02% (RA) and 0.04% (LD) under the CTC, and 0.05% (RA) and 0.21% (LD) for affine sequences. When 8-tap filters are used (which would incur higher complexity especially in terms of higher memory bandwidth for motion compensation), 0.06% (RA) and 0.12% (LD) BD-rate savings under the CTC, and 0.18% (RA) and 0.67% (LD) BD-rate savings for affine sequences can be obtained. Compared to the 8-tap filters, the proposed filters achieve one third performance recovery without any other negative impact on the VVC design.

References

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, October 2019.

J. Li, C.-W. Kuo, C. S. Lim, “CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6),” JVET-N0196, March, 2019.

Ugur K, Alshin A, Alshina E, Bossen F, HanW, Park J, Lainema J, “Motion compensated prediction and interpolation filter design in H.265/HEVC,” IEEE J Sel Top Signal Process 7(6): 946–956, 2013.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).