| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0374-v1 |
| --- | --- |

| Title: | AHG9: Cleanups on redundant signalling in HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Geonjung Ko, Jaehong Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 dongcheol.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes cleanups on redundant signalling in the SPS. The first proposal is to change the signalling condition of partition constraint parameters and CU QP delta related parameters. The maximum value of the above syntax element is decided based on the difference between MinCbSizeY(MinCbLog2SizeY) and CtbSizeY(CtbLog2SizeY). If the difference of MinCbSizeY and CtbSizeY is equal to 0, each of those syntax elements has only one value as 0. Therefore, it can be inferred to be equal to 0 without signalling. The second proposal is to include the condition of either sps_palette_enabled_flag equal to 1 or sps_transform_skip_enabled_flag equal to 1 to signal min_qp_prime_ts_minus4 in the SPS. It is because this syntax element is only used for palette mode and transform skip mode.

Introduction

On signalling of partition constraint parameters

In the current VVC draft [1], partition constraint parameters are signalled in the SPS. If partition_constraints_override_enabled_flag in the SPS is equal to 1, those parameters may be signalled in the PH again. Partition constraint parameters signalled in the PH override those in the SPS.

The followings are captured in the VVC draft [1].

7.4.3.3 Sequence parameter set RBSP semantics … The variables CtbLog2SizeY and CtbSizeY are derived as follows: CtbLog2SizeY = sps_log2_ctu_size_minus5 + 5 CtbSizeY = 1 << CtbLog2SizeY The value range of log2_min_luma_coding_block_size_minus2 shall be in the range of 0 to log2_ctu_size_minus5 + 3, inclusive. The variables MinCbLog2SizeY, MinCbSizeY are derived as follows: MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2 MinCbSizeY = 1 << MinCbLog2SizeY VSize = Min( 64, CtbSizeY ) The value of MinCbSizeY shall less than or equal to VSize.

When the value of log2_min_luma_coding_block_size_minus2 is equal to log2_ctu_size_minus5 + 3, the variable MinCbSizeY is equal to the variable CtbSizeY. However, when log2_ctu_size_minus5 is equal to 2, this does not occur because of the variable VSize.

Table 1 shows partition constraint parameters in the Sequence Parameter Set RBSP syntax

Table 1. partition constraint parameters in the Sequence Parameter Set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| … |  |

The following parameters shall be in the range of 0 to the maximum value based on the difference between CtbLog2SizeY and MinCbLog2SizeY.

The followings are captured in the VVC draft [1].7.4.3.3 Sequence parameter set RBSP semantics … - The value of sps_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows: MinQtLog2SizeIntraY=sps_log2_diff_min_qt_min_cb_intra_slice_luma + MinCbLog2SizeY - The value of sps_log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows: MinQtLog2SizeInterY = sps_log2_diff_min_qt_min_cb_inter_slice + MinCbLog2SizeY - The value of sps_max_mtt_hierarchy_depth_inter_slice shall be in the range of 0 to 2*( CtbLog2SizeY − MinCbLog2SizeY ), inclusive. - The value of sps_max_mtt_hierarchy_depth_intra_slice_luma shall be in the range of 0 to 2*( CtbLog2SizeY − MinCbLog2SizeY ), inclusive. - The value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. - The value of sps_max_mtt_hierarchy_depth_intra_slice_chroma shall be in the range of 0 to 2*( CtbLog2SizeY − MinCbLog2SizeY ), inclusive.

Table 2 shows partition constraint parameters in the Picture header RBSP syntax.

When partition_constraints_override_enabled_flag in the SPS is equal to 1 and partition_constraints_override_flag in the PH is equal to 1, partition constraint parameters are signalled in the PH. Partition constraint paratmeters signalled in the PH have the same range as those in the SPS.

Table 2. partition constraint parameters in the Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

Table 3 shows the CU QP delta related parameters in the PH. The range of those parameters is also decided based on the difference between CtbLog2SizeY and MinCbLog2SizeY.

Table 3. CU QP delta related parameters in the Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| … |  |

The following parameters shall be in the range of 0 to the maximum value based on the difference between CtbLog2SizeY and MinCbLog2SizeY.

7.4.3.6 Picture header RBSP semantics … - The value of pic_cu_qp_delta_subdiv_intra_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeIntraY + pic_max_mtt_hierarchy_depth_intra_slice_luma ), inclusive. - The value of pic_cu_qp_delta_subdiv_inter_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeInterY + pic_max_mtt_hierarchy_depth_inter_slice ) , inclusive. - The value of pic_cu_chroma_qp_offset_subdiv_intra_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeIntraY + pic_max_mtt_hierarchy_depth_intra_slice_luma ) , inclusive. - The value of pic_cu_chroma_qp_offset_subdiv_inter_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeInterY + pic_max_mtt_hierarchy_depth_inter_slice ) , inclusive.

If the difference between CtbLog2SizeY and MinCbLog2SizeY is equal to 0, all the above parameters are equal to 0. Therefore, it can be inferred to be equal to 0 without signalling in this case.

On signalling of min_qp_prime_ts_minus4

In the current VVC draft [1], the syntax element of min_qp_prime_ts_minus4 is signalled in the SPS.

Table 4. min_qp_prime_ts_minus4 in the SPS RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| bit_depth_minus8 | ue(v) |
| min_qp_prime_ts_minus4 | ue(v) |
| sps_weighted_pred_flag | u(1) |
| … |  |

The semantics of it is as follows:

7.4.3.3 Sequence parameter set RBSP semantics … min_qp_prime_ts_minus4 specifies the minimum allowed quantization parameter for transform skip mode as follows: QpPrimeTsMin = 4 + min_qp_prime_ts_ minus4 The value of min_qp_prime_ts_minus4 shall be in the range of 0 to 48, inclusive.

The variable QpPrimeTsMin is used for Palette mode and Transform skip mode in the current VVC draft [1] as follows:

Palette mode:

8.4.5.3 Decoding process for palette mode … - If bIsEscapeSample is equal to 0, the following applies: recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCbL + xL ][ yCbL + yL ] ] (454) - Otherwise (bIsEscapeSample is equal to 1), the following ordered steps apply: - The quantization parameter qP is derived as follows: - If cIdx is equal to 0, qP = Max( QpPrimeTsMin, Qp′Y ) (455) - Otherwise, if cIdx is equal to 1, qP = Max( QpPrimeTsMin, Qp′Cb ) (456) - Otherwise (cIdx is equal to 2), qP = Max( QpPrimeTsMin, Qp′Cr ) (457)

Transform skip mode:

8.7.3 Scaling process for transform coefficients … The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows: - If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies: qP = qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 ) (1133) rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 ) = = 1 ) ? 1 : 0 (1134) bdShift = BitDepth + rectNonTsFlag + (1135) ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag - Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies: qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 ) (1136)

It would be reasonable to signal min_qp_prime_ts_minus4 in the SPS only when either sps_palette_enabled_flag is equal to 1 or sps_transform_skip_enabled_flag in SPS is equal to 1.

Proposed Method

Proposal 1:

In this contribution, it is proposed that the following parameters are signalled when the variable MinCbLog2SizeY is not equal to the variable CtbLog2SizeY.

- Partition constraint parameters and partition_constraints_override_enabled_flag in the SPS

  - partition_constraints_override_enabled_flag

  - sps_log2_diff_min_qt_min_cb_intra_slice_luma

  - sps_log2_diff_min_qt_min_cb_inter_slice

  - sps_max_mtt_hierarchy_depth_inter_slice

  - sps_max_mtt_hierarchy_depth_intra_slice_luma

  - sps_log2_diff_min_qt_min_cb_intra_slice_chroma

  - sps_max_mtt_hierarchy_depth_intra_slice_chroma

- CU QP delta related parameters in the PH

  - pic_cu_qp_delta_subdiv_intra_slice

  - pic_cu_qp_delta_subdiv_inter_slice

  - pic_cu_chroma_qp_offset_subdiv_intra_slice

  - pic_cu_chroma_qp_offset_subdiv_inter_slice

The detailed syntax and semantics changes are in an attachment to this contribution. In this section, the proposed syntax changes are provided. The proposed syntax changes with the green highlighting are as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| If(MinCbSizeY !=CtbSizeY){ |  |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag && MinCbSizeY !=CtbSizeY) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| … |  |

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_qp_delta_enabled_flag && CtbSizeY != MinCbSizeY ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag && CtbSizeY != MinCbSizeY ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| … |  |

Proposal 2:

It is proposed that the syntax element of min_qp_prime_ts_minus4 is signalled in the SPS when either sps_palette_enabled_flag is equal to 1 or sps_transform_skip_enabled_flag in SPS is equal to 1.

The proposed syntax changes with the green highlighting are as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| bit_depth_minus8 | ue(v) |
| min_qp_prime_ts_minus4 | ue(v) |
| ⋯ |  |
| sps_transform_skip_enabled_flag | u(1) |
| ⋯ |  |
| if( chroma_format_idc = = 3 ) { |  |
| sps_act_enabled_flag | u(1) |
| sps_palette_enabled_flag | u(1) |
| } |  |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| min_qp_prime_ts_minus4 | ue(v) |
| ⋯ |  |
| } |  |

min_qp_prime_ts_minus4 specifies the minimum allowed quantization parameter for transform skip mode as follows:

QpPrimeTsMin = 4 + min_qp_prime_ts_ minus4		(47)

The value of min_qp_prime_ts_minus4 shall be in the range of 0 to 48, inclusive. When not present, the value of min_qp_prime_ts_minus4 is inferred to be equal to 0.

Conclusions

This contribution proposes cleanups on redundant signalling in HLS.

The first proposal is to change the signalling condition of partition constraint parameters and CU QP delta related parameters. If the difference of MinCbLog2SizeY and CtbLog2SizeY is equal to 0, each of those syntax elements has only one value as 0. Therefore, it is proposed to infer as 0 without signalling in this case.

The second proposal is to include the condition of either sps_palette_enabled_flag equal to 1 or sps_transform_skip_enabled_flag equal to 1 to signal min_qp_prime_ts_minus4 in the SPS.

References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).