| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0378r1 |
| --- | --- |

| Title: | Non-CE5: On the number of ALF Chroma filters. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrice Onno Guillaume Laroche Nael Ouedraogo Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes to fix the VVC Draft 7 specifications about the number of alternative ALF Chroma filter. It is asserted in the current VVC draft that the maximum number of Chroma filter for ALF related to the alt_ctb_filter_alt_idx is not indicated. It means that the size of the APS to store the ALF data cannot be bounded in the current VVC draft. This contribution proposes to limit the number of the ALF Chroma filters in the VVC draft.

# Introduction

As proposed in JVET-O0090 and adopted in VTM6.0, several Chroma filters can be signaled in an Adaptive Parameter Set. One particular Chroma filter can be then selected per Chroma component and per Coded Tree Unit through an index alf_ctb_filter_alt_idx in the CTU data.

In the current VVC draft [1], there is no upper limit regarding the number of Chroma filters despite the number is limited to 8 in the VTM 7.0. This issue was also detected in the VVC bug tracker as issue #713.

# Proposal

One of the solution to fix the issue is to align the VVC draft text [1] to the VTM7.0 to align the maximum number of Chroma filters to 8 as also suggested in JVET-Q0041.

In the next section, we investigated the impact when changing the maximum number (N) of Chroma ALF filters in the VTM7.0. By changing this number, different memory reduction/coding efficiency gains trade-off can be obtained.

# Results

The proposed tests are implemented on top of VTM-7.0 and the simulation are performed under common test conditions as described in JVET-N1010 [2]. Results also includes a YUV column which is computed as (8*Y+U+V)/10, to provide an overall coding gains approximation.

## N=6

## N=7

## N=10

# Memory storage reduction for ALF Chroma filter storage

# Per ALF Chroma filter with 6 coefficients, the needed storage is the following:

# 1 bit		for the clipping enabling flag

# 42 bits		for the 6 coefficients coded on 7 bits

# 6 bits		for the 6 sign bits

# 12 bits		for the 6 clipping index using 2 bits

# Total: 61 bits per filter

# The total number of APS for ALF is 8.

# By reducing the number of Chroma filters from 8 to 6 filters, 122 bytes can be saved for the ALF APS.

# Conclusion

This contribution proposed to fix the issue related to the maximum number of Chroma filter to be signalled in an APS. The value of alternative filters could be set to N=8 as in the VTM.

From a coding efficiency/memory usage point of view, and with the results provided in the above section, setting the maximum number of filter to 6 does not reduce the coding efficiency while reducing the memory by 122 bytes.

# Syntax changes

The change is highlighted in yellow below.

alf_ctb_filter_alt_idx[ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] specifies the index of the alternative chroma filter applied to the coding tree block of the chroma component, with chromaIdx equal to 0 for Cb and chromaIdx equal 1 for Cr, of the coding tree unit at luma location ( xCtb, yCtb ). The value of alf_chroma_num_alt_filters_minus1 shall be in the range of 0 to N-1,  inclusive.

When alf_ctb_filter_alt_idx[ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is infered to be equal to zero.

# References

- Versatile Video Coding (Draft 7), Document JVET-P2001_v14, B. Bross, J. Chen, S. Liu

- JVET common test conditions and software reference configurations for SDR video, JVET-N1010

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).