| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0379 |
| --- | --- |

| Title: | AhG9: On the position of APS Ids in Picture Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Nael Ouedraogo Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to move the syntax elements related to the APS ID at an early stage of the Picture header and Slice header. The aim of this proposed modification is to reduce the parsing complexity for some streaming applications that need to track the APS ID in the Picture header and Slice header to remove unused APS.

The proposed modification has no impact on the BDR performance.

# Problem Statement

Some streaming applications need to track the different APS ID, to keep only the APS used. These applications only need to parse the VPS, SPS, PPS, PH and Slice header.

For example, applications which change the first frame of a sequence to another IRAP or GDR picture, need to analyse the different APS ID of a sequence to know the APS which shall be kept or not.

In the same way, for bitstream extraction, the APS ID shall be analysed to keep the correct set of APS.

In the current VVC specification, there are 3 types of APS for ALF, LMCS and scaling list. The syntax elements to decode the APS ID are set in the Picture header. For ALF, the APS Id can also be signaled at the slice header.

In the current design, these applications shall parse the entire Picture header and the entire Slice header as these APS IDs are at the end of the Picture header and Slice header as depicted in Figure 1 and Figure 2.

Many of these different syntax elements are present only to set parameters to parse/decode the slice data. These tools information are not useful for such streaming applications.

Moreover, many of these syntax elements depend on values of other syntax element sets in the SPS and/or PPS. All these values need to be kept in memory to parse the Picture header and slice header.

| Figure 1 Positions of the APS ID in the table of Picture header syntax elements | Figure 2 Position of ALF APS ID in the table of Slice header syntax elements |
| --- | --- |

# Proposed modifications

The proposed changes compared to the draft specification are attached to this contribution.

The syntax elements related to the APS ID of ALF, LMCS and Scaling list are set just after the poc_msb_val syntax element as depicted in Figure 3.

Figure 3 Proposed modifications of the Picture header

In the same way, the syntax elements related to the APS ID of ALF is moved just after the slice_type as depicted in Figure 4.

Figure 4 Proposed modifications of the Slice header

# Conclusion

In this contribution, the syntax elements related to APS ID are moved at an early position in both Picture header and Slice header to reduce the parsing complexity of applications that track the APS ID.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).