| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0385v2 |
| --- | --- |

| Title: | Simplification of intra prediction in CIIP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin Ghaznavi-Youvalari Jani Lainema Krit Panusopone | Email: | ramin.ghaznavi-youvalari@nokia.com jani.lainema@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the intra prediction part of the combined inter intra prediction (CIIP) mode by disabling or limiting the usage of the PDPC filter. Three alternatives are proposed in this contribution. The first method is to disable the PDPC filter always when the block is coded in CIIP mode. The second method enables the PDPC filter only when at least one of the neighboring blocks is coded in intra prediction mode. The third method enables the PDPC when at least one of the neighboring blocks is coded in intra mode or includes residual coding. The simulation results in VTM-7.0 are as below:

Method 1:

RA:  0.01%, -0.03%, 0.01%

LB: -0.01%, -0.02%, -0.05%

Method 2:

RA:  0.00%, -0.01%, -0.01%

LB: -0.01%, 0.03%, -0.07%

Method 3:

RA:  0.01%, 0.00%, 0.03%

LB: -0.03%, -0.16%, -0.04%

The proposed methods reduce the complexity of CIIP mode in terms of number of calculations with almost no impact in coding performance and execution times.

# Proposed changes

The combined inter intra prediction (CIIP) tool uses planar prediction for intra prediction part. By default, the planar prediction includes applying the PDPC filters to the predicted intra block. This contribution proposes to simplify this process by disabling the PDPC filter always or applying it to the intra prediction part depending on the neighboring block’s (the same neighboring blocks that are used for weight derivation in CIIP) prediction modes. Three methods have been considered as below:

Method 1: Disable the PDPC filter always when the block is in CIIP mode.

Method 2: Enable the PDPC filter when at least one of the neighboring blocks (i.e., top and left) is coded in intra prediction mode.

Method 3: Enable the PDPC filter when at least one of the neighboring blocks (i.e., top and left) is coded in intra prediction mode or includes residual coding.

Figure 1. Top and left neighboring blocks used in PDPC usage decision

# Results

The objective performances of the proposed methods are shown in below tables in comparison to VTM-7.0 using CTC [1].

Table 1. BD-Rate (%) result of method #1 with RA and LB configurations

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | -0.08 % | -0.03 % | 100 % | 101 % |  |  |  |  |  |
| Class A2 | -0.03 % | -0.01 % | 0.05 % | 100 % | 101 % |  |  |  |  |  |
| Class B | 0.01 % | -0.04 % | 0.01 % | 100 % | 101 % | 0.01 % | -0.19 % | 0.04 % | 100 % | 100 % |
| Class C | 0.03 % | 0.01 % | 0.02 % | 100 % | 100 % | 0.01 % | -0.16 % | 0.07 % | 100 % | 101 % |
| Class E |  |  |  |  |  | -0.07 % | 0.47 % | -0.37 % | 100 % | 101 % |
| Overall | 0.01 % | -0.03 % | 0.01 % | 100 % | 101 % | -0.01 % | -0.02 % | -0.05 % | 100 % | 101 % |
| Class D | -0.03 % | -0.07 % | 0.07 % | 100 % | 102 % | -0.02 % | -0.04 % | -0.18 % | 100 % | 103 % |
| Class F | -0.01 % | -0.05 % | 0.00 % | 99 % | 101 % | 0.01 % | -0.57 % | 0.61 % | 100 % | 101 % |

Table 2. BD-Rate (%) result of method #2 with RA and LB configurations

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02 % | -0.06 % | -0.04 % | 100 % | 100 % |  |  |  |  |  |
| Class A2 | -0.01 % | -0.04 % | 0.03 % | 100 % | 100 % |  |  |  |  |  |
| Class B | 0.00 % | 0.01 % | -0.05 % | 99 % | 100 % | 0.02 % | -0.09 % | 0.06 % | 99 % | 100 % |
| Class C | 0.00 % | 0.01 % | 0.03 % | 100 % | 96 % | 0.05 % | -0.07 % | 0.12 % | 99 % | 101 % |
| Class E |  |  |  |  |  | -0.13 % | 0.37 % | -0.53 % | 99 % | 101 % |
| Overall | 0.00 % | -0.01 % | -0.01 % | 100 % | 99 % | -0.01 % | 0.03 % | -0.07 % | 99 % | 101 % |
| Class D | 0.01 % | -0.08 % | -0.10 % | 99 % | 98 % | -0.09 % | -0.01 % | -0.35 % | 99 % | 101 % |
| Class F | -0.04 % | -0.01 % | 0.02 % | 100 % | 97 % | 0.00 % | -0.27 % | 0.62 % | 99 % | 99 % |

Table 3. BD-Rate (%) result of method #3 with RA and LB configurations

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02 % | -0.06 % | 0.00 % | 101 % | 100 % |  |  |  |  |  |
| Class A2 | -0.03 % | 0.06 % | -0.02 % | 101 % | 100 % |  |  |  |  |  |
| Class B | 0.00 % | 0.03 % | 0.06 % | 100 % | 100 % | -0.01 % | -0.24 % | -0.08 % | 100 % | 99 % |
| Class C | 0.03 % | -0.04 % | 0.05 % | 99 % | 99 % | -0.04 % | -0.24 % | 0.05 % | 99 % | 100 % |
| Class E |  |  |  |  |  | -0.06 % | 0.07 % | -0.10 % | 99 % | 99 % |
| Overall | 0.01 % | 0.00 % | 0.03 % | 100 % | 100 % | -0.03 % | -0.16 % | -0.04 % | 100 % | 99 % |
| Class D | 0.00 % | -0.01 % | 0.11 % | 100 % | 100 % | -0.11 % | 0.43 % | -0.26 % | 99 % | 100 % |
| Class F | -0.02 % | -0.01 % | -0.02 % | 99 % | 99 % | 0.00 % | -0.57 % | -0.18 % | 99 % | 98 % |

# Proposed specification text

The spec text changes for the proposed method are as below:

Method 1:

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, and the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as inputs, and the output is the modified predicted sample array predSamples:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

- ciip_flag[ xTbCmp ][ yTbCmp ] is equal to 0

Method 2:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

- ciip_flag[ xTbCmp ][ yTbCmp ] is equal to 0 or if at least one of isIntraCodedNeighbourA and isIntraCodedNeighbourB (of clause 8.5.6.7) is equal to TRUE

Method 3:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

- ciip_flag[ xTbCmp ][ yTbCmp ] is equal to 0 or at least one of the following conditions is true:

- isIntraCodedNeighbourA (of clause 8.5.6.7) or isResidualCodedNeighbourA is TRUE

- isIntraCodedNeighbourB (of clause 8.5.6.7) or isResidualCodedNeighbourB is TRUE

For X being replaced by either A or B, the variables availableX and isResidualCodedNeighbourX are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as specified in clause 8.5.6.7 , checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableX.

- The variable isResidualCodedNeighbourX is derived as follows:

- If availableX is equal to TRUE and cu_cbf[ xNbX ][ yNbX ] is equal to 1, isResidualCodedNeighbourX is set equal to TRUE.

- Otherwise, isResidualCodedNeighbourX is set equal to FALSE.

Note: isResidualCodedNeighbourA and isResidualCodedNeighbourB are derived similar to what is already specified in clause 8.5.6.7.

# Conclusion

The proposed methods simplify the intra prediction scheme of CIIP tool. Furthermore, the required changes in the spec and software are very minimal.  It is proposed to adopt one of the described alternatives to the VVC draft and VTM software.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).