| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0397-v3 |
| --- | --- |

| Title: | AHG12: On subpicture extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution asserts that the current VVC draft specification misses a normative subpicture extraction process and proposes to add a subpicture specific extraction support to VVC by a two fold proposal:

- Add an extraction process for subpicture with the required parameter set rewriting which would support AU-based HRD operation.

- Extending the scalable-nesting SEI message with subpicture support to support DU-based HRD operation.

Revision 1 of the contribution adds specification text for Annex C integration of the Proposals and alignment to the decisions of the current meeting regarding picture header signaling of subpicture ID values in JVET-Q0119.

Revision 2 of this document adds specification text as attachment incorporating comments after BoG discussion.

# Introduction

The current VVC draft specification misses a normative subpicture extraction process. This contribution proposes to add a subpicture specific extraction process to VVC with support of HRD signaling by means of extending the scalable-nesting SEI message with subpicture support and an extraction process with parameter set rewriting.

The OLS-specific sub-bitstream extraction process in the VVC draft specification entails the dropping of VCL NAL units that do not correspond to the (sub-)layers and layers of the target OLS. The non-VCL NAL units of type SPS_NUT, PPS_NUT and Picture Headers are removed when the respective layers are not in the target OLS and when not removed they may refer to sub-layers which are not present in the bitstream. A decoder can simply ignore this superfluous information. The non-VCL NAL units of type VPS_NUT and DPS_NUT are to remain in any extracted output bitstream and are also allowed to refer to layers and sub-layers that are not present in the bitstream.

However, for subpicture extraction, VCL NAL units have to be dropped at a finer granularity than per layer or sub-layer. An issue arises for Parameter Sets. The design principle of OLS-specific extraction to ignore any superfluous signaling becomes too complicated and costly to maintain in case of subpicture extraction and changes to the decoding process would be required to make the decoder aware of missing subpictures.

The respective HEVC solution (in context of MCTS) are nesting SEI messages that carry replacement parameter sets for each extracted combination. It is asserted that although a possible solution, nested parameter sets might lead to a high overhead as the number of combinations increases.

Therefore, this document proposes a subpicture extraction process that consists of rewriting the original parameter sets with or without assisting information in the form of subpicture extraction hint SEI messages or leave it to external means to provide replacement parameter sets.

While this document asserts that no special treatment of BP/PT SEI messages for access unit based HRD operation will be necessary, decoding unit (DU) based HRD operation requires additional measures. This is why this document also proposes to add a subpicture nesting SEI message to carry replacement HRD SEI messages for the case that subpictures are extracted, similar to the MCTS nesting SEI messages in HEVC.

Furthermore, certain constraints are imposed in order to achieve a reasonable complexity of the extraction process.

# Proposal 1: Subpicture extraction with AU-based HRD support

The proposal is to specify an extraction process for subpictures based on removing VCL NAL units not belonging to the subpicture to be extracted, removing parameter sets and adding proper ones and removing non-relevant non-VCL NAL units.

The regular OLS-specific extraction process is invoked at the beginning of the subpicture extraction process.

For rewriting of the parameter sets in the subpicture sub-bitstream extraction, note that the parameter sets contain information about (max and actual) coded picture size, profile_tier_level, tiling/slicing grid structure, and, of course, subpicture structure and IDs, all of which requires to be adjusted when subpicture extraction is carried out.

In order to simplify specifying the subpicture sub-bitstream extraction process, a number of constraints on the input bitstream are proposed, e.g.

–	The target subpicture set for extraction subpicIdTarget[ ] is a rectangular set of subpictures within each layer of the target OLS.

–	Each subpicture in subpicIdTarget[ ] has subpic_treated_as_pic_flag[ i ] equal to 1.

C.X Subpicture Sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and an array of target subpicture ID lists subpicIdTarget[ ].

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and subpicIdTarget[ ] containing subpicture IDs present in the bitstream, as inputs, and that satisfies the following conditions shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ j ] and with slice_subpic_id equal to each of the slice_subpic_id values in subpicIdTarget[ j ] for each j in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1.

–	The output sub-bitstream contained subpictures in subpicIdTarget[ j ] form a reactangular shape in the coded picture and have for each equal to 1 for each j in the range of 0 to NumLayersInOls[ targetOlsIdx ].

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

–	When subpicIdTarget[ j ] does not contain all subpicture IDs present in each of the layers with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ j ] with j in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1, the following applies:

–	If some external means not specified in this Specification is available  to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets.

–	Otherwise, when subpicture level information subpic_treated_as_pic_flag[ i ] SEI messages are present in inBitstream, rewrite the value of general_level_idc in the ols_ptl_idx[ targetOlsIdx ]-th entry in the list of profile_tier_level() syntax structure in all VPS NAL units to be equal to SubPicSetLevelIdc derived in D.3.8 for the combined list from the array subpicIdTarget[ ].

For the j-th layer with j in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1, the following applies

–	When subpicture level information SEI messages are present in inBitstream, rewrite the value of general_level_idc in the profile_tier_level() syntax structure in all active SPS NAL units with sps_ptl_dpb_hrd_params_present_flag equa to 1 to be equal to SubPicSetLevelIdc derived in D.3.8 for the list subpicIdTarget[ j ]

–	Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all active SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all active PPS NAL units to correspond to the derived sample array size.

–	Rewrite the value of sps_num_subpics_minus1 in all active SPS NAL units and pps_num_subpics_minus1 in all active PPS NAL units to the number of entries in the list subpicIdTarget[ j ] minus 1.

–	Remove the syntax elements subpic_ctu_top_left_x[ i ], subpic_ctu_top_left_y[ i ], subpic_width_minus1[ i ], subpic_height_minus1[ i ], subpic_treated_as_pic_flag[ i ], loop_filter_across_subpic_enabled_flag[ i ], and sps_subpic_id[ i ] in all active SPS NAL units and for each i for which sps_subpic_id[ i ] is not equal to any entry in the list subpicIdTarget[ j ] that are present in the layer.

–	Rewrite the remaining syntax elements subpic_ctu_top_left_x[ i ], subpic_ctu_top_left_y[ i ], when present, in all active SPS NAL units or active PPS NAL units to indicate the correct position within the coded picutres of each subpicture with a subpicture ID equal to an entry in subpicIdTarget[ j ] that are present in the layer

–	Rewrite the tiling and slice structure related syntax elements in all active PPS to remove all tiles rows, tile columns and slices that are not associated to subpictures with a subpicture ID equal to an entry in subpicIdTarget[ j ] that are present in the layer.

–	Remove from outBitstream all NAL units with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ j ] and with slice_subpic_id not included in the list subpicIdTarget[ j ] with j in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1.

# Proposal 2: Adding DU-based HRD support

## Adding subpicture nesting to the scalable nesting SEI message

| scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| nesting_ols_flag | u(1) |
| subpic_nesting_flag | u(1) |
| if( nesting_ols_flag ) { |  |
| nesting_num_olss_minus1 | ue(v) |
| for( i = 0; i <= nesting_num_olss_minus1; i++ ) |  |
| nesting_ols_idx_delta_minus1[ i ] | ue(v) |
| } else { |  |
| nesting_all_layers_flag | u(1) |
| if( !nesting_all_layers_flag ) { |  |
| nesting_num_layers_minus1 | ue(v) |
| for( i = 1; i <= nesting_num_layers_minus1; i++ ) |  |
| nesting_layer_id[ i ] | u(6) |
| } |  |
| if ( subpic_nesting_flag ) { |  |
| subpic_nesting_num_minus1 | ue(v) |
| for( i = 0; i <= subpic_nesting_num_minus1; i++ ) { |  |
| num_nested_subpic_ids_minus1[ i ] | ue(v) |
| sei_subpic_id_len_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_nested_subpic_ids_minus1; j++ ) |  |
| nesting_subpic_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| nesting_num_seis_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i <= nesting_num_seis_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

The scalable nesting SEI message provides a mechanism to associate SEI messages with specific OLSs or with specific layers and also associate the SEI messages with specific sets of subpictures.

A scalable nesting SEI message contains one or more SEI messages. The SEI messages contained in the scalable nesting SEI message are also referred to as the scalable-nested SEI messages.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in a scalable nesting SEI message:

–	An SEI message that has payloadType equal to 132 (decoded picture hash) or 133 (scalable nesting) shall not be contained in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a buffering period, picture timing, or decoding unit information SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

It is a requirement of bitstream conformance that the following restrictions apply on the value of the nal_unit_type of the SEI NAL unit containing a scalable nesting SEI message:

–	When a scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (buffering period), 1 (picture timing), 130 (decoding unit information), 145 (DRAP indication), or 168 (frame-field information), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT.

nesting_ols_flag equal to 1 specifies that the scalable-nested SEI messages apply to specific OLSs. nesting_ols_flag equal to 0 specifies that the scalable-nested SEI messages apply to specific layers.

subpic_nesting_flag equal to 1 specifies that the scalable-nested SEI messages that apply to specific OLSs or layers, apply only to a sets of subpictures of the specified OLSs or layers. subpic_nesting_flag equal to 0 specifies that the scalable-nested SEI messages that apply to specific OLSs or layers, apply to the set of all subpictures of the specific OLSs or layers.

It is a requirement of bitstream conformance that the following restrictions apply on the value of nesting_ols_flag:

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the value of nesting_ols_flag shall be equal to 1.

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to a value in VclAssociatedSeiList, the value of nesting_ols_flag shall be equal to 0.

[…]

subpic_nesting_num_minus1 plus 1 specifies the number of sets of subpictures to which the scalable-nested SEI messages apply.

num_nested_subpic_ids_minus1[ i ] plus 1 specifies the number of the following subpicture IDs corresponding to the i-th set of subpictures. The value of num_nested_subpic_ids_minus1 shall be in the range of 0 to 254, inclusive.

sei_subpic_id_len_minus1[ i ] plus 1 specifies the number of bits used to represent the syntax element nesting_subpic_id[ i ]. The value of sei_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sei_subpic_id_len_minus1[ i ] shall be the same for all SEI messages that are present in a CLVS.

nesting_subpic_id[ i ][ j ] indicates the j-th subpicture ID of the i-th set of subpictures associated with the following scalable-nested SEI messages. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits

## Subpicture extraction support additions

The SEI message specified above may carry the PT/DUI SEI messages that apply to the extrated subpicture sub-bitstream that is extracted. Therefore, the following text needs to be added to the extraction process specified in the previous section.

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with nesting_ols_flag equal to 1 and that are applicable to outBitstream (nesting_subpic_id[ i ][ j ] is equal to subpicIdTarget[ ]), do the following:

–	Extract appropriate non-scalable-nested SEI message with payloadType equal to 1 (picture timing), or 130 (decoding unit information) from the scalable nesting SEI message and put the extracted messages messages into outBitstream.

# Proposed integration into Annex C

This section contains proposed integration of the above extraction process highlighted in green into the text of Annex C.1 based on JVET-Q0042-v2.

A set of bitstream conformance tests is needed for checking the conformance of a bitstream, which is referred to as the entire bitstream, denoted as entireBitstream. The set of bitstream conformance tests are for testing the conformance of each OP of each OLS specified by the VPS and testing the conformance of each subpicture specified in the subpicture level information SEI message. [Ed. (YK): Update the HRD text, particularly the part on describing the conformance tests, such that it does not appear that VPS would be needed even when the bitstream only has a single layer.]

For each test, the following ordered steps apply in the order listed, followed by the processes described after these steps in this clause:

- An operation point under test, denoted as targetOp, is selected by selecting a target OLS with OLS index opOlsIdx, and a highest TemporalId value opTid and a subpicList[]. The value of opOlsIdx is in the range of 0 to TotalNumOlss − 1, inclusive. The value of opOlsIdx is in the range of 0 to TotalNumOlss − 1, inclusive. The value of opTid is in the range of 0 to vps_max_sublayers_minus1, inclusive. The array of lists subpicList[] includes either all subpicture ID values of each layer of the targetOp or a single subpicture ID value per layer of corresponding target subpictures within each layer of the targetOp. Each pairtuple of selected values of opOlsIdx, and opTid, and subpicList[] shall be such that the sub-bitstream that is the output by invoking the sub-bitstream extraction process as specified in clause C.X with entireBitstream, opOlsIdx, and opTid and a subpicList[] as inputs satisify the following conditions:

–	There is at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ opOlsIdx ] in BitstreamToDecode.

–	There is at least one VCL NAL unit with TemporalId equal to opTid in BitstreamToDecode.

–	There is at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ opOlsIdx ][ j ] and with slice_subpic_id equal to each of the slice_subpic_id values in subpicIdTarget[ j ] for each j in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1.

- If the layers in targetOp include all layers in entireBitstream, and opTid is equal to the highest TemporalId value among all NAL units in entireBitstream, and subpicList[] includes all subpicture ID values of each layer of the targetOp, BitstreamToDecode is set to be identical to entireBitstream. Otherwise, BitstreamToDecode is set to be the output by invoking the sub-bitstream extraction process as specified in clause C.X with entireBitstream, opOlsIdx, and opTid and and a subpicList[] as inputs.

[Ed. The following steps are not affected by the propal after sub-bitstream extraction]

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)